3GPP TSG RAN WG2 #123 Toulouse, France	R2-2308310

Aug 21st-25th, 2023

Agenda item:	7.5.4.1

Source:	CMCC

Title:	Consideration on BSR enhancement for XR

Document for:	Discussion

1	Introduction

In RAN2 122[1], consensus has been reached on BSR solution for XR, and leaving serveral FFS and future works for RAN2 123. Some of them we would like to disscuss, which is shown in the following:

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification.FFS which UEs support this.

When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

In this contribution, a discussion on BSR enhancement will be conducted for XR service.

2	Discussion

2.1 BSR with delay status

In the previous discussion, RAN2 agreed that the UE should calculate remaining time based on the PDCP discard timer. Considering that the XR traffic flow contains multiple kinds of data, such as video, audio, and pose/control information, Besides, there can be various of flows within certain kind of data, e.g., different slices and frames in video data (i.e., I/IDR frame vs B frame or I-slice vs B-slice).

Moreover, current granularity of BSR reporting is per LCG, and there is probability that multiple different remaining times exist for data of different types within the same LCG. If multiple remaining times and the corresponding caches were used to assemble BSRs and reported to the MAC scheduler, ambiguity could easily arise and cause the scheduler to misunderstand the true remaining time of the data.

Specifically, if the remaining time can be multi-valued, then each remaining time will have its corresponding buffer level value. For legacy BSR mechanism, the buffer level reported is the total amount of all available in the corresponding LCG buffer. To avoid gNB MAC scheduler misunderstanding the remaining time corresponding buffer level as the total available data of the LCG and resulting in the allocation of a too small UL grant, per LCG buffer aggregation is needed in gNB MAC layer. This buffer reporting and handling mechanism has significant differences compare to the legacy mechanism and may result in implementation complexity and specification impact.

Considering all the above, it is believed that a single-valued remaining time reflecting overall delay status for all available data per LCG is preferred. Besides, this single-value remaining time can be defined as the average remaining time and the minimum remaining time of available data per LCG.

Proposal 1: RAN2 to agree to use a comprehensive single value remaining time reflecting the overall remaining time for new BSR, which can be either the shortest or average remaining time.

Another related issue is UE assembling Transmission Blocks priority. In general, the priority is determined by the logical channel group and the logical channel. However, after introducing remaining time and PSDB, the priority of filling data when assembling TBs may be affected. We believe that this problem can be left to the UE implementation to solve.

Regarding UE capability for new BSR, we believe that there is no need to set limitation on new BSR. The new BSR can be treated as an optional UE capability. XR services have a high similarity with traditional MTSI, DASH, and other multimedia services. Using the new BSR in similar services to XR can also help improve system capacity. In addition, since the calculation of remaining time is based on the PDCP discard timer rather than the PSDB, the remaining time is usually a valid value in most cases. If the PDCP discard timer is somehow not configured, the terminal can report an invalid value, e.g., 0xFFFF, to implicitly indicate that the remaining time of the new BSR being reported is meaningless.

Observation 1: MTSI and DASH UE can also use new BSR to obtain the benefits of lower quantization error.

Proposal 2: Non-XR UE, e.g., MTSI and DASH capable UE, can also use new BSR.

Proposal 3: For UE cannot calculate remaining time, UE can report an invalid remaining time indicating it.

2.2 BSR Table for XR

Regarding the BSR Table, the consensus in RAN2 is to use an 8-bit static BSR Table. Therefore, the main issues are determining the maximum and minimum values, as well as the step size, for the BSR Table. In RAN2-121bis, RAN2 has agreed to use a narrower range for the new BSR Table compared to the legacy table.

Design/configuration for new BSR table(s) should include support for narrower ranges (i.e., finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g., pre-definition vs RRC configuration) is made.

The total number of BS levels is determined (i.e., 8bit for 256) for the static linear BSR Table, and therefore, only the maximum and minimum values need to be determined. Therefore, the main point of discussion is the determination of the maximum and minimum buffer values of the BSR Table.

For XR, the most important component of the XR traffic flow is the video stream. Therefore, the maximum and minimum values of the BSR Table can be determined based on the size of the video stream.

For the maximum value, there is a theoretical upper limit for the size of a video frame, which is no more than 30MB for uncompressed 10-bit 2160p or 4K video frames and 24MB for uncompressed 8-bit 2160p or 4K video frames. Higher resolutions and deeper colour depths are not widely supported now and can be considered in the future.

As for the minimum value, there are two options: zero and non-zero value. The specific choice depends on whether the UE can only use the legacy Table when sending an LCG with no available data in the BSR. A non-zero minimum value has smaller quantization errors, and the problem of not being able to report zero buffer can be solved by designing a new MAC CE format and adding corresponding LCIDs for the new BS. That is, when UE needs to report zero buffer, it must use the legacy BSR.

Regarding the definition of the specific minimum value, we believe that this minimum value is related to the maximum compression rate of the video codec. There are already many video codecs available, such as HEVC/HSVC, VVC, AV1, and NVENC, and these video codec have varying performance on compression rate. However, the minimum value of buffer size can be determined by the maximum uncompressed video frame size for the minimum supported resolution and the maximum compression rate of the Video Codec. If the RAN2 assumes the minimum supported resolution to be 720p, then the maximum value of video frame is 3.3MB (uncompressed). Supposing the maximum compression rate of the video codec is 40, the minimum value of buffer size should be 68KB.

Observation 2: When determining the range of BS table, the range of video frame size and compression rate of video codec should be considered. If not provided, video code rate should be considered.

Since the content related to video codecs is not within the scope of RAN2, we suggest sending an LS to the relevant WGs to determine the range of the BS table that is sufficient to cover the range of XR data. Additionally, we can determine an initial range based on the simulation assumptions in TR 38.838:

Table 5.5.2.1-1: Statistical parameters for AR UL Model 1 (one stream model) Parameters; unit; value. Packet size; byte; Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%). packet generation rate: F; Hz; 60. Jitter; ms; Optional, follows the description in clause 5.1.1.2. Data rate: R; Mbps; 10 (baseline), 20 (optional). PDB; ms; 30 (baseline), 10 or 15 or 60 (optional).

Therefore, the upper limit of the new BSR should at least be greater than 341 KB (20 Mb/60), and the minimum value using a compression rate of 40 is the uncompressed frame size of 720p, which is approximately 68 KB.

Proposal 4: The preliminary range of the new BS table should cover [68, 341] KB, An LS for SA4 is needed for comments on video frame size and codec compression rate.

2.3 BSR MAC CE format

Due to the introduction of the narrow range BSR table and the BSR Table configuration per LCG, it is necessary to redesign the short and long BSR MAC CE format. In order to distinguish the new BSR from the legacy BSR, we suggest introducing formats for the new BSR MAC CE. This will allow UE to report BSR which has the least quantization error BSR.

To distinguish between new and legacy BSR. we believe that 2 new LCIDs should to be introduced, which stands for Short and Short Truncated BSR MAC CE.

Proposal 5: RAN2 to agree at least 2 LCID should be introduced for new short and short truncated BSR MAC CE.

For the BSR format, we proposal BSR MAC CE format with remaining time, and the short and short truncated BSR MAC CE format with remaining time and new BS table can be:

Figure 2.3-1: New Short BSR and Short Truncated BSR MAC CE

In which the 0x1F value of Remaining time field stands for invalid remaining time, or infinite, which also works.

For long and long truncated BSR MAC CE, since the BSR table is configured per LCG, there can be multiple BSR tables within single long or long truncated BSR MAC CE. RAN2 can either update legacy long and long truncated BSR MAC CE or introduce new long and long truncated BSR MAC CE. We are open to either.

Proposal 6: RAN2 to discuss whether to modify legacy long and long truncated BSR MAC CE format for new BS table and remaining time or to introduce new LCID and BSR MAC CE format.

The long and long truncated BSR MAC CE format with remaining time and new BS table can be:

Figure 2.3-2: New Long BSR and Long Truncated BSR MAC CE

In which the 0xFF value of Remaining time field stands for invalid remaining time, or infinite, which also works. And 0x0 value for BT field stands for legacy BS table, 0x1 value for BT field stands for new BS table.

Proposal 7: RAN2 to discuss the proposed new BSR MAC CE format.

3	Summary

This contribution discusses potential capacity enhancement for XR, the following are the observations and proposals that may be taken into consideration.

Delay reporting aspect:

Proposal 1: RAN2 to agree to use a comprehensive single value remaining time reflecting the overall remaining time for new BSR, which can be either the shortest or average remaining time.

Observation 1: MTSI and DASH UE can also use new BSR to obtain the benefits of lower quantization error.

Proposal 2: Non-XR UE, e.g., MTSI and DASH capable UE, can also use new BSR.

Proposal 3: For UE cannot calculate remaining time, UE can report an invalid remaining time indicating it.

BS table design aspect:

Observation 2: When determining the range of BS table, the range of video frame size and compression rate of video codec should be considered. If not provided, video code rate should be considered.

Proposal 4: The preliminary range of the new BS table should cover [68, 341] KB, An LS for SA4 is needed for comments on video frame size and codec compression rate.

BSR format aspect:

Proposal 5: RAN2 to agree at least 2 LCID should be introduced for new short and short truncated BSR MAC CE.

Proposal 6: RAN2 to discuss whether to modify legacy long and long truncated BSR MAC CE format for new BS table and remaining time or to introduce new LCID and BSR MAC CE format.

Proposal 7: RAN2 to discuss the proposed new BSR MAC CE format.

References

- R2-2306542, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia).