# 3GPP TSG-RAN WG2 Meeting #123	   R2-2308325

# Toulouse, France, Aug 21st - Aug 25th, 2023

# Agenda Item:	7.13.5

# Source:	CMCC

# Title:	SON enhancement for NR-U

# Document for:	Discussion and Decision

1	Introduction

In RAN2#122, we discussed SON/MDT for NR-U and the following agreements are made.

Agreements:

1	Only the preamble transmission attempts for which LBT was successful are represented in the “per RA attempt info list” for a given beam.

2	On how to represent the preamble transmission attempts blocked by LBT,

Introduce a field (or reusing the existing field) that counts the number of preamble transmissions blocked by LBT per RA procedure, and a flag indicating transmission failures experienced right before beam switching. Details can FFS.

3	For the RA-Report, the enhancements on the handling of the “per RA attempt info list” (i.e. as per Proposal 1) apply only to the last RA procedure in the last BWP prior to the random access success.

4	For the other BWPs in which the UE experienced the consistent LBT failure, the UE logs in the RA-InformationCommon:

a.	The locationAndBandwidth information of the BWP

b.	The subcarrierSpacing information of the BWP

c.	The absoluteFrequencyPointA information of the BWP ( How to log once for all the BWPs of the cell is FFS)

5	As baseline, RAN2 assumes the following:

a.	Enhancements discussed for the RA-InformationCommon for the RA-Report are applicable also to the RLF-Report

b.	The detailed “per RA attempt info” are only reported in the RLF-Report for the last RA procedure before RLF/HOF, FFS whereas limited information are reported for the other BWPs in which consistent LBT failure is detected

c.	The above bullets may be revisited case by case depending on future agreements.

6	The UE logs RA-InformationCommon including LBT info in the RLF-Report, in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure (as in legacy).

7	The UE logs the available RSSI measurement in the RLF-Report. FFS in which case.

8	The UE should log the following RSSI values in the RLF-Report:

a.	For RLF, the latest measured RSSI of the NR-U channel of the last serving cell if measRSSI-ReportConfig is configured for the corresponding frequency.

b.	FFS: For HOF, the latest measured RSSI of the NR-U channel of the source cell, and  the latest measured RSSI of the NR-U channel of the target cell, if measRSSI-ReportConfig is configured for the corresponding frequency.

In this contribution we provide our opinions about the FFS issue for NR-U.

2	Discussion

2.1 RLF report:

In RAN2#122, the remaining issues for RLF report are including as following:

Proposal 9	FFS: The UE logs in the RLF-Report the BWP information (at least the locationAndBandwidth, and the subcarrierSpacing) of all the BWPs in which the UE detected the consistent UL LBT failures right before the RLF/HOF.

Proposal 11	FFS:Support these further options on when to log the RA-InformationCommon including LBT info in the RLF-Report:

b.	When the RLF cause is lbtFailure, and the UE was performing random access in other BWPs due to triggered consistent UL LBT failures

Proposal 18	FFS: UE to log indication on whether the detected power at the moment of LBT failure was above the configured EDT threshold (maxEnergyDetectionThreshold).

Proposal 19	UE logs lbt-FailureRecoveryConfig in the RLF-Report only upon re-establishment procedure failure.

For BWP information, RAN2 has agreed to log the BWP information (e.g. locationAndBandwidth, subcarrierSpacing and absoluteFrequencyPointA) in RA-InformationCommon, and leaving log the BWP information in the RLF-Report as FFS.

As UE may experience some consistent LBT failures in other BWPs before RLF occurs, when the lbt-FailureRecoveryConfig is configured, it’s beneficial for the network to understand which BWP is suffered from consistent LBT failures. And the network may derive the load information for different BWPs, such as the BWP in which UE detected consistent LBT failures may experience heavy load and network may not configure too many UE/traffic to the BWP which is reported with consistent LBT failures. And also, since RA-InformationCommon IE is included in both RLF report and SHR, the BWP information is naturally added to the RLF report and SHR. So, it’s reasonable to log the BWP information of all the BWPs in which the UE detected the consistent UL LBT failures before RLF/HOF in RLF report.

Observation 1: As RA-InformationCommon IE is included in both RLF report and SHR, the BWP information is naturally added to the RLF report and SHR.

Proposal 1: If UE suffers from consistent LBT failure before RLF/HOF, UE log the BWP information in RLF report for all the BWPs in which the UE detected the consistent UL LBT failures.

Then, RAN2 also agreed to log RA-InformationCommon including LBT info in the RLF-Report, in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure (as in legacy). According to existing specs, if consistent uplink LBT failures are detected on all the UL BWP(s) with configured RACH resources, the UE declares RLF and stores the failure information in RLF report with failure cause (i.e., rlf-Cause) set as lbtFailure. If the RLF cause is lbtFailure, we think it means that the consistent LBT failures directly impact the RLF/HOF, and log the RA-InformationCommon including LBT info in the RLF-Report may help the network analyze and resolve the RLF problems of NR-U.

Proposal 2: Log the RA-InformationCommon including LBT info in the RLF-Report when the RLF cause is lbtFailure.

Also, in previous meeting, the EDT information related proposals are discussed in email discussion. Including whether to log the indication on whether the detected power at the moment of LBT failure was above the configured EDT threshold (maxEnergyDetectionThreshold). For EDT, we think reporting the applied EDT is more reasonable. Although the applied EDT depends on the UE implementation, it can help the network to identify whether the LBT fails because the maximum EDT configured by the network side is unreasonable or the EDT set by the UE itself is inappropriate. For the detected power, we think that the detected power can be used to compare with applied EDT to determine whether it’s LBT failure, and analyze the channel load.

The indication about whether the detected power was above the configured EDT threshold mentioned in RAN2#122 is FFS, we think it is unnecessary to introduce this indication. This indication indicates the relationship between detected power and configured EDT threshold (maxEnergyDetectionThreshold), but the detected power is needed to compare with applied EDT to determine whether LBT is failure. LBT failure still occurs when the detected power is smaller than the maximum EDT but larger than the applied EDT. If LBT success and transmit successful, the detected power should be smaller than the maximum EDT and also smaller than the applied EDT, reporting this seems meaningless. If LBT failure occurs, the indication will indicate two cases: 1) detected power is smaller than the configured EDT threshold and larger than the applied EDT; 2) detected power is larger than the maximum EDT. For case 1), only the indication cannot clearly determine which kind of problems caused the LBT failure, it can be the detected power is not extremely high but the applied EDT set by UE implementation is inappropriate, or the the detected power is extremely high and close to the maximum EDT. So, we propose to directly report the applied EDT and average detected power.

Proposal 3: No need to introduce the indication on whether the detected power at the moment of LBT failure was above the configured EDT threshold (maxEnergyDetectionThreshold).

Proposal 4: Log the applied EDT and average detected power in RLF report.

Then, for whether to log the lbt-FailureRecoveryConfig in RLF report, according to the RAN3 LS, the network can retrieve the UE context or the configuration used for the UE in the last serving node based on the information provided from the UE (last serving PCell ID and C-RNTI). But it’s only available when UE attempts re-connection and reports the failure right after connection failure. If the failure information is fetched from the UE hours after the failure, whether the UE configuration can be retrieved is depended on the RAN implementation.

So, in order to enable the network obtain the UE configuration, the simple way is UE logs lbt-FailureRecoveryConfig in the RLF-Report upon re-establishment procedure failure.

Proposal 5: UE logs lbt-FailureRecoveryConfig in the RLF-Report upon re-establishment procedure failure.

2.2 SHR

In RAN2#122, the remaining issues for SHR are including as following:

Proposal 20	RAN2 to further discuss if any of the following new triggering conditions should be considered for the SHR in NR-U:

a.Number of UL LBT failures during HO higher than a certain threshold

b.Consistent UL LBT failures triggered during HO in at least one UL BWP on the source cell and consistent UL LBT failures triggered during HO in at least one UL BWP on the target cell

Proposal 21	FFS: Related to the target cell, the UE logs in the SHR the random access information, same as for the RA- and RLF-Report, i.e. including the number of UL LBT failures during HO (depending on the outcome of Proposal 2), and the information on the multiple BWPs (depending on the outcome of Proposal 4) in which consistent UL LBT failures was triggered. FFS on the trigger conditions to log.

Proposal 23	FFS: RAN2 to discuss what LBT information (if any) related to the source cell of the HO should be included in the SHR.

For SHR, one of the remaining issues is whether to introduce the new trigger condition for NR-U. The SHR can be used to log the information or problems during HO. During HO, even if the HO is successful finally, UE may detect several LBT failures during the HO procedure. When the number of LBT failures is large, the channel load may be extremely high. Even if the HO is successful, it’s near failure. Introduce new SHR trigger condition “Number of UL LBT failures during HO higher than a certain threshold” is beneficial to the network that the HO problems can be logged for NR-U HO procedure. And this information may help the network finding out the near failure HO procedure and derive the channel load.

Besides, since the random access information, including the number of UL LBT failures and the information on the multiple BWPs in which UE detected consistent UL LBT failures, are added to the RA report and RLF report, we also think that logging these information in SHR is beneficial for the network to optimize the HO procedure.

Proposal 6: Introduce new SHR trigger condition “Number of UL LBT failures during HO higher than a certain threshold” for NR-U.

Proposal 7: Log the number of UL LBT failures of target cell during HO and the information on the multiple BWPs in which UE detected consistent UL LBT failures in SHR.

3	Conclusion

Observation 1: As RA-InformationCommon IE is included in both RLF report and SHR, the BWP information is naturally added to the RLF report and SHR.

Proposal 1: If UE suffers from consistent LBT failure before RLF/HOF, UE log the BWP information in RLF report for all the BWPs in which the UE detected the consistent UL LBT failures.

Proposal 2: Log the RA-InformationCommon including LBT info in the RLF-Report when the RLF cause is lbtFailure.

Proposal 3: No need to introduce the indication on whether the detected power at the moment of LBT failure was above the configured EDT threshold (maxEnergyDetectionThreshold).

Proposal 4: Log the applied EDT and average detected power in RLF report.

Proposal 5: UE logs lbt-FailureRecoveryConfig in the RLF-Report upon re-establishment procedure failure.

Proposal 6: Introduce new SHR trigger condition “Number of UL LBT failures during HO higher than a certain threshold” for NR-U.

Proposal 7: Log the number of UL LBT failures of target cell during HO and the information on the multiple BWPs in which UE detected consistent UL LBT failures in SHR.

4	Reference

[1] 	RP-221825, “New WID on Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC”, CMCC