3GPP TSG-RAN WG2 Meeting #122	R2-2308363

Toulouse, France, August 21-25, 2023

Agenda Item:	7.14.4

Source:	CATT

Title:	Discussion on support of QoE measurement for NR-DC

Document for:	Discussion and Decision

# 1 Introduction

In last RAN2 meeting [1], the QoE measurement in NR-DC was discussed and some agreements were achieved as follow.

- 1: The network can use explicit indication per QoE config to indicate which SRB is used for the QoE reporting. Details can be discussed in Stage-3. - FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements) - FFS how the above case works if SCG is deactivated or released. - RAN2 will follow RAN3 agreement on QoE config RRC IDs. - 2: RAN2 will follow RAN3 agreements (for NR-DC when SRB5 is not configured) on forwarding the received encapsulated QoE reports to the correct recipient (i.e. MCE or SN)

In this discussion paper, we will further analyse the remaining issues.

# 2 Discussions

## 2.1 QoE reporting in case of SCG failure, deactivated or released

Accroding to the previous agreement, the UE will send the QoE report to network via SRB4 or SRB5 according to the explicit indication per QoE configuration. But in last RAN2 meeting, there is an FFS left on how to handle the QoE report transmission if only one SRB configured but the QoE report is not configured for that SRB. And a similar case is when UE is configured that the QoE report should be sent to network via SRB5 but the SCG is deactivated or released.

- FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements) - FFS how the above case works if SCG is deactivated or released.

Firstly, we need to discuss whether these cases will exist. According to the RAN3’s agreement[2], the SN can indicate to the MN that the reports are to be sent via the SRB5. And the SN can request the use of SRB4 for QoE reporting, which the MN can confirm or reject. Then the MN/SN can send the QoE configuration with an explicit SRB indication to the UE.

The SN can indicate to the MN that the reports are to be sent via the SRB5. The SN can request the use of the SRB4 for reporting, which the MN can confirm or reject.

This means that which SRB does UE use to send the QoE report is in control of the network. And before the SRB indication sent to UE, the coordination is needed for MN and SN. Therefore, in our understanding, before the SCG is deactived or released, the SN has confirmed with MN that the QoE report can be reported via SRB4. Otherwise the SN will not be deactived or released. We can specify that it is UE’s default behaviour that if SCG is deactived or released, UE will not consider the SRB indication and sent the QoE report to MN via SRB4 directly. Or the network updates the QoE configuration to indicate the all QoE reports need to be sent to MN via SRB4. This will introduce more signalling overload which is not prefered. If the MN reject to receive the SN-assocaited QoE report, the SN can release these QoE configuration. So in case of UE is configured that send the QoE report via SRB5 but the SN is deactived or released, UE can send the QoE reports according to the updated QoE configuration or just send the QoE report to MN via SRB4 defaultly.

Proposal 1: In case of UE is configured that send the QoE report via SRB5 but the SN is deactived or released, UE can send the QoE reports according to the updated QoE configuration or just send the QoE report to MN via SRB4 defaultly.

Besides, it is also possible that the SN occurs the radio link faillure after UE receives the QoE configuration with SRB indication. In this case, the coordination on reporting leg between MN and SN may be not completed when the failure occurs. So in this case the MN may be not ready to receive the QoE report which shold be sent to SN. In this case, we need to wait for RAN3 to decide whether MN can receive these QoE reports and how to handle these QoE report. Or the UE can pause to send these QoE report and after the MN sending the indication, UE can send the QoE reports to MN via SRB4.

Proposal 2: RAN2 need to discuss how the UE handles the QoE report when SCG failure occures.

## 2.2 How to forward the QoE report associated with the other node

Another remaining issues is that how MN knows to corrrectly forward SN-associated QoE reports received via SRB4. According to RAN3’s agreements[2], if MN is asked to forward the SN-associated QoE reports to MCE, the SN should indicate to the MN the QoE reference and MCE IP address; if SN is asked to forward the MN-associated QoE reports to MCE, the MN should indicate to the SN the QoE Reference, the MCE IP address and RRC ID.

If the SN is asked by the MN to forward to the MCE the QoE reports pertaining to a measurement configured by the MN, the MN should indicate to the SN the QoE Reference, the MCE IP Address and the RRC ID. If the MN is asked by the SN to forward to the MCE the QoE reports pertaining to a measurement configured by the SN, the SN should indicate to the MN the QoE Reference and the MCE IP Address.

So, according to the R3’s agreement, the MN can correctly forward the SN-associated QoE report to MCE or SN based on the information from SN.

Proposal 3: According to the RAN3’s agreement, the MN can correctly forward the SN-associated QoE report to MCE or SN based on the information from SN.

## 2.3 RAN overload

According to the RAN2#121bis agreement [3], Rel-17 pause/resume procedure is reused for UE in NR-DC. But the detail is still FFS. For exmple, whether paused QoE reports can be reported to SN when the SN is not overload.

- 14: As a baseline, Rel-17 pause/resume procedure is reused to pause/resume reporting of one or multiple QoE measurement configurations in a UE for NR-DC. Details are FFS, e.g. whether paused QoE reports can be reported to SN (if SN is not overload).

In our understanding, when UE is in NR-DC, if one node, SN or MN is overload, this node will cordinate with the other node firstly. If the other node allows to receive the QoE report which should be sent to the overload node, the network will update the QoE configuration with changed leg indication firstly. Only if the other node cannot receive these QoE reports, the overload node will send the pause indication to UE. So in this case, if the node sends the pause indication to UE, it implicitly indicates that UE cannot send the QoE report to the other node. UE just needs to store the QoE report and resumes to send QoE report to network after receive the resume indication as in Rel-17.

Proposal 4: When UE in NR-DC receives the pause indication from one node, UE cannot send the paused report to the other node.

# 3 Conclusions

According to the above discussion, the following proposals are made:

Proposal 1: In case of UE is configured that send the QoE report via SRB5 but the SN is deactived or released, UE can send the QoE reports according to the updated QoE configuration or just send the QoE report to MN via SRB4 defaultly.

Proposal 2: RAN2 need to discuss how the UE handles the QoE report when SCG failure occures.

Proposal 3: According to the RAN3’s agreement, the MN can correctly forward the SN-associated QoE report to MCE or SN based on the information from SN.

Proposal 4: When UE in NR-DC receives the pause indication from one node, UE cannot send the paused report to the other node.

# 4 Reference

- Report of 3GPP TSG RAN2 meeting #122

- Report of 3GPP TSG RAN3 meeting #119bis

- Report of 3GPP TSG RAN2 meeting #121bis