3GPP TSG-RAN WG2 Meeting #123	R2-2308364

Toulouse, France, August 21-25, 2023

Agenda Item:	7.18.2

Source:	CATT

Title:	The remaining issues on CP aspects for MT-SDT

Document for:	Discussion and Decision

1	Introduction

In previous RAN2#122 meeting [1], we have discussed the issues on MT-SDT and have made some agreements as below:

1. Allow support of only MT-SDT in a cell. A separate SIB configuration will be introduced. FFS what is put in there. 2. For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list. 3. gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging ). 4. UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message 5. MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15). 6. SRB2 can be used for MT-SDT (i.e. similar to MO-SDT) 7. No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT 8. When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”. 9. A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

In this contribution, we will further analyse the remaining issues on CP aspects and provide our views.

2	Discussion

2.1 The separate SIB configuration for MT-SDT

According to the agreement in last RAN2 meeting, it is allowed that only the MT-SDT is configured in a cell. And a separate SIB configuration will be introduced. According to the Running RRC CR to MT-SDT, a new IE mt-SDT-ConfigCommonSIB-r18 is introduced for MT-SDT for the case of only MT-SDT is configured. And the new IE will reuse the SDT-ConfiguCommonSIB-r17 structure as below:

SIB1-v1740-IEs ::=               SEQUENCE {

si-SchedulingInfo-v1740          SI-SchedulingInfo-v1740                                            OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v18xx-IEs                                                     OPTIONAL

}

SIB1-v1800-IEs ::=               SEQUENCE {

mt-SDT-ConfigCommonSIB-r18   SDT-ConfigCommonSIB-r17                                                OPTIONAL,  -- Cond MT-SDT-Only

nonCriticalExtension         SEQUENCE {}                                                            OPTIONAL

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=    ENUMERATED {a, b, c}

UAC-AC1-SelectAssistInfo-r16 ::=     ENUMERATED {a, b, c, notConfigured}

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range                                                            OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

Although this method can simply the ASN.1 structure, there may be some different configuration between the SIB configuration of MO-SDT and MT-SDT. As in MT-SDT procedure, the data volume checking is only handling by gNB but not by UE. So in case of only MT-SDT configured in the cell, the data volume threshold is not needed to be sent to UE. But in current Running CR, there seems no such limitation. As in SDT-ConfigCommonSIB-r17 IE, the presence of sdt-DataVolumeThreshold is mandatory. So if we reuse the SDT-ConfigCommonSIB-r17 IE, the gNB always needs to broadcast the data volume threshold in case of only MT-SDT configured which will result in a waste of 4bits in SIB1. Besides, we need to specify that the UE will not consider the sdt-DataVolumeThreshold in the filed description in case of mt-SDT-ConfigCommonSIB-r18 is configured. An alternative solution is to introduce a new IE for MT-SDT SIB configuration which will not include the data volume threshold.

Proposal 1: Considering the data volume threshold is not needed in case of only MT-SDT configured, it is better to define a new IE for SIB configuration for MT-SDT.

2.2 Separate RSRP

In last RAN2 meeting, it was agreed that a separate sdt-RSRP threshold for MT-SDT can be configured, at least in case of only MT-SDT is configured in the cell.

A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

During the post-email discussion, some company think that in case of both MO-SDT and MT-SDT configured, the network can also configure a separate sdt-RSRP threshold for MT-SDT. Considering the current agreement does not exclude this possibility, we need to make it clear whether allow the network to configure separate sdt-RSRP threshold for MT-SDT in this case.

In R17, the sdt-RSRP threshold was introduced to help UE to decide whether the quality of radio link is good enough to initial MO-SDT procedure. The reason why we ensure the UE is in good radio condition is that the small data PDUs are usually larger than the typical CCCH payload size. Besides, for CG-SDT, the UL transmission has a higher probability of timing is un-synchronization when UE is at the edge of cell. So the good radio condition is important for CG-SDT.

Although when UE initials the MT-SDT procedure, there is no pending uplink small data to be transmitted, the requirement of good radio condition for CG-based MT-SDT should be same as MO-SDT. There is no sufficient reason to introduce a separate sdt-RSRP for MT-SDT in case of both MO-SDT and MT-SDT are configured.

Proposal 2: There is no need to define a separate sdt-RSRP threshold in case of both MO-SDT and MT-SDT configured in the cell.

2.3 MT-SDT termination

Rel-17 MO-SDT is terminated via the following ways [3]. These can be reused for MT-SDT procedure.

Once initiated, the SDT procedure is either: - successfully completed after the UE is directed to RRC_IDLE (via RRCRelease) or to continue in RRC_INACTIVE (via RRCRelease or RRCReject) or to RRC_CONNECTED (via RRCResume or RRCSetup); or - unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or expiry of SDT-specific timing alignment timer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

Proposal 3: The mechanism of Rel-17 MO-SDT termination is reused for MT-SDT procedure.

3 Conclusion

According to the above discussion, the following proposals are formulated:

Proposal 1: Considering the data volume threshold is not needed in case of only MT-SDT configured, it is better to define a new IE for SIB configuration for MT-SDT.

Proposal 2: There is no need to define a separate sdt-RSRP threshold in case of both MO-SDT and MT-SDT configured in the cell.

Proposal 3: The mechanism of Rel-17 MO-SDT termination is reused for MT-SDT procedure.

4 References

- Report of 3GPP TSG RAN WG2 meeting #122;

- TS 38.300, V17.5.0