3GPP RAN WG2 Meeting #123	R2-2308394
Toulouse, France, August 21st – 25th, 2023

Agenda Item:	7.3.1

Source:	InterDigital

Title:	TS 38.321 open issues for NES

Document for:	Discussion, Decision

# Introduction

In this contribution, MAC open issues for NES are discussed along with some release notes for the initial version of the draft running TS 38.321 CR [3] for the NES work item [1].

# Discussion

## Notes about the initial draft running TS 38.321 CR for NES

A new section (5.x) is added in the running CR [3] to capture cell DTX and DTX operation, including starting on duration timers, activation/deactivation, and defining the Cell DTX/DRX Active Period and Non-Active Period. A structure similar to that of C-DRX is used.

The agreed UE behaviour during the cell DTX/DRX active period and non-active periods is described in the relevant sections (e.g. in the SR, HARQ, configured DL assignments, and C-DRX sections), and a high level description of the behaviour and referencing to those sections is added at the beginning of 5.x.

Wording of “cell DTX activated or deactivated” is generally avoided, as it would require defining each signals and channels that are expected when we usually specify “activated/deactivated”. Instead, the language “cell DTX activation indication has been received from lower layers …” is used, which is similar to how other L1 activation signalling is captured in TS 38.321. The exact name of this activation indication is to be determined or revisited once RAN1 decides on naming.

A definition of the cell DTX/DRX Active Period is added in section 5.x, which is then used in various sections to capture the agreed UE behaviours. The definition includes the case when cell DTX/DRX is configured, activated, and the On Duration timer is running. The definition also includes the case when cell DTX/DRX is deactivated, as the same behaviour should apply when it is deactivated.

## Open issues and editor’s notes

Issue 1: Applicability and configuration of cell DTX/DRX per serving cell vs. per MAC entity

During the study item phase [2], it was agreed that “Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA”. During the work item, it was agreed that “Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.”, which can be understood to confirm the SI agreement that DRX/DTX is configured per cell. RAN1 [4] has also agreed that “DCI format for group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation. FFS: handling of multiple cells including when UE supports different number of cells”. The running CR [3] thus is implemented with such assumption, i.e. cell DTX/DRX is configured and applicable per serving cell, however an editor’s note is added to confirm these assumptions:

Editor’s note: TBC whether cell DTX/DRX is configured per serving cell. Instances of “for the Serving Cell” and “for each Serving Cell” will be removed if it is configured per MAC entity.

It would therefore be beneficial for RAN2 to discuss and confirm the SI assumptions. This discussion is separate from the FFS on whether we support multiple cell DTX/DRX configurations per cell, as that is captured in a separate editor’s note. Even if multiple cell DTX/DRX configurations per cell is agreed, only one can be activated in a cell at a certain time.

Proposal 1: 	RAN2 to discuss confirming the following SI agreement: “Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA”.

Proposal 2: 	RAN2 to discuss the following SI/RAN1 assumption: In CA, cell DTX/DRX can be activated for one Serving Cell and deactivated on another; or activated on more than one Serving Cell.

During the study item phase [2], it was agreed that “At least the following parameters can be configured per cell DTX/DRX configuration: periodicity, start slot/offset, on duration”. During the work item phase, it was agreed that “Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.”, which can imply that each cell can have different values configured. In CA scenarios, given there is only one MAC entity per CG and the UE only maintains one C-DRX configuration per MAC entity, it can be beneficial that multiple serving cells have the same Cell DTX/DRX configuration for alignment purposes with C-DRX. On the other hand, it may be useful to leave configuration up to the network, i.e. whether to align all cells with the same periodicities and offsets or not. The following editor’s note is thus added in TS 38.321 running CR [3]:

Editor’s note: TBC whether cell DTX/DRX parameters can be configured with different values per serving cell.

Proposal 3: 	RAN2 to discuss confirming the following SI agreement: “Cell DTX/DRX configuration parameters (e.g. periodicity, start slot/offset, and on duration) can be configured per cell.” i.e. cell DTX/DRX parameters can be configured with different values per serving cell.

Issue 2: UE PDCCH monitoring behaviour when cell DTX is configured and C-DRX is not configured.

The running CR captures the agreement “UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time” in the C-DRX section. It can be further discussed whether this agreement is also applied to the case when cell DTX is configured and C-DRX is not configured, though it would complicate PDCCH monitoring for retransmissions. Another possibility is to restrict configuring cell DTX only to the case when C-DRX is configured, which would result in less complexity in TS 38.321. The following options are possible for the UE PDCCH monitoring behaviour when cell DTX is configured and C-DRX is not configured:

- Restrict configuring cell DTX in RRC only to the case when C-DRX is configured; or

- When C-DRX is not configured, the UE doesn’t monitor PDCCH during Cell DTX non-active period for dynamic grants/assignments (e.g. for both new transmissions and retransmissions); or

- When C-DRX is not configured, the UE doesn’t monitor PDCCH during Cell DTX non-active period for dynamic grants/assignments, unless there is at least one pending HARQ process.

Option 1 is simplest and does not require any additional complexity in the MAC specifications. With Option 2,  the UE doesn’t monitor PDCCH at all during the non-active period, even if there is a pending transmission or HARQ ACK/NACK determination. Option 2 thus delays retransmissions to the active period only. Option 2 may also be problematic as the UE is specified to flush the buffer (e.g. for CGs) upon expiry of the CG timer, especially when a long cell DTX periodicity is configured, thus resulting in false ACK and data loss. Option 3 follows the agreement for the scenario when C-DRX is configured. However, unlike the C-DRX case, there are no retransmission timers during which the UE monitors PDCCH, and the UE may end up monitoring PDCCH for a very long period of time if the initial PUSCH transmission wasn’t received by the gNB (e.g. up to a maximum of 10 seconds), thus wasting the UE’s battery power.

Proposal 4: 	RAN2 to discuss the UE PDCCH monitoring behaviour when cell DTX is configured and C-DRX is not configured. The following options are possible:

- Restrict configuring cell DTX in RRC only to the case when C-DRX is configured; or

- When C-DRX is not configured, the UE doesn’t monitor PDCCH during Cell DTX non-active period for dynamic grants/assignments (e.g. for both new transmissions and retransmissions); or

- When C-DRX is not configured, the UE doesn’t monitor PDCCH during Cell DTX non-active period for dynamic grants/assignments, unless there is at least one pending HARQ process.

Issue 3: Addition of cell DTX/DRX SlotOffset

RAN2 have agreed to a cell DTX Start Offset which defines the subframe during which cell DTX cycle starts. Looking at the C-DRX formula and specification, there is also a “slot offset” parameter to define the delay in ms fractions after the beginning of the subframe as to when the UE starts the C-DRX on duration timer. For C-DRX, there are 32 sub-ms values of SlotOffset (value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on). In order for the network to be able to align C-DRX and Cell DTX, a similar parameter for Cell DTX is needed.

Per the agreement “The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration”, the rapporteur understands that “start slot/offset” are two parameters similar to C-DRX. The parameters celldtx-StartOffset and celldtx-SlotOffset are thus added in the running CR [3] and is assumed to be needed as well.

Proposal 5: 	RAN2 confirms that both StartOffset and SlotOffset parameters are introduced part of cell DTX and cell DRX configurations.

# Conclusion

In this contribution, MAC open issues for NES are discussed along with some release notes for the initial version of the draft running TS 38.321 CR for the NES work item. The following proposals are provided:

Proposal 1: 	RAN2 to discuss confirming the following SI agreement: “Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA”.

Proposal 2: 	RAN2 to discuss the following SI/RAN1 assumption: In CA, cell DTX/DRX can be activated for one Serving Cell and deactivated on another; or activated on more than one Serving Cell.

Proposal 3: 	RAN2 to discuss confirming the following SI agreement: “Cell DTX/DRX configuration parameters (e.g. periodicity, start slot/offset, and on duration) can be configured per cell.” i.e. cell DTX/DRX parameters can be configured with different values per serving cell.

Proposal 4: 	RAN2 to discuss the UE PDCCH monitoring behaviour when cell DTX is configured and C-DRX is not configured. The following options are possible:

- Restrict configuring cell DTX in RRC only to the case when C-DRX is configured; or

- When C-DRX is not configured, the UE doesn’t monitor PDCCH during Cell DTX non-active period for dynamic grants/assignments (e.g. for both new transmissions and retransmissions); or

- When C-DRX is not configured, the UE doesn’t monitor PDCCH during Cell DTX non-active period for dynamic grants/assignments, unless there is at least one pending HARQ process.

Proposal 5: 	RAN2 confirms that both StartOffset and SlotOffset parameters are introduced part of cell DTX and cell DRX configurations.

# References

RP-223540, “New WID: Network energy savings for NR”, Huawei

TR 38.864, “Study on network energy savings for NR”, v 18.0.0

R2-2308393, “Running CR to 38.321 for Network energy savings”, InterDigital

RAN1#113 Chairman’s Notes, May 2023

# Annex A: Agreements on Cell DTX/DRX

- RAN2#121

Agreements

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

- Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

- Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

- RAN2#121bis

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

- RAN2#122

Agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.