3GPP TSG-RAN2 Meeting #123	R2-2308406

Toulouse, France, 21st - 25th August 2023

Agenda item:	7.18.3

Source:	Qualcomm Incorporated

Title:	Remining issues on user plane aspects of MT-SDT

WID/SID:	NR_MT_SDT-Core – Release 18

Document for:	Discussion and Decision

# Introduction

In the RAN2 #122 meeting, the following agreements were made for the user plane aspects of MT-SDT.

Agreements:

- RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

- LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

- Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

- RA-SDT resources are not used for MT-SDT initiation RACH

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

In this paper, we would like to discuss the remaining issues of user plane aspects of MT-SDT.

# Discussion

During MAC Running CR discussion [1], one open issue is that whether UE needs to check the UL data volume when SDT is triggered for MT-SDT. We have done some analysis in our contribution [2]. Thus, we have following proposal.

Proposal 1: UE is not required to check the UL data volume for Small Data Transmission procedure triggered for MT-SDT.

Another issue that needs more discussions in the MAC Running CR is that whether a separate RSRP threshold for MT-SDT should be configured from the Rel-17 sdt-RSRP-Threshold. Noted that it has been agreed that a separate RSRP threshold for MT-SDT can be configured in the case where MO-SDT is not configured by network in the last meeting. However, even the MO-SDT resource and the associated Rel-17 sdt-RSRP-Threshold is configured in the cell, such threshold is mainly to ensure the link budget for UL data transmission reliability which is the different use cases from the DL data transmission in MT-SDT. The DL data link budget has different requirements from that of UL data transmission. To ensure the efficient DL data scheduling under good reliability, it is desirable to configure the threshold separately. Thus, we have following proposal.

Proposal 2: The SDT RSRP threshold for MT-SDT is configured separately from the Rel-17 SDT RSRP threshold.

Regarding the long CG periodicity, RAN2 #122 TEI intends to agree to support longer CG periodicity under the conditions with small RAN1 impacts.

Basically, if the configured CG periodicity is too long, it may cause an unnecessary delay between the UL data arrival and next available CG resource which is not desirable. For the MT-SDT, similar issues could be observed that network may assume the paging failure if no response is sent from UE in time due to long CG periodicity. To resolve such issues, one solution was briefly discussed in the RAN2 #112 SDT.

A time gap threshold is configured by network to control the maximum waiting time for the next CG-SDT resource. UE determines the time gap between UE checking the conditions of the SDT initiation and the next CG-SDT occasion. If the time gap is not within the threshold, it implies that the CG-SDT resource is invalid, and UE should consider selecting RA resource for SDT procedure. Thus, we have following proposals.

Proposal 3: A time gap threshold is configured by network to control the maximum waiting time for the next CG-SDT occasion.

Proposal 4: When UE checking the SDT initiation conditions, UE should determine whether the gap to the next CG-SDT occasion is within the time gap threshold configured by network.

Proposal 5: If the gap is not within the time gap threshold, UE should consider selecting RA resource for SDT instead of CG resource for SDT.

If the initial transmission of CG-SDT including CCCH message is failed to be detected by network and no response is sent to UE, UE is allowed to automatically retransmit the CCCH message on the next CG resource. However, if the CG-SDT periodicity is long, UE has to wait for the next CG occasion with a quite long time or even the SDT failure timer may be expired before the next CG occasion. The efficient way would be to allow UE to select other resource to transmit the CCCH message if there is no response received from the network. Therefore, the preferred solution is to allow UE switching to RA based SDT. Since the network may even not be aware that UE has initiated a SDT procedure or not, switching to trigger RA based SDT is helpful for both UE and network to recover the SDT service. Otherwise, UE has to cancel the whole SDT procedure and goes to idle state which is obviously not preferred.

Proposal 6: UE is allowed to switch to RA based SDT procedure if no response is received for the initial CG-SDT transmission.

In MT-SDT, the UL data in response to the DL data may be generated after contention resolution in a legacy RACH. In the subsequent phase of MT-SDT, if no UL resource is available, UE has to trigger another RACH to request the UL resource for UL data handling which is inefficient. In this case, if SR resource is configured, it will be quite helpful to use SR resource to request more UL grant for UL data handling in the subsequent MT-SDT. Having SR resource configuration for MT-SDT can also avoid excessively triggering RACH just for small application layer UL data feedback in the subsequent phase of MT-SDT.

Proposal 7: RAN2 should study to have SR resource configuration for supporting UL data handling in MT-SDT procedure.

In Rel-17 SDT study, the power consumption of SDT UE has less consideration. There is no window/timer control for PDCCH monitoring after each uplink transmission. UE has to keep monitoring the network response or potential DL/UL scheduling and wait for network sending the RRC message to terminate the whole SDT procedure even though UE only plans to send one-shot UL data. In a worse case, UE has to monitor PDCCH for several seconds till the T319a expiry.

If we consider Rel-17/18 SDT for different UE device types in different use cases, for example, for a RedCap wearable device, it may require much more power saving SDT procedure considering its reduced capability. However, we think there is no power saving mechanism defined in Rel-17 SDT study.

In Rel-18 MT-SDT, UE still needs to monitor PDCCH for network scheduling the DL data in the subsequent data phase. Since the DL traffic profile and scheduling gaps are handled and controlled by network, network has full knowledges on when to send DL data to UE. It can be up to network to configure proper DRX to adapt to the DL traffic pattern so that UE can wake up for reception only when DL data is arrived to save UE power. Especially for the case that there could be round-trip delay at application layer for the DL data, it is bad for such as the RedCap device to monitor the network response during a long gap which obviously wastes UE power and is not power efficiency. If DRX is configured, UE can take time to sleep during the time of network fetching data and then UE wakes up in the configured active window for DL data reception. It should be noted that for UE entering connected state, DRX mechanism can be enabled by network for UE normal DL/UL data exchanges. There is no reason that SDT UE has worse power consumption in RRC inactive state than the UE performing normal data communication in RRC connected state. In that sense, it is important to consider the power saving mechanism for Rel-18 MT-SDT, and DRX mechanism could be one of candidate schemes.

Proposal 8: RAN2 should study power saving mechanism such as to support DRX for MT-SDT.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: UE is not required to check the UL data volume for Small Data Transmission procedure triggered for MT-SDT.

Proposal 2: The SDT RSRP threshold for MT-SDT is configured separately from the Rel-17 SDT RSRP threshold.

Proposal 3: A time gap threshold is configured by network to control the maximum waiting time for the next CG-SDT occasion.

Proposal 4: When UE checking the SDT initiation conditions, UE should determine whether the time gap to the next CG-SDT occasion is within the time gap threshold configured by network.

Proposal 5: If the gap is not within the time gap threshold, UE should consider selecting RA resource for SDT instead of CG resource for SDT.

Proposal 6: UE is allowed to switch to RA based SDT procedure if no response is received for the initial CG-SDT transmission.

Proposal 7: RAN2 should study to have SR resource configuration for supporting UL data handling in MT-SDT procedure.

Proposal 8: RAN2 should study power saving mechanism such as to support DRX for MT-SDT.

# References

- R2-2306958, [Post122][309][MT-SDT] 38.321 Running CR (Huawei), Huawei, HiSilicon (Rapporteur).

- R2-2308405, Remining issues on control plane aspects of MT-SDT, Qualcomm.

# Appendix