3GPP TSG-RAN WG2 #123	R2-2308426

Toulouse, France, 2023-08-21 - 2023-08-25

Agenda Item:	7.13.7

Source:	Ericsson

Title:	SON Support for NPN

Document for:	Discussion, Decision

# 1	Introduction

During RAN2#119bis, the following were agreed:

- Include the NPN ID into SON/MDT report, whether SNPN ID or PNI-NPN ID related info should be included can be discussed per use case.

In RAN2#120 meeting, further discussion took place-

Agreements:

- Details of the checking of NPN IDs (e.g., Proposal 1 of R2-2211354) are FFS.

- Introduce SPNP ID (e.g., NID) to RLF/HOF report. Details of how to introduce it are FFS.

FFS: Introduce SPNP ID to logged MDT report.

FFS: Introduce PNI-NPN ID to RLF/HOF report. Details of how to introduce it are FFS .

FFS: Introduce PNI-NPN ID (e.g., CAG ID) to logged MDT report. Details of how to introduce it are FFS.

FFS: Discuss whether to introduce of new NPN specific variables for PNI-NPNs.

FFS: Discuss whether to introduce of new NPN specific variables for SNPNs.

During RAN2 #122 meeting the following FFS was left

FFS: Include the SNPN ID/CAG ID(s) in the logged MDT report or cell type indication (e.g., NPN cell).

This contribution aims to discuss the SON and MDT enhancements for private networks optimization including both SNPN and PNI-NPN scenarios considering the above agreements and the FFSs into account.

# 2	Discussion

## 2.1	Enhancements of SON Reports

In RAN2#120 meeting, the discussions have been around checking NID for RLF/HOF report. In our opinion, it should be extended to RA Report also.

As per current specifications, UE checks the PLMN information to determine if the stored variable should be cleared.

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report:

2> clear the information included in VarRA-Report;

Furthermore, UE checks the PLMN information before transmitting the RA report to network-

1> if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the RPLMN is included in plmn-IdentityList stored in VarRA-Report: 2> set the ra-ReportList in the UEInformationResponse message to the value of ra-ReportList in VarRA-Report; 2> discard the ra-ReportList from VarRA-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

However, if the UE moves between SNPN-1 and SNPN-2, current specifications does not restrict the UE to log RA report for SNPN-1 into RA report for SNPN-2 and send the report to SNPN-2 upon network request. This is against RACH principal and needs to be blocked at the UE.

UE checks if NID of the current SNPN matches the SNPN of the previously logged RA reports

  - Before logging a new RA report

  - Before transmitting a RA report to the network.

On the other hand, if two SNPNs are served by same cell and the cell has separate RACH resources allocated for each SNPN, it would be useful for the network to receive NID in the RA Report from the UE. The network can perform additional optimizations and load balancing between RACH resources based on the resource utilization reported by the UE.

UE logs NID in the RA report.

## 2.2	Wastefulness of MDT mechanism in co-existence of SNPN and PN

RAN3 has sent an LS [R3-232118] to RAN2; the content of the LS is copied here.

RAN3 noticed that logged MDT reports collected in one type of network, e.g., an SNPN, are lost when UE moves to a network of another type, e.g., a Public Network, due to the UE deregistering from the SNPN network. RAN3 believes that this would lead to a loss of the stored logged MDT reports if the reports are not retrieved before moving to the network of another type.

RAN3 would like to check with RAN2 if there are any solutions to avoid the loss of stored logged MDT reports upon moving from a network of one type to another type, even upon deregistration.

Considering the scenario that SNPN is deployed/managed by the enterprises independently from the PN, the data collection as part of MDT features lend itself special considerations when users connected to the SNPN moves frequently between SNPN and PN. According to the RAN3 observation in such scenario collecting data as part of MDT reports when SNPN and PN co-exist, becomes significantly inefficient specially when the UE moves back and forth between SNPN and PN. Therefore, when a user (holding a UE) moves frequently between SNPN and PN, the SON reports collected in one network (PN) should not be ruined by moving to the other network and vice versa.

A SNPN user (holding an SNPN allowed UE) may move frequently between SNPN and PN which may ruin collected measurements for one network in the other networks.

- The current logged MDT framework is wasteful when PN and SNPN co-exist.

In our opinion, a UE with subscription to both public and private networks i.e., SNPN, can log MDT reports associated to the SNPN in a new variable instead. Therefore, the MDT reports associated to PN is not overridden by the other network types i.e., SNPN. With this, the UE can report the associated logged MDT related information to a network wherein the event that triggered the report occurred. However, it should also be considered that resuming collection of logged MDT report once the UE is returned from SNPN to PN is an additional overhead on the UE side. One solution is that UE may delete the logged MDT configuration but store the collected report in a separate variable.

Upon moving to SNPN (and deregistration in PN) UE stores logged MDT report collected in PN in a separate variable, and releases the MDT configuration received in PN.

RAN2 to agree on the draft LS reply in the Annex

If the above proposals are agreeable RAN2 needs to discuss how to assure co-existence of the PN and SNPN MDT reports in particular whether the UE needs to consider additional memory or it is possible to share the existing memories between PN and SNPN MDT reports e.g., sharing 68KB of memory allocated to the logged MDT report or doubling the memory size i.e., 64 KB memory for PN MDT and 64 KB memory for NPN MDT.

RAN2 discuss whether to consider additional memory (e.g., additional 64KB) for co-existence of PN and SNPN MDT measurement results.

## 2.3	Enhancements for OoC events

In addition, some companies proposed to discuss out of coverage (OoC) in MDT report when UE is detecting coverage holes e.g., an area that no sufficient signal level is detected by UE for accessing specific services. In our opinion, when there is no suitable cell for a UE to camp on while the UE is requested to access a specific service (e.g., from upper layers), the UE logs an OoC indication in either RA report or CEF or new report. With this information, the network can identify the OoC for NPN. It is worthwhile to mention that particularly in scenarios that the upper layers (e.g., an SNPN service/application) request the UE to join an SNPN network and UE find no suitable cell for the requested SNPN, there might not be an appropriate logged MDT configuration from the same network at the UE to log the OoC event in the corresponding MDT report. Hence for such critical scenarios (in which the SNPN applications request the UE to register to an SNPN network) we cannot rely on the MDT reports. Hence, we propose:

RAN2 discuss to log an SNPN out-of-coverage (OoC) indication either in

  - an RA report, or

  - a CEF report, or

  - a new report.

## 2.4	Enhancements for MHI

In NR Mobility History information (MHI) report, if the UE is out of NR or LTE coverage, an indication is included to indicate that UE was out of NR or LTE coverage.

1> upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in 'any cell selection' state or 'camped on any cell' state in NR or LTE: 2> include an entry in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following: 3> set the field timeSpent of the entry as the time spent in 'any cell selection' state and/or 'camped on any cell' state in NR or LTE.

However, if the UE registers to SNPN, it is not necessarily out of coverage of NR. Thus, upon returning to PN, it is uncertain what it should log in the MHI report for NR.

If a UE served by PN, registers to SNPN and later returns to PN, it is unclear what is logged and reported by the UE in the MHI.

In this scenario, it is important to identify in the MHI of the PN that the UE was served by an SNPN and was not out of coverage. Hence, we propose the UE to log a timeSpent entry in the PN MHI containing time spent in the SNPN indicating it was connected to an SNPN.

UE logs time spent in the SNPN network in an entry in the existing PN MHI report.

Furthermore, as per current specification, if the UE has a MHI report available it includes indication in the RRCRsetupComplete and RRCResumeComplete messages without performing PLMN check. Upon receiving request from the network, UE transmits the MHI to the network. Hence, there is a possibility that UE may report SNPN MHI to PN and vice versa. Such reporting could be avoided if UE performs PLMN check before transmitting MHI report to the network.

UE performs PLMN check before transmitting MHI report to network.

## 2.5	Building coverage map for NPN and PN

One important use case of the logged MDT report is to build and analyze the coverage map for a given network. When both PN and NPN are deployed in an area and MDT reports are used to build coverage map of the area, it is important to the network knowing from the measurements in the MDT report which cells provide coverage to NPN and which cells provide coverage to PN. As of now the UE includes the measurements results of the neighboring cells associated with the PCI of the neighboring cells in the logged MDT report. Based on the current solution OAM is not capable to differentiate the network type based on the provided measurements (i.e., PCI and the RSRP measurements of the cells in an area). Hence it is not possible to build a coverage map for a specific network.

In the following, we discuss the required enhancements in the MDT report as well as MDT configuration, to enable the network to build and analyze the coverage map for a specific network e.g., PN or NPN.

    - Enhancement on the MDT report

In our understanding, since a UE only includes PCI and carrier frequency information for the neighbor cells in the logged MDT report (on frequency level), it is not possible for the network to identify whether the cell provides coverage to NPN or not (on cell level). This is shown in the following excerpt from the RRC specification.

MeasResultLoggingNR-r16 ::=          SEQUENCE {

physCellId-r16                       PhysCellId,

resultsSSB-Cell-r16                  MeasQuantityResults,

numberOfGoodSSB-r16                  INTEGER (1..maxNrofSSBs-r16) OPTIONAL

}

UE only logs the PCI and the radio measurements of the neighboring cells in the logged MDT report.

From the current logged MDT reports it is impossible for the network to deduce if the provided coverage is by a PN or an NPN cell.

By collecting logged measurements from the UE, where the network type is also included, the network can derive an accurate coverage map of each of the network types. This allows for optimization of network performance and better fulfilment of the service level agreements for each of the networks.

In addition the following FFS is captured in RAN2#122 meeting notes.

=>	FFS: Include the SNPN ID/CAG ID(s) in the logged MDT report or cell type indication (e.g., NPN cell).

Thus, to address the above FFS, we propose the following.

RAN2 to enhance the logged MDT report with cell type indication (e.g., NPN cell) as part of the measurement results.

If a UE configured with LoggedMeasurementConfiguration goes to anyCellSelection state, it logs the following information upon performing a PLMN check-

3> if the UE is in any cell selection state (as specified in TS 38.304 [20]): 4> set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found; 4> if the reportType is set to eventTriggered in the VarLogMeasConfig; and 4> if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport; and 4> if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig: 5> set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on; 5> set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

However, as per Rel-18 running CR, the UE can be configured with a CAGConfigList that includes a list of CAG identities. If the UE is camped normally on a cell that does not belong to the plmn-IdentityList provided in the LoggedMeasurementConfiguration but broadcasts CAG identities that are part of the CAGConfigList and goes to anyCellSelection state, i.e., UE goes to out of coverage, it will not log any information in the logged MDT report since it does not satisfy the PLMN checking condition. This is however, an important information for network to perform coverage analysis of the NPN network. Hence, we propose that upon going to anyCellSelection state, UE logs information of the last suitable cell if CAG identities broadcasted by the last suitable cell was part of the CAGConfigList.

UE in anyCellSelection state logs cell identity and measurements of the last suitable cell if the CAG identities broadcasted by the cell is part of the CAGConfigList received in LoggedMeasurementConfiguration.

    - Enhancement on the MDT Configuration

Similarly, a PN UE may collect MDT measurements on configured target frequencies for neighbor cells belonging to NPN when the cells operate on the same target carrier frequency. In other words, the UE collects both PN and NPN related MDT measurements in logged MDT report wherein two neighbor NPN and PN cells are sharing the target carrier frequency (e.g., configured as part of InterFreqTargetInfo). Therefore, building a coverage map for network types e.g., PN, or NPN for a given frequency may not be enough accurate.

As of now, building coverage map for an area (including both PN and NPN coverage) can be misleading for coverage analysis purpose, as MDT reports does not reflect the cell type (e.g., PN or NPN type), and a coverage of NPN maybe counted as coverage of PN and vice versa. Note that neighbouring cells measurement in MDT reports includes only PCI info of the cell.

We request RAN2 to discuss UE behavior for collecting logged MDT measurements concerning the target carrier frequency:

RAN2 to enhance the MDT configuration (interFreqTargetInfo) to enable logging only NPN or PN cells per frequency.

In addition, in the meeting RAN2#122 it has been agreed to include the CAG IDs in the MDT area configuration.

Agreements:

1	Include CAG ID(s) in the logged MDT area configuration.

This has been done based on the RAN3 progress in adding the NPN related area configuration in the area MDT configuration to enable logging MDT measurements when UE camps in PNI-NPN network. However, RAN3 has also agreed to include the SNPN related identities in the area configuration. We think RAN2 should enable the SNPN based configuration in the area configuration of the MDT. Therefore, we have the following proposal.

RAN2 to enhance the MDT area configuration with the list of SNPN identities as agreed in RAN3.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A SNPN user (holding an SNPN allowed UE) may move frequently between SNPN and PN which may ruin collected measurements for one network in the other networks.

Observation 2	The current logged MDT framework is wasteful when PN and SNPN co-exist.

Observation 3	If a UE served by PN, registers to SNPN and later returns to PN, it is unclear what is logged and reported by the UE in the MHI.

Observation 4	UE only logs the PCI and the radio measurements of the neighboring cells in the logged MDT report.

Observation 5	From the current logged MDT reports it is impossible for the network to deduce if the provided coverage is by a PN or an NPN cell.

Observation 6	As of now, building coverage map for an area (including both PN and NPN coverage) can be misleading for coverage analysis purpose, as MDT reports does not reflect the cell type (e.g., PN or NPN type), and a coverage of NPN maybe counted as coverage of PN and vice versa. Note that neighbouring cells measurement in MDT reports includes only PCI info of the cell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE checks if NID of the current SNPN matches the SNPN of the previously logged RA reports

a.	Before logging a new RA report

b.	Before transmitting a RA report to the network.

Proposal 2	UE logs NID in the RA report.

Proposal 3	Upon moving to SNPN (and deregistration in PN) UE stores logged MDT report collected in PN in a separate variable, and releases the MDT configuration received in PN.

Proposal 4	RAN2 to agree on the draft LS reply in the Annex

Proposal 5	RAN2 discuss whether to consider additional memory (e.g., additional 64KB) for co-existence of PN and SNPN MDT measurement results.

Proposal 6	RAN2 discuss to log an SNPN out-of-coverage (OoC) indication either in

a.	an RA report, or

b.	a CEF report, or

c.	a new report.

Proposal 7	UE logs time spent in the SNPN network in an entry in the existing PN MHI report.

Proposal 8	UE performs PLMN check before transmitting MHI report to network.

Proposal 9	RAN2 to enhance the logged MDT report with cell type indication (e.g., NPN cell) as part of the measurement results.

Proposal 10	UE in anyCellSelection state logs cell identity and measurements of the last suitable cell if the CAG identities broadcasted by the cell is part of the CAGConfigList received in LoggedMeasurementConfiguration.

Proposal 11	RAN2 to enhance the MDT configuration (interFreqTargetInfo) to enable logging only NPN or PN cells per frequency.

Proposal 12	RAN2 to enhance the MDT area configuration with the list of SNPN identities as agreed in RAN3.

# References

[RP-221825], Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC, 3GPP TSG RAN meeting #96, Budapest, Hungary, June 6-9, 2022.

# Annex

## Reply LS to RAN3

3GPP TSG-RAN2 Meeting #123-	R2-23xxxx

Toulouse, France 21st Aug- 25th Aug 2023

Title:	Reply LS on potential override of logged MDT reports upon moving from SNPN to PLMN

Response to:	R2-2304628 (R3-232118)

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	TSG RAN WG2

To:	TSG RAN WG3

Cc:	-

Contact Person:

Name:	 Ali Parichehreh

E-mail Address: ali.parichehreh@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In the LS R2-2304628/R3-232118, it mentions:

RAN3 noticed that logged MDT reports collected in one type of network, e.g., an SNPN, are lost when UE moves to a network of another type, e.g., a Public Network, due to the UE deregistering from the SNPN network. RAN3 believes that this would lead to a loss of the stored logged MDT reports if the reports are not retrieved before moving to the network of another type.

RAN3 would like to check with RAN2 if there are any solutions to avoid the loss of stored logged MDT reports upon moving from a network of one type to another type, even upon deregistration.

RAN2 reply:

RAN2 thanks RAN3 for the questions. RAN2 has analysed the scenario and agrees on the below solution-

- UE stores logged MDT report in separate variables for PN upon moving to the SNPN. Thus, collecting logged MDT report in SNPN would not delete any potentially collected report in PN.

- To reduce complexity, UE would not store the logged MDT configuration in separate variables. Thus, if a UE is configured with logged MDT configuration in public network (PN), moves to a SNPN and receives a new logged MDT configuration in SNPN, it will overwrite the public network’s logged MDT configuration.

2. Actions:

To RAN WG3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take account of the feedback.

3. Date of Next TSG-RAN WG2 Meeting:

3GPP RAN2#123	from 2023-08-21	to	2023-08-25		Toulouse , FR

3GPP RAN2#123-bis	from 2023-10-09	to	2023-10-13		Xiamen, CN