3GPP TSG-RAN WG2 Meeting #123	R2-2308444

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.12.2.1

Source:	Ericsson

Title:	Remaining issues for supporting RACH-less in mobile IAB

Document for:	Discussion, Decision

# 1	Introduction

In the last RAN2#122 meeting, the following agreement has been reached regarding the support of RACH-less handover for mobile IAB.

RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured).

For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

Send LS to RAN3 to check whether there are issues / feasibility concerns

Rapp suggest to discuss the cond HO event T1 that was left FFS

-	Chair wonder how the network can know if the UE has access to absolute time.

-	LGE think we can mandate that the UE is GNSS capable.

-	QC think that non-GNSS UEs can use SIB9.

-	CATT think timing may not be so important, this has not been clarified. Nokia think speading out handovers by this mechanism is not needed.

-	Chair: it seems the support is limited, and the benefits not so clear. However there is also some support and maybe impact is very low. If time, can maybe revisit (later, not next meeting)

No conclusion for now

In this contribution we will address the remaining issues for supporting RACH-less handover for mobile IAB.

# 2	Discussion

According to the agreement taken in the last RAN2 meeting, a RACH-less handover for mobile IAB may only be done on cell that are known at the UE. According to TS 38.133:

“a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown”

On the other side, another agreement taken in the last RAN2 meeting is the following:

R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

Since the original understanding in RAN2 is that RACH-less handover for mobile IAB may only be done on cell that are known at the UE, the only way for the network (i.e., second logical DU) to know on which beam the UE should be handed off is only based on measurements reported by the UE.

For RACH-less handover in mobile IAB, the network knows/selects a beam to indicate to the UE only based on the UE measurement report.

A further FFS to be discussed based on the taken agreement is about the handling of the supervision timer and when HO is considered successfully complete. Since RACH-less handover is discussed in multiple Wis (e.g., mobility and NTN) and this problem is common to all the other WI, it would be good to align the outcome, whenever possible. According to the discussions taken so far, it seems that two options are mainly on the table.

- Timer stopped at the first C-RNTI addressed PDCCH transmission.

- Timer stopped after receiving the RLC ACK of the RRCReconfigurationComplete message.

Anyway, the IAB WI can wait for the other WIs to make a final decision and can align its outcome, whenever possible.

RAN2 to consider the following two options for stopping the supervision timer for RACH-less handover in mobile IAB:

  - Timer stopped at the first C-RNTI addressed PDCCH transmission

  - Timer stopped after receiving the RLC ACK of the RRCReconfigurationComplete message.

The IAB WI to align the decision on when to stop the supervision timer for RACH-less handover in mobile IAB according to the outcome of the discussions in the other WI (e.g., mobility and/or NTN).

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For RACH-less handover in mobile IAB, the network knows/selects a beam to indicate to the UE only based on the UE measurement report.

Proposal 2	RAN2 to consider the following two options for stopping the supervision timer for RACH-less handover in mobile IAB:

a.	Timer stopped at the first C-RNTI addressed PDCCH transmission

b.	Timer stopped after receiving the RLC ACK of the RRCReconfigurationComplete message.

Proposal 3	The IAB WI to align the decision on when to stop the supervision timer for RACH-less handover in mobile IAB according to the outcome of the discussions in the other WI (e.g., mobility and/or NTN).

# 4	References

RP-221815, WID on Mobile IAB for NR, 3GPP TSG RAN#96, Budapest, Hungary, June 2022