3GPP TSG-RAN2#123	R2-2308461

Toulouse, France, August 21-25, 2023

Agenda item:		7.22.3

Source:		Lenovo

Title:		 LP-WUS in RRC Connected Mode

Document for:		Discussion and Decision

- Introduction

Currently, the UE’s Main Radio (MR) needs to periodically wake up once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it is turned on [1]. The following agreements have been made at the RAN2 #122 meeting:

Agreements

- RAN2 expect that different coverage LR/MR may have RAN2 impact, e.g., UE need to stop using LP-WUS when moving out of LR coverage, other aspects FFS. What to cover (if anything) in TS 38.304 is FFS.

- For UE in RRC_IDLE/RRC_INACTIVE state, it is FFS to what extent the network is or need to be aware of which receiver the UE uses MR/LR or both (for paging reception etc). A potential drawback of not knowing could be increased LP WUS load, a potential drawback of awareness is increased signalling.

- RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. 
(Other control methods not precluded)

- RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false alarms rate (depend on L1 capacity to carry payload).

This contribution looks at the possible RRC Connected Mode procedures for an LP-WUR enabled UE including the use of LP-WUS in Connected Mode DRX and the use of LP-SS for serving cell measurements.

- Discussion

  - LP-WUS for Connected-Mode DRX

As mentioned above, it has been agreed in the previous meeting that the UE will use LP-WUS when some preconfigured condition(s) are fulfilled. As per the R16 power saving item, in RRC Connected Mode, the MR may be configured with a DCI 2_6 (Wake-Up Signal). This DCI format 2_6 is monitored outside of the DRX active time to decide whether the MR needs to be woken up in the next DRX active time. RAN2 should discuss how the UE may use the LP-WUS to wake up the MR such that the power saving gain achieved by using the LR must be higher than the power saving gain achieved by using only the DCI format 2_6, at the least.

In this contribution, we propose two alternatives to use the LP-WUS to wake up the MR either by itself or in conjunction with the existing DCI format 2_6.

Alternative 1: LP-WUS replaces the DCI format 2_6 to wake up the MR.

In this alternative, the LP-WUS signal is used to wake up the MR when there is some data available for the UE, i.e., the MR is woken up in the next DRX ON Duration. If there is no data available for the UE, the MR is allowed to continue remaining in sleep mode, where the sleep mode may be micro/ light/ deep sleep. Here, the LP-WUS replaces the DCI format 2_6 signal. Here, the power saving gain achieved is maximum as the MR only wakes up when there is data available for it, and the LP-WUS is used to indicate this. With this alternative, the fallback mechanism when UE moves out of LR coverage can be to use the legacy DCI 2_6 signal.

Alternative 2: LP-WUS can indicate to MR whether to monitor the DCI 2_6 signal that determines if the MR enters DRX ON Duration.

In this alternative, the LP-WUS signal can be used to indicate whether the MR needs to monitor for the DCI 2_6 signal outside of the DRX active time. If the LP-WUS indicates to monitor for DCI 2_6, the DCI 2_6 signal then determines whether the MR enters DRX ON Duration in the next DRX cycle.  If the LP-WUS indicates that DCI 2_6 need not be monitored, the MR remains in sleep mode where sleep mode may be micro/ light/ deep sleep. With this alternative, the power saving gain is slightly reduced in comparison to Alternative 1. This alternative may be beneficial when combined with subgrouping such that a group of UEs that are expecting similar data traffic at the same time could be indicated by the LP-WUS to monitor for DCI 2_6 and the remaining UEs may continue to sleep.

Hence, RAN 2 should discuss how LP-WUS may be used for DRX in RRC Connected Mode.

RAN2 to discuss and evaluate the following alternatives for the use of LP-WUS in DRX:

- Alt 1:  LP-WUS replaces DCI 2_6 signal.

- Alt 2: LP-WUS is used in conjunction with the DCI 2_6 signal.

  - Serving Cell Measurements

When the UE is in RRC Connected Mode, it is configured to perform intra-frequency and inter-frequency measurements by means of some dedicated signaling. The MR needs to stay awake to measure the synchronization signals and report them back to the network which leads to certain power consumption by the UE. This power consumption could be reduced by allowing the LR to perform the serving cell measurements by means of a Low-Power Synchronization Signal (LP-SS). When the LR performs these measurements, the MR may remain in sleep mode allowing for increased power saving gain.

- The power consumed by UE for serving cell measurements can be reduced by using the LR to measure LP-SS.

In the previous meetings, it has also been agreed that RAN2 can assume different coverage for LR and MR. As the coverage of LR is lower than the MR, the LP-SS may not always be used to perform serving cell measurements. For this purpose, we propose that the LR is configured with a threshold (configurable by the network) for performing serving cell measurements using LP-SS. If the signal strength as measured by LR falls below this threshold, the MR can be woken up to perform the serving cell measurements.

RAN2 to discuss the use of LP-SS for serving cell measurements.

Network configures the LR with a threshold for performing serving cell measurements using LP-SS.

# Conclusion

This document discusses some procedures for a LR capable UE in the RRC Connected Mode. Following are the proposals and observations made in this document:

- Proposal 1: RAN2 to discuss and evaluate the following alternatives for the use of LP-WUS in DRX:

- Alt 1:  LP-WUS replaces DCI 2_6 signal.

- Alt 2: LP-WUS is used in conjunction with the DCI 2_6 signal.

- Observation 1: The power consumed by UE for serving cell measurements can be reduced by using the LR to measure LP-SS.

- Proposal 2: RAN2 to discuss the use of LP-SS for serving cell measurements.

- Proposal 3: Network configures the LR with a threshold for performing serving cell measurements using LP-SS.

# References

[1] 3GPP TR 38.869 v0.2.0: Study on low-power wake up signal and receiver for NR (Release 18)