3GPP TSG-RAN WG2 Meeting #123	R2-2308480

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.2.2

Source:	Ericsson

Title:	Sidelink positioning

Document for:	Discussion and Decision

# 1	Introduction

The work items for SL in RAN2 include

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

In last meetings, RAN2 progressed with below agreements;

RAN2 122

Agreements:

SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast.

Inform SA2 about our agreements on sidelink positioning, with “take into account” action.

SLPP carried over NAS is used between UE and LMF. FFS on how to manage the session/transaction.

Agreements:

Anchor UE selection is supported by information about the candidate anchor UEs.  At least the following list can be discussed for use in anchor UE selection:

1.	UE roles

2.	Supported positioning method

3.	In coverage or not

4.	RSRP

5.	LOS/NLOS

6.	Location

7.	PLMN

A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified.

RAN2 impact of this information to be determined.

FFS which information would be determined statically/dynamically.

Agreements:

SLPP can support multiple target UEs in the same session when LCS requests.

RAN2 will not specify group management for multiple target UEs.  RAN2 assumption is that a group ID will be provided from upper layers.

FFS how session IDs are managed between multiple UEs.

RAN2 121bis

Agreement:

LS on group positioning is postponed for a reply from next meeting.

WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2.

WA: At least part of the group management for group positioning is performed at upper/application layer.

Agreement:

Sidelink Positioning Architecture in Figure 1-4 on R2-2304301 is taken as baseline in TS 38.305 for further discussion.

RAN2 understanding is that there is no impact to LTE specs from this objective.

Agreement:

Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2.  Server UE can be further discussed at least for the case that the server UE is separate from the target and anchor.

Agreement:

Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario.

Agreement:

Anchor UE selection can be included in the sidelink positioning procedures at least for out of coverage scenario.

Agreements:

RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles.  FFS how much information is indicated about anchor UEs (e.g., knowledge of location).

The UE role information is indicated in the discovery SLPP metafield.  FFS if this applies to both discovery modes and which messages.

Agreements:

R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location related measurements/location estimates, to transfer assistance data, or to exchange of capabilities.

RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request.

TP in R2-2304005 is postponed.

RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response)”

RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

In this paper, we discuss the 1) SL positioning architecture 2) SLPP procedures for OoC 3) location server in IC 4)SL positioning parameter for discovery 5) multiple target UE positioning 6) potential signalling procedures for LPP extension to Hybrid Positioning (Joint PC5-Uu scenario) 7) hybrid positioning measurement report 8) Partial Coverage 9) LMF to UE protocol Discussion 10) SL ranging measurement when sync source changes 11) Overhearing

# 2	Discussion

## 2.1 SL positioning architecture

Figure 1: Figure 1-4 on R2-2304301

In the previous meeting, the Figure 1-4 from R2-2304301 was agreed to be taken as baseline for further discussion. There are two connections marked as FFS. We consider the connection marked as “FFS LTE-Uu” not needed, as the scope of SL positioning is NR. We do not see that even the LTE transport is in the scope. The other FFS link should also be removed as there is no strong motivation to keep it. Since, there is already NR PC5 connection between UE A and UE C, and between UE A and UE B. Hence, there is further no need to keep between UE C and UE D.

Figure 2 UE Positioning Overall Architecture applicable to NG-RAN

RAN2 to consider modification of Figure 2 on top of the baseline architecture figure.

## 2.2 SLPP procedures in OoC

Figure 3 SLPP procedure in OoC. UE1 and UE2 can be target UE, anchor UE, or SL positioning UE server. Target UE or anchor UE can act as SL Positioning Server UE, and the procedures involving SL positioning UE server are marked in dash lines.

- Target UE discovers candidate anchor UE or SL positioning UE server.

- SL positioning UE server request capabilities from target UE and/or anchor UE, and target UE and/or anchor UE send capability to SL positioning UE server.

- Anchor UE(s) is selected.

Note: it is FFS whether SL positioning UE server and/or target UE decide the anchor UE

- SL positioning UE server and target UE and/or anchor UE exchange assistance data including the positioning method to use and the SL-PRS config to measure.

- Target UE and/or anchor UE performs the Ranging/SL Positioning procedures.

- Target UE and/or anchor UE sends the measurement/positioning information to SL positioning UE server.

- SL positioning UE server calculates the Ranging/SL positioning results based on the received ranging/SL positioning measurements and location information received from target UE and/or anchor UE sends.

Take the procedure in Figure 3 as baseline SLPP procedures.

## 2.3 In-Coverage (IC) Scenario

In IC scenario, target UE and anchor UE(s) are all in network coverage. LMF establishes SLPP session with target UE via SLPP for capability exchange, AD distribution, obtaining measurement, and ranging/SL positioning calculation, LMF can also establish SLPP session with anchor UE that are PRU to perform these procedures. For non-PRU anchor UEs, target UE can forward their messages to LMF and the message from LMF to them. Thus in IC, there is no need to include SL positioning UE server.

SL positioning UE server is not involved for in-coverage SL positioning, and no LMF to SL positioning UE server interaction to be supported in Rel-18

## 2.4 Sidelink positioning parameters in discovery signalling

In the last meeting, RAN2 has agreed that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE and server UE roles. Besides the agreed UE roles, for Hybrid Positioning (Uu+SL) we see the need that Anchor/Reference UE broadcast whether it is PRU capable or not. In this way, the Target UE should be able to identify a PRU UE during discovery procedure and inform this to LMF to initiate hybrid Positioning (Uu+SL). This discovery procedure can be initiated also upon LMF request.

Anchor/Reference UE indicate whether it is PRU capable or not in the SL positioning discovery message.

Besides indicating UE roles in the SL positioning discovery message, anchor UE indicating its supported QoS and target UE indicating the required QoS is beneficial to save latency and resources. Moreover, indicating the SL positioning applies in both discovery mode.

Target UE may include QoS requirement for SL positioning in discovery message, and anchor UE may include supported QoS for SL positioning in discovery message.

Sidelink positioning parameters can be indicated in both discovery modes.

## 2.5 Multiple target UEs positioning

From last meeting, RAN2 has come to the working assumption is group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request. The next questions would be how AMF deals with the LCS request. If AMF forward the LCS request transparently to LMF or AMF can divide the LCS request to multiple requests for each target UE to LMF, and the latter has no impact for LMF or LPP session. For either way, the multiple UE IDs in the LCS request should be provided to AMF and LMF.

For multiple target UEs positioning, the multiple target UE IDs in LCS request should be provided to AMF and LMF.

## 2.6 Hybrid Positioning

There are different use cases which require LMF to perform hybrid positioning such as request from external client to be able to provide the range between two UEs or improve the UE positioning accuracy with assistance from another UE such as reference UE/PRU. For ranging determination, to improve the reliability and accuracy of range estimation; LMF can perform hybrid positioning such as to obtain the absolute UE location and also range estimation between UEs.

For SL ranging/positioning, signalling procedures including capability exchange, reference/anchor UE selection, assistance data transfer, ranging/sidelink positioning measurement, report and calculation are needed. In RAN2 122, it is agreed that SLPP carried over NAS is used between UE and LMF.

An LPP session and SLPP session between the target UE and LMF are set up for hybrid positioning. If there is discovered PRU anchor UE(s) associated with LMF, LMF may initiate separate SLPP session(s) with PRU anchor UE(s) to send AD and/or request measurement. As the session ID are correlation ID assigned by AMF for non-periodic or triggered positioning session, whether same session ID or different session IDs are assigned for the above sessions, the association between SLPP session(s) and Uu session are maintained by LMF. The transaction ID(s) for the above sessions to target UE and PRU anchors may be set to different range or kept same for the same request and response actions, which is also determined and maintained by LMF. An example procedure for hybrid positioning is illustrated in Figure 4.

9. LMF requests Uu and/or SL measurement with session ID X and transaction ID k

UE

Target UE

LMF

PRU

5. Device & Service Discovery

8. Setup SLPP Positioning Session with session IDs Y, Z with anchor PRUs, Provide SL AD with Session IDs Y, Z

15. Compute UE Location using Uu & SL Measurements with Session IDs X, Y, Z,...

2. Uu LPP positioning Measurement with session ID X

3.  Determine If SL Measurements needed

4. Request to initiate SL procedure & provide AD(potential anchor UEs) with session ID X

7. Along with AMF Resolve SUPI from App/L2 ID

10. LMF request Uu and/or SL measurement in LPP and SLPP with session IDs Y, Z and transaction ID k

6. SLPP report discovered anchor UE(s) ID(s) with session ID X

1. Uu LPP positioning Request with session ID X

13. Target UE responds with Uu and/or SL measurement in LPP and SLPP with session ID X and transaction ID k

14.  PRU Anchor UEs respond with SLPP measurement with session IDs Y, Z and transaction ID k

UE

UE

11. SL-PRS transmission and measurement

12. Anchor UE(s) forward SL measurement in SLPP

9. LMF requests Uu and/or SL measurement with session ID X and transaction ID k

UE

Target UE

LMF

PRU

5. Device & Service Discovery

8. Setup SLPP Positioning Session with session IDs Y, Z with anchor PRUs, Provide SL AD with Session IDs Y, Z

15. Compute UE Location using Uu & SL Measurements with Session IDs X, Y, Z,...

2. Uu LPP positioning Measurement with session ID X

3.  Determine If SL Measurements needed

4. Request to initiate SL procedure & provide AD(potential anchor UEs) with session ID X

7. Along with AMF Resolve SUPI from App/L2 ID

10. LMF request Uu and/or SL measurement in LPP and SLPP with session IDs Y, Z and transaction ID k

6. SLPP report discovered anchor UE(s) ID(s) with session ID X

1. Uu LPP positioning Request with session ID X

13. Target UE responds with Uu and/or SL measurement in LPP and SLPP with session ID X and transaction ID k

14.  PRU Anchor UEs respond with SLPP measurement with session IDs Y, Z and transaction ID k

UE

UE

11. SL-PRS transmission and measurement

12. Anchor UE(s) forward SL measurement in SLPP

Figure 4 Hybrid positioning procedure.

Step 1. A Uu LPP positioning request with session ID X is either sent from LMF to target UE.

Step 2. Target UE reports Uu measurement to LMF with session ID X.

Step 3. Based on the Uu measurement from target UE in step 2, LMF decide if SL measurement is needed. If not needed, LMF calculates target UE’s position. If needed, continue to below steps.

Step 4. LMF sends request to target UE to initiate SL procedure & provide AD(potential anchor UEs)

Step 5. Target UE discover anchor UEs.

Step 6. Target UE report discovered anchor UE(s) ID(s) to LMF using Uu LPP. The UE IDs may be App ID, SUPI or GPSI.

Step 7. For App ID, along with AMF, LMF resolve SUPI of the anchor UEs from their App ID.

Step 8. If there is discovered PRU anchor UE(s) that is associated with LMF, LMF may set up SLPP Positioning Session with anchor UEs with session IDs Y, Z, Provide SL AD with Session IDs Y, Z. The session ID Y, Z may be same/equal as X.

Step 9. LMF requests the target UE to perform Uu and/or SL measurement in LPP and SLPP with session ID X and transaction ID k.

Step 10. LMF requests the anchor UEs to perform SL measurement in SLPP with session IDs Y, Z and transaction ID k.

Step 11. Target UE and anchor UEs including anchor PRUs performs SL-PRS transmission and measurement.

Step 12. Anchor UE(s) forward the SLPP measurement to target UE.

Step 13. Target UE responds to LMF with Uu and/or SL measurement in LPP and SLPP with session ID X and transaction ID k

Step 14. Anchor PRUs responds to LMF with SLPP measurement with session IDs Y, Z and transaction ID k

Step 15. LMF computes target UE Location using Uu & SL Measurements with Session IDs X, Y, Z and transaction ID k

The target UE shall discover anchor UE/PRU and report to LMF and LMF can setup SLPP positioning session with the discovered PRU directly to send AD or receive SLPP measurement report.

LMF sets up LPP session with target UE and SLPP sessions with multiple UEs and obtains both Uu and SL measurements for hybrid positioning. LMF maintains the session ID(s) and transaction IDs for the hybrid positioning session.

To implement SL ranging/positioning, discovery procedure is needed to establish PC5 path, thus LMF should be able to request ProSe Capability from UE. For SL ranging/positioning, the UE capability to transmit SL-PRS, measure and calculate for each detailed SL ranging/positioning method such as RTT, SL-TDOA should also be defined and supported by LPP.

- LPP should support to request and provide 5G ProSe Capability.

- The PC5 Capability for Ranging/SL positioning should be defined, and LPP should support to request and provide PC5 Capability for Ranging/SL positioning.

## 2.7 Hybrid positioning measurement alignment in time domain

Hybrid positioning collects both Uu and SL positioning measurement to determine and/or improve UE’s positioning/ranging result. However, Uu and SL measurement periods cannot always be aligned. Furthermore, the availability of the SL PRS for the SL positioning measurements depends on the SL resource pool configuration e.g. slots which are configured for the SL. Therefore, there is no guarantee that the Uu and SL positioning measurements can always be completed by the UE during the same time or even close in time. They will most likely be misaligned. When the target UE moves, misalignment between Uu and SL positioning measurements will cause degradation to the positioning accuracy, or even fail to compute positioning results with hybrid measurements.

When the target UE has non-zero velocity, misalignment between Uu and SL positioning measurements will cause degradation to the positioning accuracy, or even fail to compute positioning results with hybrid measurements.

To better align the Uu and SL measurement in the time domain, NW may instruct UE by sending assistance data to UE of the Uu and SL reference signals that are close in time domain and/or request UE to only measure Uu and SL reference signals within a certain time interval.

NW provides configuration to UE for the Uu and SL reference signals that are close in time domain and request UE to measure Uu and SL reference signals within a certain time interval.

## 2.8 Partial Coverage

Partial coverage is a way to provide the out of coverage UE (remote UE) cellular coverage by means of a relay UE (U2N relay). It is possible that anchor UE, reference UE etc. can also act as relay UE. As provided in the below Figure, one option is to use Layer 2 U2N relay where remote UE can be reached by the LMF using LPP.

Figure 5 LPP Protocol Stack with L2 NW-to-UE relay for PC

Partial coverage is supported by means of L2 U2N Relay where the LPP protocol terminates/originates at remote UE and LMF.

## 2.9 SL Ranging measurements when Synch Source or Coverage changes

For SL positioning/ranging measurements, accurate synchronisation between UEs is essential. The sidelink synchronization reference source can change for different reasons, e.g. due to UE mobility, channel conditions etc., while UE is performing SL positioning measurements which may lead inaccurate measurements. How would UE cope when the synch source during positioning measurement changes? Additionally, another question would be: UEs may be (pre)configured resources to perform SL ranging measurements in different coverage scenario (IC, PC, OOC). A group of UEs (at least 2 UEs; e.g. car platooning) may perform ranging continuously; however in the coverage crossing area (IC to OOC or vice versa) where UEs may have performed the measurement in one coverage but now happen to be in different coverage; would the ongoing measurements be valid? The impact on change of coverage and synchronization reference source while performing the SL positioning measurements needs to be investigated.

The Synchronization and measurement validity are RAN4 related question which should be provided by RAN4. Depending upon the outcome of the results, the SL procedure would have impact, such as whether to continue the SL procedure or restart the procedure.

Send LS to RAN4 requesting how would the ongoing SL measurements be impacted when the synch source changes and/or coverage status changes.

## 2.10 Overhearing

As part of the Rel-18 work item on sidelink (SL) positioning, a new SL positioning reference signal (SL-PRS) will be specified to be used for measurements between UEs. PRU is UE with known location. It would benefit if PRU is also involved in the ranging/SL positioning process to provide extra measurement by overhearing the SL-PRS by the target UE(s). For example, it can improve the accuracy if the target UE performing ranging are in NLOS condition, and can validate the positioning result by the overhearing measurement as PRU’s location is known.

Figure 6 and Figure 7 show the mechanism of PRU overhear SL-PRS requested by LMF and target UE respectively.  From Figure 6 when the PRU is requested by LMF to overhear, it causes no extra overhead to the target UEs.

UE1

UE2

PRU(s)

LMF

3. LPP: Request to PRU            to listen/ overhear UE(s) within coverage

4. LPP: Provide an ACK/ NACK

7. LPP: Report the result

6. SL-PRS measurements

1. UEs reserve SL-PRS

2. LMF request and UEs in coverage provide SL-PRS configuration

5. LPP: Forward the SL-PRS configurations to overhear

Figure 6 LMF requests PRU to overhear SL-PRS

Step 1: UEs performing SL positioning/ranging reserve SL-PRS by either scheme.

Step 2: LMF requests for UEs that are in coverage to send their SL-PRS configuration, and UEs provide LMF the SL-PRS configuration.

Step 3: LMF request PRU to measure SL-PRS transmitted by the UEs via LPP.

Step 4: PRU reply to the request of LMF via LPP.

Step 5: LMF forward the SL-PRS configuration to PRU.

Step 6: PRU measures SL-PRS.

Step 7: PRU send the measurement results to LMF.

UE1

UE2

Listening UEs

LMF

9a. LPP: Report the result

8. SL-PRS measurements

1. UEs reserve SL-PRS

2. LMF sends AD about potential Listening UEs (PRUs)

3. PC5: UEs discover listening UEs (PRU(s))

5. PC5: Provide an ACK/ NACK

4. PC5: Request to UE to listen/overhear SL-PRS and send SL-PRS configurations

7. Establish LPP session

6. LPP: Forward the ID(s) of PRU(s)

9b. PC5: Report the result

10. LPP: Forward the measurements from PRU

Figure 7 target UE requests PRU to overhear SL-PRS.

Step 1: UEs performing SL positioning/ranging reserve SL-PRS by either scheme.

Step 2: Optionally, LMF may send potential PRUs to the target UEs.

Step 3: Target UEs discover the PRUs if they have not done it from previous SL communication or SL positioning.

Step 4: Target UEs request PRU to measure/overhear SL-PRS and send the SL-PRS configuration via PC5.

Step 5: UE/PRU reply to the request from the target UE.

If the ranging/SL positioning result is to be collected by LMF then

Step 6: Target UE forward the L2 ID or Application ID to LMF.

Step 7: LMF retrieve the SUPI of PRU with the help of AMF. LMF establish a LPP session with PRU.

Step 8: UE/PRU measures SL-PRS.

Step 9: UE/PRU send the measurement results to LMF and/or target UE based on the request received from LMF or target UE.

Step 10. If 9b is performed, target UE may forward the measurements to LMF and the information about PRU including PRU’s location. In this case, Step 6 and 7 are not required.

UE overhearing SL-PRS benefit the ranging/SL positioning accuracy causing no or little overhead to the UEs.

UE overhearing SL-PRS is supported.

# 3	Conclusion

Observation 1	When the target UE has non-zero velocity, misalignment between Uu and SL positioning measurements will cause degradation to the positioning accuracy, or even fail to compute positioning results with hybrid measurements.

Observation 2	UE overhearing SL-PRS benefit the ranging/SL positioning accuracy causing no or little overhead to the UEs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to consider modification of Figure 2 on top of the baseline architecture figure.

Proposal 2	Take the procedure in Figure 3 as baseline SLPP procedures.

Proposal 3	SL positioning UE server is not involved for in-coverage SL positioning, and no LMF to SL positioning UE server interaction to be supported in Rel-18

Proposal 4	Anchor/Reference UE indicate whether it is PRU capable or not in the SL positioning discovery message.

Proposal 5	Target UE may include QoS requirement for SL positioning in discovery message, and anchor UE may include supported QoS for SL positioning in discovery message.

Proposal 6	Sidelink positioning parameters can be indicated in both discovery modes.

Proposal 7	For multiple target UEs positioning, the multiple target UE IDs in LCS request should be provided to AMF and LMF.

Proposal 8	The target UE shall discover anchor UE/PRU and report to LMF and LMF can setup SLPP positioning session with the discovered PRU directly to send AD or receive SLPP measurement report.

Proposal 9	LMF sets up LPP session with target UE and SLPP sessions with multiple UEs and obtains both Uu and SL measurements for hybrid positioning. LMF maintains the session ID(s) and transaction IDs for the hybrid positioning session.

Proposal 12	NW provides configuration to UE for the Uu and SL reference signals that are close in time domain and request UE to measure Uu and SL reference signals within a certain time interval.

Proposal 13	Partial coverage is supported by means of L2 U2N Relay where the LPP protocol terminates/originates at remote UE and LMF.

Proposal 14	Send LS to RAN4 requesting how would the ongoing SL measurements be impacted when the synch source changes and/or coverage status changes.

Proposal 15	UE overhearing SL-PRS is supported.

# 4	References

- [1] TR 23.700