3GPP TSG-RAN WG2 Meeting #123	R2-2308495

Toulouse, France, August 21 – 25, 2023

Agenda Item:	7.25.1

Source:	Ericsson

Title:	Support of lower MSD capability

Document for:	Discussion, Decision

# 1	Introduction

A follow up LS was received from RAN4 (R4-2310276) on lower MSD capability. This contribution discusses initial signaling aspects considering the follow up LS and previous input received by RAN4 (R4-2306594).

# 2	Discussion

The previous LS from RAN4 (R4-2306594) seems to focus on cases of 2 bands and 3 bands, i.e. an UL band (or two UL bands) that affect a certain DL band, where the UE may want to indicate support of lower MSD capability. Despite this focus on a small set of bands, it is beneficial to design the signaling such that it would not require the UE to include an explicit fallback band combination to indicate support of this feature. As RAN4 points out, there are many factors and group of bands that could result in different MSD values, which could easily fill in the UE capability size if fallback band combinations would be explicitly reported for the sake of indicating different lower MSD capabilities.

Lower MSD capability design should not imply in the inclusion of fallback band combinations for the purpose of indicating different MSD capability.

- MSD for different power classes

-	UE reports the lower MSD capability class per MSD type for the highest supported power class for the band combination

▪	UE can additionally report lower MSD capability class per MSD type for other power classes if requested by the network/regulator

- MSD orders

-	No need to report the order for harmonic/ harmonic mixing/cross band isolation MSD types

▪	Lower MSD capability class reported should apply for all specified orders of the MSD type

-	IMD order is up to 5 in Rel-18

- MSD types

-  The following MSD types/orders are agreed to be reported based on the existing spec

▪	Harmonic, harmonic mixing, crossband isolation, IMD with order=2/3/4/5

-  It is agreed to add a new special lower MSD type as “ALL”, which means all above mentioned MSD types/orders could meet the reported lower MSD threshold/lower MSD capability class

▪	FFS on details of the “ALL” type

-  Other new MSD types may be added later

The follow up RAN4 LS states the following:

In the yellow highlighted aspects above, it is expected that a lower MSD capability is reported for each MSD type in a band combination, while at least 7 types are defined by RAN4 so far. This signaling on per band combination level can generate a significant overhead even if fallback band combinations are not reported for the sake of this feature. Note also that according to the LS other MST types may be added later, which may increase even more the signaling for this feature.

In RAN2#122, it was raised that the signaling could be defined outside the band combination list. This approach could achieve the goal of reducing the signaling size and also avoid inclusion of fallback band combinations for the purpose of indicating different MSD capability. This can also be more easily expandable in case further MSD types would be defined. Therefore, unless any outstanding issue is identified, RAN2 should assume to define the signaling outside the band combination list.

RAN2 assumes to define the signaling for lower MSD capability outside the band combination list.

The blue highlighted aspect in the LS hints that a filtering of UE capabilities should be defined for this feature. However, it should be ultimately up to RAN2 whether to include a UE capability filtering. To base this decision, more information is required on which additional support of MSD capability is needed. If the signaling is designed outside the band combination list, RAN2 could as well just add such additional support cases rather than have a UE capability filter.

It is up to RAN2 whether to add UE capability filtering, which may not be required for this feature if it is designed outside the band combination list.

The green highlighted aspect in the LS states that the lower MSD capability class reported should apply for all specified orders of the MSD type. This can work for the currently specified orders of MSD types, but RAN2 may need to design additional signaling for any order specified after the introduction of the lower MSD capability, since a UE reporting this feature cannot be assumed to support any newly specified order for a given MSD type.

If new orders of MSD type are defined e.g. in later releases, RAN2 may need to add additional signaling for those cases.

Hence, it would be good to inform RAN4 on the observation aspects discussed above and the initial UE capability design.

Send LS to RAN4 to:

- Inform initial UE capability design;

- Ask for more information on additional report of lower MSD capability assumed to be requested by the network;

- Ask RAN4 to inform RAN2 of any new orders of MSD types defined after introduction of this capability.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is up to RAN2 whether to add UE capability filtering, which may not be required for this feature if it is designed outside the band combination list.

Observation 2	If new orders of MSD type are defined e.g. in later releases, RAN2 may need to add additional signaling for those cases.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Lower MSD capability design should not imply in the inclusion of fallback band combinations for the purpose of indicating different MSD capability.

Proposal 2	RAN2 assumes to define the signaling for lower MSD capability outside the band combination list.

Proposal 3	Send LS to RAN4 to:

Inform initial UE capability design;

Ask for more information on additional report of lower MSD capability assumed to be requested by the network;

Ask RAN4 to inform RAN2 of any new orders of MSD types defined after introduction of this capability.