3GPP TSG-RAN WG2 Meeting #123	R2-2308508

# Toulouse, France, 21th – 25th August, 2023

Agenda Item:	7.25.4

Source: 	ZTE Corporation, Sanechips

Title:  	Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology

Document for:	Discussion and Decision

- Introduction

Agreements RAN2#122 1.RAN2 will perform the evaluations of user plane latency, control plane latency, and mobility interruption time. 2.Evaluate the control plane latency from RRC_INACTIVE to RRC_CONNECTED. 3.Evaluate the control plane latency based on the 2-step RACH. 4.For user plane latency evaluation, HARQ disabling should be assumed. Agreements from Friday CB session: 1.Evaluate the mobility interruption in beam mobility 2.Confirm 0ms mobility interruption time is achieved by NR in beam mobility 3.For RTD we consider the same scenario as considered by RAN1

This document aims at provide self-evaluation results on control and user plane latency, against the technical performance requirements for eMBB-s defined in Report ITU-R M.2514, considering above agreements achieved last meeting.

- Discussion

RAN working groups has specified methodology to evaluate User plan latency and control plan latency in subclause of TS 37.910[2], which can be reused with adoptions to adapt NTN specific characteristics, e,.g. disabled HARQ feedback.

Observation 1: Methodology specified for TN (i.e., as in TS 37.910) can be reused to evaluate the user plane latency and control plane latency with NTN specific assumptions (e.g., disabled HARQ feedback).

According to the methodology given in TS 37.910, the user plane latency may be impacted on the transmission pattern  (e.g., TDD/FDD) and the spectrum used (e.g., shared or licensed spectrum). Considering NR-U and TDD is not support in this release of NTN, it is proposed when evaluating the user plane latency, FDD and licensed spectrum is assumed.

Observation 2a: Considering TDD mode and NR-U is not support in this release of NTN, RAN2 doesn’t need to consider both configuration mentioned here when evaluating the user plane latency.

Proposal 1a: FDD and licensed spectrum is assumed in RAN2 when evaluating the user plane latency in NTN.

It has been agreed that RAN2 use the RTD of the RAN1 agreed scenarios for evaluation. RAN1 has agreed to to consider LEO 600km for performance evaluation, therefore it is proposed to confirm the RTD of LEO 600Km is considered in user plane/control plane latency evaluation.

Observation 2b: RAN1 has agreed to to consider LEO 600Km for performance evaluation.

Proposal 1b: RAN2 agrees to consider RTD of LEO 600km in user plane/control plane latency evaluation.

2.1 DL User plane latency

In below table summarized the adapted calculation steps for NTN DL user plane latency based on the table 5.7.1.1.1.-1 of TS 37.910.

Table 5.7.1.1.1-1 DL user plane procedure for NR

| ID | Component | Notations | Value | Adaptive values for NTN |
| --- | --- | --- | --- | --- |
| 1 | DL data transfer | T1 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx+ttof |  | Time of flight between UE and BS is needed to be considered for NTN, where details can be found in new added step 1.5 |
| 1.1 | BS processing delay | tBS,tx The time interval between the data is arrived, and packet is generated. | Tproc,2/2, with d2,1= d2,2= d2,3=0. (Tproc,2 is defined in Section 6.4 of TS38.214) (NOTE1, NOTE2) | According to TS 38.214: Since NR-U is not supported in NTN, for non-shared spectrum Text =0. According to assumption d2,2 and d2,1 equals to 0. For non-overlapped PUSCH resource cases, d2=0. For DL case, = 0 Therefore the equation can be simplified to as below: Where, is defined as 4.1 of 38211, which equals to 64. where Δfmax=480∙103 Hz and , where , and . μ is one of the SCS of UL/DL(e.g., μDL or μUL) that results in larger Tproc,2 is given in table 6.4 of 38.214, which varies based on the SCS given. |
| 1.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot; Twait is the waiting time for next available DL slot if the current slot is not DL slot. | According to 37.910, it is assumed that the packet arrives at any time of any OFDM symbol. In this case, the 0.5symbol length is added as the “average symbol alignment time” at the beginning of the procedure, i.e., TFA = 0.5 symbols. Twait depends on the PDSCH allocation type as well the scheduling granularity (e.g., slot, 4OS non-slot or etc.) In this contribution, it is assumed that the PDCCH monitoring at every symbols, and the transmission cannot cross slot (i.e., the same assumption for TN in TS 37.910). The Twait used for evaluation is the averaged Twait with above assumptions. |
| 1.3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-1/slot selected in evaluation. | - 1/2μ ms for slot level scheduling (μ is determined by SCS); or - 2/4/7 OFDM symbol length, where the symbol length varies with SCS and CP types (i.e., normal or extended CP) For example, for extended CP with 60kHz, there are only 12 symbols for one slot instead of 14. |
| 1.4 | UE processing delay | tUE,rx The time interval between the PDSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. (NOTE3) | According to TS 38.214: For licensed spectrum Text =0, assuming non-overlapped PUSCH cases, d2=0. Therefore above equation can be simplified as below: , where, N1 is determined based on table 5.3-1 and table 5.3-2 of TS 38.214. |
| 1.5 | Time of the flight (new) | ttof One way time-of-flight between the terminal and the BS | 12.885 ms | This value is based on RAN1 agreed scenario for evaluation, which is LEO 600Km, assuming min elevation angle 10 degrees One way delay is 12.885 ms between UE and BS |
| 2 | HARQ retransmission | THARQ = T1 + T2 T2 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx (For Steps 2.1 to 2.4) |  | Based on HARQ disabling assumption, T2 = 0, therefore THARQ = T1 |
| 2.1 | UE processing delay | tUE,tx The time interval between the data is decoded, and ACK/NACK packet is generated. | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. (NOTE4) | Equals to zero since HARQ disabling is assumed |
| 2.2 | UL frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for the next available UL slot | TFA + Twait, TFA is the frame alignment time within the current UL slot; Twait is the waiting time for next available UL slot if the current slot is not UL slot | Equals to zero since HARQ disabling is assumed |
| 2.3 | TTI for ACK/NACK transmission | tUL_duration | 1 OFDM symbol | Equals to zero since HARQ disabling is assumed |
| 2.4 | BS processing delay | tBS,rx The time interval between the ACK is received and the ACK is decoded. | Tproc, 2/2 with d2,1= d2,2= d2,3=0. (NOTE1, NOTE5) | Equals to zero since HARQ disabling is assumed |
| 2.5 | Repeat DL data transfer from 1.1 to 1.4 | T1 |  |  |
| - | Total one way user plane latency for DL | TUP= T1 + n×THARQ where n is the number of re-transmissions (n≥0) |  | According to above analysis, TUP= (1+n)T1 The averaged TUP,ave = (1+p)T1 , where p is the initial transmission error probability which is assumed to have value 0 or 0.1. |
| Note: 1. The value is used for evaluation only; gNB processing delay may vary depending on implementation. 2. For the case of a TDD band (30 kHz SCS) with an SUL band (15 kHz SCS), the value of this step is Tproc,2 =30kHz) / 2 for Initial transmission, and Tproc,2 =15kHz) / 2 for re-transmission. 3. For the above case, the UE is processing PDSCH reception on TDD band with 30kHz SCS, and it is assumed that the value of this step is Tproc,1 =30kHz)/2. 4. For the above case, the value of this step is Tproc,1( =15kHz) - Tproc,1( =30kHz)/2. 5. For the above case, the value of this step is Tproc,2( =15kHz)/2. |  |  |  | Since FDD is assumed in NTN, note 2-5 is doesn’t count to the calculation. |

Since Disabling HARQ feedback is assumed , the time spent from step 2.1 to step 2.4, which is used to transmit and process ACK/NACK can be omitted.

Observation 3: The calculation steps used to evaluate ACK/NACK processing and transmission time (i.e., step 2.1 to 2.4 in table Table 5.7.1.1.1-1 of TS 37.910) can be deleted from NTN DL user plane latency calculation since disabling HARQ feedback is assumed in NTN.

With the assumption proposed in proposal 1 and above observations, a modified procedure for NTN DL user plane latency calculation is given in Annex 1, it is proposed RAN2 to agree the procedures as attached in Annex 1.

Proposal 2: RAN2 agrees to use the procedure given in annex 1 for NTN DL user plane latency calculation.

As analyzed above the total one way DL user plane latency without flight-on path delays can be calculated based on below equation:

TUP= (1+n)T1 = (1+n)(tBS,tx + tFA,DL+ tDL_duration + tUE,rx) = , replace the parameters with equations given from step 1.1 to step 1.4, the final equation is as below:

,

where, according to TS 38.214 and 38.211, k equals to 64, Tc equals to 5.08626E-7 ms, 15*2μ is the SCS used, and N1 is given in table 5.3-1 and table 5.3-2 of TS 38.214 and N2 is given in table 5.3-1 and table 5.3-2 of TS 38.214. which are given in the Annex 2. Assuming typical SCS of {15, 30} kHz, with typical retransmision time with 4, the evaluation results of DL user plane latency for for NTN for resource mapping type A/B is given in below table:

Table 1 DL user plane latency for NR FDD (ms) without UE-BS transmission delay

| DL user plane latency – NR FDD |  |  | UE capability 1 |  | UE capability 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS |  | SCS |  |
|  |  |  | 15 kHz | 30 kHz | 15 kHz | 30 kHz |
| Resource mapping Type A | M=4 (4OS non-slot) | p=0 | 1.42 | 0.75 | 0.89 | 0.62 |
|  |  | p=0.1 | 1.56 | 0.82 | 0.98 | 0.68 |
|  | M=7 (7OS non-slot) | p=0 | 1.64 | 0.85 | 1.10 | 0.73 |
|  |  | p=0.1 | 1.80 | 0.94 | 1.21 | 0.80 |
|  | M=14 (14OS slot) | p=0 | 2.32 | 1.20 | 1.79 | 1.16 |
|  |  | p=0.1 | 2.55 | 1.32 | 1.96 | 1.28 |
| Resource mapping Type B | M=2 (2OS non-slot) | p=0 | 1.03 | 0.55 | 0.49 | 0.30 |
|  |  | p=0.1 | 1.13 | 0.61 | 0.54 | 0.33 |
|  | M=4 (4OS non-slot) | p=0 | 1.17 | 0.62 | 0.64 | 0.37 |
|  |  | p=0.1 | 1.29 | 0.68 | 0.70 | 0.41 |
|  | M=7 (7OS non-slot) | p=0 | 1.46 | 0.77 | 0.93 | 0.55 |
|  |  | p=0.1 | 1.61 | 0.84 | 1.02 | 0.61 |

It can be observed that the maximum user plane latency without consideration on UE-BS propagation delay is 2.55 ms which fulfills the 10 ms requirement.

If UE-BS propagation delay is considered, then the DL user plane latency are given in below table:

Table 2 DL user plane latency for NR FDD (ms) with UE-BS transmission delay assuming LEO 600km

| DL user plane latency – NR FDD |  |  | UE capability 1 |  | UE capability 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS |  | SCS |  |
|  |  |  | 15 kHz | 30 kHz | 15 kHz | 30 kHz |
| Resource mapping Type A | M=4 (4OS non-slot) | p=0 | 14.31 | 13.63 | 13.77 | 13.36 |
|  |  | p=0.1 | 15.74 | 15.00 | 15.15 | 14.70 |
|  | M=7 (7OS non-slot) | p=0 | 14.52 | 13.74 | 13.99 | 13.47 |
|  |  | p=0.1 | 15.97 | 15.11 | 15.39 | 14.82 |
|  | M=14 (14OS slot) | p=0 | 15.21 | 14.08 | 14.67 | 13.81 |
|  |  | p=0.1 | 16.73 | 15.49 | 16.14 | 15.19 |
| Resource mapping Type B | M=2 (2OS non-slot) | p=0 | 13.91 | 13.44 | 13.38 | 13.17 |
|  |  | p=0.1 | 15.31 | 14.78 | 14.72 | 14.48 |
|  | M=4 (4OS non-slot) | p=0 | 14.06 | 13.51 | 13.52 | 13.24 |
|  |  | p=0.1 | 15.46 | 14.86 | 14.87 | 14.56 |
|  | M=7 (7OS non-slot) | p=0 | 14.35 | 13.65 | 13.81 | 13.38 |
|  |  | p=0.1 | 15.78 | 15.02 | 15.19 | 14.72 |

Based on above, the maximum UL user plane latency without time of flight between UE and BS is 2.55 ms and the maximum UL user plane latency with time of flight between UE and BS is 16.73 ms. And according to the performance requirement defined in the report of ITU-R M2514[2], the minimum user plane latency for DL/UL is 10 ms without time of flight and 650 ms considering UE-BS propagation delays.

Observation 4: The performance requirement defined in the report of ITU-R M2514 for user plane latency for DL/UL is 10 ms without time of flight and 650 ms considering UE-BS propagation delays.

Observation 5: Assuming LEO 600km, the maximum DL user plane latency without time of flight between UE and BS is 2.55 ms and the maximum DL user plane latency with time of flight between UE and BS is 16.73 ms,which is smaller than ITU requirement.

Therefore it can be concluded that DL user plane latency of NR FDD (ms) in NTN fulfills the performance requirements for eMBB-s defined in Report ITU-R M.2514.

Proposal 3: RAN2 confirms the DL user plane latency of NR FDD (ms) in NTN fulfills the the performance requirements for eMBB-s defined in Report ITU-R M.2514.

2.2 UL User plane latency

In TS 37.910, grant free transmission is assumed in evaluation of UL user plane latency calculation of TN, the same principle can be reused in NTN when evaluating the UL user plane latency. The steps to evaluate UL user plane latency quite similar to that for DL, the difference is that for UL THARQ needs to be counted in the total latency calculation, also frame alignment is different for UL/DL transmission. The detailed calculation steps is given in Annex 2, and it is proposed that RAN2 adopt the procedure in annex 2

Proposal 4: Gran free transmission is assumed to evaluate the UL user plane latency in NTN and adopt the procedure given in annex 2 for NTN UL user plane latency calculation.

Based on procedure specified in annex 2, the evaluation results with/without UE-BS propagation delay are given as below:

Table 3 UL user plane latency for NR FDD (ms) without UE-BS transmission delay

| DL user plane latency – NR FDD |  |  | UE capability 1 |  | UE capability 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS |  | SCS |  |
|  |  |  | 15 kHz | 30 kHz | 15 kHz | 30 kHz |
| Resource mapping Type A | M=4 (4OS non-slot) | p=0 | 1.61 | 0.84 | 1.07 | 0.80 |
|  |  | p=0.1 | 1.91 | 1.00 | 1.26 | 0.93 |
|  | M=7 (7OS non-slot) | p=0 | 1.82 | 0.95 | 1.29 | 0.91 |
|  |  | p=0.1 | 2.14 | 1.11 | 1.50 | 1.05 |
|  | M=14 (14OS slot) | p=0 | 2.32 | 1.20 | 1.79 | 1.16 |
|  |  | p=0.1 | 2.69 | 1.39 | 2.05 | 1.32 |
| Resource mapping Type B | M=2 (2OS non-slot) | p=0 | 1.03 | 0.55 | 0.50 | 0.30 |
|  |  | p=0.1 | 1.23 | 0.66 | 0.59 | 0.36 |
|  | M=4 (4OS non-slot) | p=0 | 1.36 | 0.71 | 0.82 | 0.55 |
|  |  | p=0.1 | 1.61 | 0.85 | 0.96 | 0.64 |
|  | M=7 (7OS non-slot) | p=0 | 1.57 | 0.82 | 1.04 | 0.66 |
|  |  | p=0.1 | 1.84 | 0.96 | 1.20 | 0.76 |
|  | M=14 (14OS slot) | p=0 | 2.32 | 1.20 | 1.79 | 1.16 |
|  |  | p=0.1 | 2.69 | 1.39 | 2.05 | 1.32 |

Table 4 UL user plane latency for NR FDD (ms) with UE-BS transmission delay assuming LEO 600km

| DL user plane latency – NR FDD |  |  | UE capability 1 |  | UE capability 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS |  | SCS |  |
|  |  |  | 15 kHz | 30 kHz | 15 kHz | 30 kHz |
| Resource mapping Type A | M=4 (4OS non-slot) | p=0 | 14.49 | 13.72 | 13.96 | 13.46 |
|  |  | p=0.1 | 16.08 | 15.17 | 15.44 | 14.85 |
|  | M=7 (7OS non-slot) | p=0 | 14.71 | 13.83 | 14.17 | 13.56 |
|  |  | p=0.1 | 16.32 | 15.29 | 15.67 | 14.97 |
|  | M=14 (14OS slot) | p=0 | 15.21 | 14.08 | 14.67 | 13.81 |
|  |  | p=0.1 | 16.87 | 15.56 | 16.22 | 15.24 |
| Resource mapping Type B | M=2 (2OS non-slot) | p=0 | 13.92 | 13.44 | 13.38 | 13.17 |
|  |  | p=0.1 | 15.41 | 14.83 | 14.77 | 14.51 |
|  | M=4 (4OS non-slot) | p=0 | 14.24 | 13.60 | 13.71 | 13.33 |
|  |  | p=0.1 | 15.78 | 15.02 | 15.14 | 14.70 |
|  | M=7 (7OS non-slot) | p=0 | 14.46 | 13.71 | 13.92 | 13.44 |
|  |  | p=0.1 | 16.02 | 15.14 | 15.37 | 14.82 |
|  | M=14 (14OS slot) | p=0 | 15.21 | 14.08 | 14.67 | 13.81 |
|  |  | p=0.1 | 16.87 | 15.56 | 16.22 | 15.24 |

Based on above tables, the maximum UL user plane latency without time of flight between UE and BS is 2.69 ms and the maximum UL user plane latency with time of flight between UE and BS is 16.87 ms, it can be concluded that UL user plane latency of NR FDD (ms) in NTN fulfills the performance requirements for eMBB-s defined in Report ITU-R M.2514.

Observation 6: Assuming LEO 600km, the maximum UL user plane latency without time of flight between UE and BS is 2.69 ms and the maximum UL user plane latency with time of flight between UE and BS is 16.87 ms which is smaller than ITU requirement.

Proposal 5: RAN2 confirms the UL user plane latency of NR FDD (ms) in NTN fulfills the the performance requirements for eMBB-s defined in Report ITU-R M.2514.

2.3 Control plane latency

As agreed in last RAN2 meeting, 2step RACH is used to evaluate control plane latency in NTN, therefore the procedure to evaluate control plane latency modified based on the TN model is as shown below:

Figure 1 C-plane procedure for NR NTN assuming 2step RACH

UE

gNB

1. Delay for RACH Scheduling Period

3. Processing delay in gNB

5. Processing delay in UE

2. Msg A

(Preamble + RRC message)

()

- Msg B

(including RRC message)

6. RRC Complete message

Control plane procedure

An procedure to evaluate control plane latency for NTN assuming 2step RACH is give in Annex 3, it is proposed that RAN2 adopts the procedure given in Annex 3 for evaluation.

Proposal 6: RAN2 agrees to use the procedure given in annex 3 for NTN control plane latency calculation.

Even when 2step RACH is assumed, MsgA still includes preamble +Msg3 and MsgB still contains the same amount of information as Msg2+Msg4, therefore the required waiting time for resource for MsgA and MsgB transmission as well as the processing time in gNB and UE is still the same as the time used for processing Msg1-Msg4. Therefore if the same assumption used to evaluate control plane latency as specified in TS37.910 is reused in NTN, then the evaluated control plane latency excluding propagation delay between BS-UE is the same regardless the NW type is TN or NTN.

Observation 7: Control plane latency excluding propagation delay between BS-UE is the same regardless the NW type is TN or NTN since the required waiting time for resource for MsgA and MsgB transmission as well as the processing time in gNB and UE is the same for both 2/4step RACH.

For NTN if 2step RACH is assumed, the additional delay is the non-negligible propagation delay used to transmits MsgA/B and RRCResumeComplete message which equals to 1 round trip delay. Therefore the total NTN control plane latency can be the sum of TN control plane latency plus additional1 round trip delay.

Observation 8: Additional 1 RTT is introduced in control plane latency when 2step RACH is assumed in NTN.

Proposal 7: The total NTN control plane latency can be the sum of TN control plane latency plus additional 1 round trip delay.

According to P6, tables on control plane latency assuming LEO-600km is given as below:

Table 5.7.2.1-2 Control plane latency for NR FDD (ms) assuming LEO 600km

| PRACH length |  | 2 OFDM symbols |  |  |  | 6 OFDM symbols |  |  |  | 1ms |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE capability |  | UE capability 1 |  | UE capability 2 |  | UE capability 1 |  | UE capability 2 |  | UE capability 1 |  | UE capability 2 |  |
| SCS |  | 15KHz | 30KHz | 15KHz | 30KHz | 15KHz | 30KHz | 15KHz | 30KHz | 15KHz | 30KHz | 15KHz | 30KHz |
| Type A | M=4 (4OS non-slot) | 41.17 | 38.87 | 40.77 | 38.57 | 41.37 | 39.27 | 40.87 | 38.77 | 42.07 | 39.37 | 42.07 | 39.37 |
|  | M=7 (7OS non-slot) | 41.37 | 39.17 | 40.97 | 38.97 | 41.57 | 39.37 | 41.07 | 38.87 | 42.27 | 40.07 | 42.27 | 40.07 |
|  | M=14 (14OS slot) |  |  |  |  |  |  |  |  | 42.77 | 40.27 | 42.77 | 40.27 |
| Type B | M=2 (2OS non-slot) | 39.07 | 37.77 | 38.77 | 37.67 | 39.47 | 38.07 | 39.17 | 37.77 | 39.87 | 38.67 | 39.57 | 38.47 |
|  | M=4 (4OS non-slot) | 39.57 | 38.07 | 39.17 | 37.87 | 39.97 | 38.27 | 39.67 | 38.07 | 40.47 | 39.07 | 40.07 | 38.67 |
|  | M=7 (7OS non-slot) | 40.47 | 38.57 | 40.07 | 38.37 | 41.07 | 38.77 | 40.57 | 38.57 | 41.57 | 39.57 | 40.77 | 39.07 |

Based on above table, the maximum control plane latency considers RAN1 agreed scenarios is 42.77 ms, which is smaller than the 1.15 s latency proposed in ITU-R report[2]. Therefore it is suggested to confirm that the NTN control plane latency fulfills the performance requirements for eMBB-s defined in Report ITU-R M.2514.

Observation 9: The maximum control plane latency considers RAN1 agreed scenarios is 42.77 ms, which is smaller than the 1.15 s latency proposed in ITR-R report.

Proposal 8: RAN2 confirms that the NTN control plane latency fulfills the performance requirements for eMBB-s defined in Report ITU-R M.2514.

- Summary and Proposals

User plane latency

Observation 1: Methodology specified for TN (i.e., as in TS 37.910) can be reused to evaluate the user plane latency and control plane latency with NTN specific assumptions (e.g., disabled HARQ feedback).

Observation 2a: Considering TDD mode and NR-U is not support in this release of NTN, RAN2 doesn’t need to consider both configuration mentioned here when evaluating the user plane latency.

Observation 2b: RAN1 has agreed to to consider LEO 600Km for performance evaluation.

Observation 3: The calculation steps used to evaluate ACK/NACK processing and transmission time (i.e., step 2.1 to 2.4 in table Table 5.7.1.1.1-1 of TS 37.910) can be deleted from NTN DL user plane latency calculation since disabling HARQ feedback is assumed in NTN.

Observation 4: The performance requirement defined in the report of ITU-R M2514 for user plane latency for DL/UL is 10 ms without time of flight and 650 ms considering UE-BS propagation delays.

Observation 5: Assuming LEO 600km, the maximum DL user plane latency without time of flight between UE and BS is 2.55 ms and the maximum DL user plane latency with time of flight between UE and BS is 16.73 ms,which is smaller than ITU requirement.

Observation 6: Assuming LEO 600km, the maximum UL user plane latency without time of flight between UE and BS is 2.69 ms and the maximum UL user plane latency with time of flight between UE and BS is 16.87 ms which is smaller than ITU requirement.

Proposal 1a: FDD and licensed spectrum is assumed in RAN2 when evaluating the user plane latency in NTN.

Proposal 1b: RAN2 agrees to consider RTD of LEO 600km in user plane/control plane latency evaluation.

Proposal 2: RAN2 agrees to use the procedure given in annex 1 for NTN DL user plane latency calculation.

Proposal 3: RAN2 confirms the DL user plane latency of NR FDD (ms) in NTN fulfills the the performance requirements for eMBB-s defined in Report ITU-R M.2514.

Proposal 4: Gran free transmission is assumed to evaluate the UL user plane latency in NTN and adopt the procedure given in annex 2 for NTN UL user plane latency calculation.

Proposal 5: RAN2 confirms the UL user plane latency of NR FDD (ms) in NTN fulfills the the performance requirements for eMBB-s defined in Report ITU-R M.2514.

Control plane latency

Observation 7: Control plane latency excluding propagation delay between BS-UE is the same regardless the NW type is TN or NTN since the required waiting time for resource for MsgA and MsgB transmission as well as the processing time in gNB and UE is the same for both 2/4step RACH.

Observation 8: Additional 1 RTT is introduced in control plane latency when 2step RACH is assumed in NTN.

Observation 9: The maximum control plane latency considers RAN1 agreed scenarios is 42.77 ms, which is smaller than the 1.15 s latency proposed in ITR-R report.

Proposal 6: RAN2 agrees to use the procedure given in annex 3 for NTN control plane latency calculation.

Proposal 7: The total NTN control plane latency can be the sum of TN control plane latency plus additional 1 round trip delay.

Proposal 8: RAN2 confirms that the NTN control plane latency fulfills the performance requirements for eMBB-s defined in Report ITU-R M.2514.

4. References

- 3GPP TS 37.910

- Report ITU-R  M.2514-0 (09/22)

Annex 1 DL user plane procedure of NTN

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | DL data transfer | T1 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx |  |
| 1.1 | BS processing delay | tBS,tx The time interval between the data is arrived, and packet is generated. | Tproc,2/2, with d2,1= d2,2= d2=0. (Tproc,2 is defined in Section 6.4 of TS38.214) (NOTE1) |
| 1.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot; Twait is the waiting time for next available DL slot if the current slot is not DL slot, which equals to Zero in NTN. (NOTE 2) |
| 1.3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | UE processing delay | tUE,rx The time interval between the PDSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. |
| 1.5 | Time of flight | Ttof The one way propagation time between UE and base station | 12.885ms (NOTE 3) |
| 2 | HARQ retransmission | THARQ = T1 + T2 T2 = (tUE,tx + tFA,UL) + tUL_duration +Ttof + tBS,rx (For Steps 2.1 to 2.5) | T2 = 0 (NOTE 4) |
| 2.1 | UE processing delay | tUE,tx The time interval between the data is decoded, and ACK/NACK packet is generated. | tUE,tx = 0 (NOTE 4) |
| 2.2 | UL frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for the next available UL slot | tFA,UL = 0 (NOTE 4) |
| 2.3 | TTI for ACK/NACK transmission | tUL_duration | tUL_duration = 0 (NOTE 4) |
| 2.4 | Time of flight for ACK/NACK | Ttof The one way propagation time to transmits NACK/ACK from UE to base station | Ttof = 0 (NOTE 4) |
| 2.5 | BS processing delay | tBS,rx The time interval between the ACK is received and the ACK is decoded. | tBS,rx = 0 (NOTE 4) |
| 2.6 | Repeat DL data transfer from 1.1 to 1.4 | T1 |  |
| - | Total one way user plane latency for DL | TUP= T1 + n×THARQ where n is the number of re-transmissions (n≥0) |  |
| - | Averaged one way user plane latency for DL | TUP= T1 + p×THARQ where p is the initial transmission error probability. |  |
| Note: - The value is used for evaluation only; gNB processing delay may vary depending on implementation. - FDD is assumed in NTN - LEO 600 km is assumed with min elevation equals to 10 degrees. The propagation time between UE and BS considers both service link and feeder link delay. - Disabled HARQ feedback is assumed for NTN. |  |  |  |

Annex 2 UL user plane procedure of NTN

able 5.7.1.1.2-1 UL user plane procedure for NR

| Step | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | UL data transfer | T1 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx + |  |
| 1.1 | UE processing delay | tUE,tx The time interval between the data is arrived, and packet is generated; | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214), with d2,1= d2,2= d2,3=0 |
| 1.2 | UL Frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for next available UL slot | TFA + Twait, TFA is the frame alignment time within the current UL slot, Twait is the waiting time for next available UL slot if the current slot is not UL slot. |
| 1.3 | TTI for UL data packet transmission | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | BS processing delay | tBS,rx The time interval between the PUSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”; It is assumed that BS processing delay is equal to UE processing delay as for PDSCH (Note1) |
| 1.5 | Time of flight | Ttof The one way propagation time between UE and base station | 12.885ms (Note 2) |
| 2 | HARQ retransmission | THARQ = T2 + T1 T2 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx (For Steps 2.1 to 2.4) |  |
| 2.1 | BS processing delay | tBS,tx The time interval between the data is decoded, and PDCCH preparation | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. |
| 2.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot; Twait is the waiting time for next available DL slot if the current slot is not DL slot; |
| 2.3 | TTI for PDCCH transmission | tDL_duration | 1 OFDM symbols for PDCCH scheduling the retransmission. |
| 2.4 | UE processing delay | tUE,rx The time interval between the PDCCH is received and decoded. | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214), with d2,1= d2,2= d2,3=0 |
| 2.5 | Repeat UL data transfer from 1.1 to 1.4 | T1 |  |
|  | Total one way user plane latency for UL | TUP= T1 + n×THARQ where n is the number of re-transmissions (n≥0) |  |
|  | Averaged one way user plane latency for DL | TUP= T1 + p×THARQ where p is the initial transmission error probability. |  |
| Note: - The value is used for evaluation only; gNB processing delay may vary depending on implementation. Note: - LEO 600 km is assumed with min elevation equals to 10 degrees. The propagation time between UE and BS considers both service link and feeder link delay. Note: 3.The grant free transmission is assumed to use the following start symbols: For 2-symbol PUSCH, the start symbol can be symbols {0,2,4,6,8,10,12} for PUSCH resource mapping type B For 4-symbol PUSCH, the start symbol can be: For PUSCH resource mapping type B: symbols {0,7} For PUSCH resource mapping type A: symbol 0; For 7-symbol PUSCH, the start symbol can be: For PUSCH resource mapping type B: symbols {0, 7} For PUSCH resource mapping type A: symbol 0; For 14-symbol PUSCH, the start symbol can be at symbol #0 for PUSCH resource mapping type A and B. |  |  |  |

Annex 3 Control plane procedure for NTN assuming 2step RACH

| Step | Description | CP Latency for UL data transfer [ms] |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |
| 2a | Transmission of MsgA, which includes three parts of latency: - Transmission of preamble, - Transmission of RRC message - One way propagation delay between UE and BS | - Transmission of preamble: Length of the preamble according to the PRACH format as specified in [6 38.211] - Transmission of RRC message: Ts (the length of 1 slot / non-slot) NOTE: the length of 1 slot or 1 non-slot is equal to PUSCH allocation length. - One way propagation delay between UE and BS: 12.885 ms NOTE: one way UE-BS delay of LEO 600 km with min 10-degree elevation. |
| 3 | Processing time in gNB, which includes two parts: - Preamble detection - processing time and L2/RRC processing time | - Preamble detection and processing time = Tproc,2 (assuming d2,1=0) - Processing delay (L2 and RRC) = 3 |
| 4 | Transmission of Msg B, which includes three parts of : - Transmission of RA response - Transmission of RRC message - one way propagation delay between UE and BS | - Transmission of RA response: Ts (the length of 1 slot / non-slot) NOTE: the length of 1 slot or 1 non-slot include PDCCH and PDSCH (the first OFDM symbol of PDSCH is frequency multiplexed with PDCCH). - Transmission of RRC message: Ts (the length of 1 slot / non-slot) - One way propagation delay between UE and BS: 12.885 ms NOTE: one way UE-BS delay of LEO 600 km with min 10-degree elevation. |
| 5 | UE Processing Delay,which includes: - Msg B payload decoding (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Resume Request) - RRC message processing delay including grant reception | - Msg B payload decoding : NT,1+NT,2+0.5 ms - RRC message processing delay: 7 |
| 6 | Transmission of RRC complete message and UP data | 0 |
| Notes: - 2step RACH is assumed in the evaluation. 1. For step 1, the procedure for transition from a most “battery efficient” state has yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. 2. For step 3, the value of Tproc,2 is used only for evaluation. gNB processing delay may vary depending on implementation. 4. For step 3, the processing delay in gNB (L2 and RRC) has been reduced to 3 ms. The delays due to inside-gNB or inter-gNB communication are not included in Step 3. Such delays may exist depending on deployment, but are not within the scope of this evaluation. 3. For step 5, the latency of NT,1+NT,2+0.5ms is used according to Section 8.3 of TS 38.213. NT,1 is a time duration of N1 symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 when additional PDSCH DM-RS is configured; and NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for PUSCH processing capability 1. The value of N1 and N2 are shown in Table 5.3-1 and Table 6.4-1 of TS38.214, respectively.. 5. For step 5 for UL data transfer, the processing delay in the UE (L2 and RRC) is considered, e.g., from reception of RRC message to the reception of UL grant. The transmission of UL grant by gNB and processing delay in the UE (processing of UL grant and preparing for UL tx) are also considered. The RRC message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Complete message and the data is transmitted over common search space with DCI format 0. 6. For step 6, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |  |

Annex 4 N1/ N2 value as cited from TS 38.214

| Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, or if none of the higher layer parameters is configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. 5; 80; 96. 6; 160; 192. | Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured. 0; 3. 1; 4.5. 2; 9 for frequency range 1. |
| --- | --- |

| Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1 PUSCH preparation time N2 [symbols]. 0; 10. 1; 12. 2; 23. 3; 36. 5; 144. 6; 288. | Table 6.4-2: PUSCH preparation time for PUSCH timing capability 2 PUSCH preparation time N2 [symbols]. 0; 5. 1; 5.5. 2; 11 for frequency range 1. |
| --- | --- |