3GPP RAN WG2 Meeting #123	R2-2308529

Toulouse, France August 21st – 25th, 2023

Agenda Item:	7.8.3

Source:	InterDigital

Title:	Flightpath reporting for UAV

Document for:	Discussion, Decision

# Introduction

A new work item on NR support for aerial UEs (UAVs) was agreed for Rel-18. Since aerial UEs are supported via LTE as from Rel-15, work done in LTE is a starting point and NR solutions should be harmonized as much as possible. Among detailed objectives for Rel-18 is the introduction of flight path reporting to NR.

RAN2#119e has captured the following relevant agreements [2]:

- As in LTE, flight path plan reporting will be introduced.  Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report.  FFS if timestamp is mandatory or optional for NR.  FFS if further enhancements are needed

RAN2#119bis-e has captured the following [3]:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

RAN2#120 has captured the following [4]:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline.  FFS on granularity

- No requirements are placed on spatial distribution of waypoints

RAN2#121 has captured the following [5]:

- The granularity of flightpath timestamp is 1s.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

And RAN2#121bis-e has further captured the following [7]:

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

This contribution performs an analysis of the current flightpath report size, and discusses the benefits of supporting delta flightpath signalling.

# Delta flightpath reporting for UAV

Recent agreements on the contents of a UAV flight path and reporting procedure largely align with LTE. One exception is support for updating a previously reported flight path, which is a new enhancement unique to NR. Summarizing agreements from past meetings, once an update to a previously reported flight path is available, the UE can notify the network via the UE Assistance Information (UAI) message. An updated flight path is retrieved via the UE information request/response procedure like the initial flight path. This contribution discusses flightpath reporting, whereas the update notification is discussed in a companion contribution [8].

When a flight path is updated/changed, it is likely only a portion of the overall flight path is impacted. For example, the UAV may encounter a delay (e.g., turbulence, collision avoidance etc.), where the waypoints remain the same and only the timestamp information has changed. In another example, the existing flight path may remain valid and one or more additional waypoints may be known to the UE and can be appended to the previously reported flight path. If only a subset of a previously reported flight path has changed (which is likely the case), resending the entire flight path is inefficient from a signalling perspective and can introduce interference.

Observation 1:	Sending an entire flight path report is inefficient if only a subset of the previously reported information has changed.

A UE may instead indicate one or more waypoints/timestamps (e.g., the ones which have been updated, added, or removed) without sending a full flight path, described in recent discussion as “delta signalling”.  The following cases have been captured in the RAN2#122 chair notes [11]:

- Case 1: Some outdated waypoints are removed as the UAV UE has already travelled through them.

- Case 2: Some waypoints’ timestamp is updated but location remains the same.

- Case 3: Some waypoints’ location (and possibly timestamp if it was indicated earlier) is updated due to change in planned path.

- Case 4: Some new waypoint(s) is/are inserted due to change in planned path.

## Flight path report size

To demonstrate the gains of delta flightpath reporting, an analysis of the flightpath report size is first provided. Summarizing past agreements, a flight path report is defined as the following:

- A list of up to 20 waypoints (3D locations) using the existing parameter type LocationCoordinates defined in TS 37.355

- Time information associated to each waypoint via the format AbsoluteTimeInfo-r16 and granularity of 1s.

Referring to TS 37.355, LocationCoordinates can take several different formats/sizes depending on the CHOICE:

LocationCoordinates ::= CHOICE { ellipsoidPoint Ellipsoid-Point, ellipsoidPointWithUncertaintyCircle Ellipsoid-PointWithUncertaintyCircle, ellipsoidPointWithUncertaintyEllipse EllipsoidPointWithUncertaintyEllipse, polygon Polygon, ellipsoidPointWithAltitude EllipsoidPointWithAltitude, ellipsoidPointWithAltitudeAndUncertaintyEllipsoid EllipsoidPointWithAltitudeAndUncertaintyEllipsoid, ellipsoidArc EllipsoidArc, ..., highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510 HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15, highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510 HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15, ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680 HA-EllipsoidPointWithScalableUncertaintyEllipse-r16, ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680 HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16 }

At minimum, describing a 3D location requires altitude in addition to latitude and longitude. In this case, the minimum sized IE which can describe a waypoint is EllipsoidPointWithAltitude, shown below [9]:

#### – EllipsoidPointWithAltitude The IE EllipsoidPointWithAltitude is used to describe a geographic shape as defined in TS 23.032 [15]. -- ASN1START EllipsoidPointWithAltitude ::= SEQUENCE { latitudeSign ENUMERATED {north, south}, degreesLatitude INTEGER (0..8388607), -- 23 bit field degreesLongitude INTEGER (-8388608..8388607), -- 24 bit field altitudeDirection ENUMERATED {height, depth}, altitude INTEGER (0..32767) -- 15 bit field } -- ASN1STOP

Introducing uncertainty to each waypoint via selecting the EllipsoidPointWithAltitudeAndUncertaintyEllipsoid requires additional bits to support parameters like uncertaintySemiMajor or uncertaintySemiMinor (7 bits each). Selecting a high-accuracy format like HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid takes even more.

Observation 2:	Each waypoint requires at least 8 bytes (if described via EllipsoidPointWithAltitude) but can be even higher if uncertainty information is included or a high accuracy format is selected.

If configured and available at the UE, a flightpath may also contain timestamp information associated with each waypoint. Based on agreement from RAN2#121, this information is encoded using AbsoluteTimeInfo-r16 IE:

#### – AbsoluteTimeInfo The IE AbsoluteTimeInfo indicates an absolute time in a format YY-MM-DD HH:MM:SS and using BCD encoding. The first/ leftmost bit of the bit string contains the most significant bit of the most significant digit of the year and so on. AbsoluteTimeInfo information element -- ASN1START -- TAG-ABSOLUTETIMEINFO-START AbsoluteTimeInfo-r16 ::= BIT STRING (SIZE (48)) -- TAG-ABSOLUTETIMEINFO-STOP -- ASN1STOP

As can be seen, each timestamp will require an additional 6 bytes based on the size of AbsoluteTimeInfo IE.

Observation 3:	Each timestamp will require 6 bytes based on the size of AbsoluteTimeInfo-r16.

Therefore, a flightpath report with the maximum allowable number of waypoints requires at least 160 bytes assuming EllipsoidPointWithAltitude is used to represent each waypoint. Adding the associated timestamp information increases the size by an additional 120 bytes, for a total of 280 bytes.

Observation 4:	A flightpath report with 20 waypoints using the minimum format to describe a 3D location (EllipsoidPointWithAltitude) requires 160 bytes. Adding timestamp information increases the total size to 280 bytes.

## Delta Flightpath reporting

In LTE, a flightpath consists of sequence of waypoints and timestamps (if configured) up to the maximum size maxWayPoint (agreed as 20 in both LTE and NR). Assuming the NR format is similar, the flightpath report will look like the following [10]:

FlightPathInfoReport-r15 ::= SEQUENCE { flightPath-r15 SEQUENCE (SIZE (1..maxWayPoint-r15)) OF WayPointLocation-r15 OPTIONAL, dummy SEQUENCE {} OPTIONAL } WayPointLocation-r15 ::= SEQUENCE { wayPointLocation-r15 LocationInfo-r10, timeStamp-r15 AbsoluteTimeInfo-r10 OPTIONAL }

As can be immediately seen, although timeStamp is optional, WayPointLocation-r15 requires a waypoint, meaning in LTE timestamp information cannot be sent alone.

Observation 5:	LTE flightpath reporting only allows UE to report timestamp information with a waypoint.

Introducing a timestamp-only report (i.e., addressing Case 2) consisting of a sequence of timestamps with length 1..maxWayPoint can allow the network to extract the timestamp information for each individual waypoint. The gNB could then update the timestamp information for each waypoint within a previously reported flightpath 1-1 without changing the current flightpath reporting structure.

Observation 6:	A timestamp-only report consisting of a sequence of timestamps with length 1..maxWayPoint allows the network to extract and update the timestamp information for each waypoint 1-1 without changing the current flightpath reporting structure.

Assuming the waypoint format uses EllipsoidPointWithAltitude, (8 bytes), reporting only the timestamp information reduces the overall report size by 63%. The savings in signalling overhead could be even higher if a more accurate format is used to describe the waypoints.

Observation 7:	Allowing UAV to update timestamp information only (i.e., a sequence of timestamps with size 1..maxWayPoint but without associated waypoints) reduces the reporting size by at least 63% if only time information has changed.

Considering the savings in signalling overhead and minimal specification impact, it is proposed that as baseline the UE is allowed to report only updated timing information, which addresses Case 2.

Proposal 1:	UAV can report timestamp information only (i.e., a sequence of timestamps with size 1..maxWayPoint) by making waypoint reporting optional.

Whether the UE reports full or timestamp-only flightpath should ultimately be decided by the network (e.g., based on the assistance information provided in the update notification, refer to companion contribution [8]). The network may explicitly indicate whether to report a full or partial flight path (e.g., within the information request message). Alternatively, the ability to report timestamp-only flight path update may be enabled via configuration, where UE will by default only report the modified/new flight path information.

Proposal 2:	NW decides whether UE reports full or timestamp-only flight path. FFS if timestamp-only flight path reporting is enabled by configuration or explicit request.

One downside of the above solution is that it isn’t useful if a waypoint also needs to be updated (e.g., Cases 1, 3 and 4). Extending delta reporting further could allow individual waypoint/timestamp information to be updated. Although this would provide the highest benefit in terms of signalling overhead, it may require the most specification impact. For example, instead of the current sequence used in LTE, the UE may adopt an indexed structure, where each waypoint can be added/modified/removed similar to measurement configuration. Considering the advanced stage of Rel-18 and specification impact, support for updating individual waypoints can be postponed to Rel-19.

Proposal 3:	Other types of delta flightpath reporting (e.g. updating/adding individual waypoints) is postponed to Rel-19.

# Conclusion

In this contribution the following proposals are made concerning delta flightpath reporting for aerial UEs in NR:

Observation 1:	Sending an entire flight path report is inefficient if only a subset of the previously reported information has changed.

Observation 2:	Each waypoint requires at least 8 bytes (if described via EllipsoidPointWithAltitude) but can be even higher if uncertainty information is included or a high accuracy format is selected.

Observation 3:	Each timestamp will require 6 bytes, based on the size of AbsoluteTimeInfo-r16.

Observation 4:	A flightpath report with 20 waypoints using the minimum format to describe a 3D location (EllipsoidPointWithAltitude) requires 160 bytes. Adding timestamp information increases the total size to 280 bytes.

Observation 5:	LTE flightpath reporting only allows UE to report timestamp information with a waypoint.

Observation 6:	A timestamp-only report consisting of a sequence of timestamps with length 1..maxWayPoint allows the network to extract and update the timestamp information for each waypoint 1-1 without changing the current flightpath reporting structure

Observation 7:	Allowing UAV to update timestamp information only (i.e., a sequence of timestamps with size 1..maxWayPoint but without associated waypoints) reduces the reporting size by at least 63% if only time information has changed.

Proposal 1:	UAV can report timestamp information only (i.e., a sequence of timestamps with size 1..maxWayPoint) by making waypoint reporting optional.

Proposal 2:	NW decides whether UE reports full or timestamp-only flight path. FFS if timestamp-only flight path reporting is enabled by configuration or explicit request.

Proposal 3:	Other types of delta flightpath reporting (e.g. updating/adding individual waypoints) is postponed to Rel-19.

# References

RP-223545 – WI on NR support for UAV

R2-2209301 – RAN2#119e Meeting Report

R2-2211101 – RAN2#119bis-e Meeting Report

R2-2300002 – RAN2#120 Meeting Report

R2-2302401 – RAN2#121 Meeting Report

R2-2302867 – Report from [POST121][314][UAV] Flight path reporting (Intel)

R2-2306553 – RAN2#121bis-e Meeting Report

R2-2308528 – Flightpath update notification for UAV - InterDigital

3GPP TS 37.355: LTE Positioning Protocol (LPP) v17.4.0

3GPP TS 36.331: Radio Resource Control (RRC) v17.4.0

Draft_R2_122_meeting_report_v2