3GPP TSG-RAN WG2 Meeting #123	 					     R2-2308532

Toulouse, France, August 21-25, 2023

Source:	Continental Automotive

Title:                       Discussion on LP-WUS in RRC_CONNECTED

Agenda Item:         7.22.3

Document for:	Discussion and Decision

# Introduction

For LP-WUS, the following agreements were achieved in RAN1 #110b-e meeting [1]:

Agreement:

Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI.

In this contribution discusses the possible further enhancements for LP-WUS in RRC CONNECTED.

# Discussion

The motivation of LP-WUS is to enable the UE to place its MR in an ultra-deep sleep state when there is no data or signal exchange required. Instead, a separate LP-WUR is used to monitor a LP-WUS, which serves as an indication to wake up the MR. This approach allows for significant power-saving gains by efficiently managing power consumption during periods of inactivity.

In the connected mode, UE power saving is achieved based on C-DRX. When the UE receives DCP and it indicates the start of the timer, the UE should wake up and start the onDurationTimer. When the UE is configured to monitor LP-WUS, the UE starts the onDurationTimer if its LP-WUR detects the LP-WUS. Therefore, we think the functionalities of LP-WUS and DCP are identical hence, LP-WUS may act as a replacement for DCP. Consequently, the UE is not required to monitor the DCP when it is configured to monitor the LP-WUS.

Observation 1: When LP-WUR of the UE is configured to monitor LP-WUS, the UE upon receiving LP-WUS may start the onDurationTimer, hence it may act as a replacement for DCP.

Proposal 1: When the UE is configured to monitor the LP-WUS, it is not required to monitor the DCP.

It was agreed in 3GPP RAN1#112bis meeting [2] that the LP-WUR monitoring the LP-WUS may not always be able to meet the LP-WUS RSRP/RSRQ threshold configured by the network for LP-WUS. Therefore, it is possible that the UE may miss the LP-WUS if the UE is at the edge of the LP-WUS coverage as shown in figure.

Figure: UE located at LP-WUS coverage edge

As a result, the UE will not trigger the MR and will not start the onDurationTimer. Hence, if the gNB has scheduled any DL data during the DRX cycle then the UE may miss the reception of the DL data, which would be detrimental if the DL data was latency sensitive. To solve this issue, we suggest that the network should configure repetition of LP-WUS when it determines that the UE is at the LP-WUS coverage edge. In the connected mode, the gNB is always aware of the UE’s location in the cell based on the UE’s measurement report. The repetition of the LP-WUS would increase its reliability and improve its coverage.

Observation 2: There is a possibility to miss the LP-WUS if the UE is located at the LP-WUS coverage edge.

Proposal 2: gNB configures the repetition of LP-WUS based on the measurement report from the UE if needed.

# Conclusions

In this contribution, we discussed the possible enhancements for LP-WUS in RRC_CONNECTED. Additionally, we ask RAN2 to discuss the following observations and proposals:

Observation 1: When LP-WUR of the UE is configured to monitor LP-WUS, the UE upon receiving LP-WUS may start the onDurationTimer, hence it may act as a replacement for DCP.

Proposal 1: When the UE is configured to monitor the LP-WUS, it is not required to monitor the DCP.

Observation 2: There is a possibility to miss the LP-WUS if the UE is located at the LP-WUS coverage edge.

Proposal 2: gNB configures the repetition of LP-WUS based on the measurement report from the UE if needed.

# References

[1] RAN1#110b-e meeting agreements

[2] RAN1#112bis meeting agreements.