3GPP TSG RAN WG2 Meeting #123													                 R2-2308533

Toulouse, France, August 21st – August 25th, 2023

Agenda item:		7.3.4 Cell selection/re-selection

Title:						Cell Selection and Re-Selection for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

During the ongoing discussion on cell selection / re-selection the following agreements were concluded during the preceding RAN2 meeting #122 [1]:

We will define UE capabilities with signaling, Details are FFS and will be discussed later during the WI phase. Separate camping restrictions for NES-capable and non-NES UEs will be defined. FFS if it is a single bit or more.

The agreement is to specify mechanisms for avoiding legacy UEs accessing cells applying Rel-18 NES techniques. This contribution discusses the conventional barring method and identify potential challenges and limitations that could arise with the introduction of Rel-18 NES techniques.

# Cell Selection and Re-selection

During the previous meetings, the importance of barring legacy UEs from cells applying NES techniques was discussed. Cell barring may prevent legacy UEs to access or camp on a cell, once NES techniques are applied, e.g. during low traffic load scenarios.

However, as some NES features were selected for Rel-18 and further features are likely to be introduced on future releases there will be hardly a “NES cell”, but instead there will be cells supporting a certain set of NES features. The discussion of cell barring needs to be revisited according to this feature-set view. As diverse NES techniques may be implemented, there might be multiple groups of UEs supporting diverse types of NES techniques and within a technique there may even be multiple UE capabilities. As such, simply separating UEs into legacy UEs and NES-capable UEs may not provide the best match for operator needs. Instead, a finer granularity may be considered. Namely, only those UEs should be barred from camping on a cell that are not able to handle the applied NES techniques.

Even within the Rel-18, there can be variability in the extent to which UEs support NES techniques. While certain UEs are equipped to handle a group of techniques, including cell DTX/DRX and spatial adaptation methods, others might have limited compatibility, accommodating only specific methods. As an example, should a UE supporting spatial adaptation but not Cell DTX/DRX be precluded from a cell applying Cell DTX/DRX? Quite certainly yes.   But is such UE a “NES-capable UE”? If yes, it cannot be barred. If no, it will be barred even on cells applying just spatial adaptation. Clearly, applying selective barring to UEs becomes critical to ensure optimal network functioning. As illustrated in Figure 1 below, scenarios with multiple UEs and gNBs supporting one or more NES techniques will become more and more common. When a gNB applies a NES technique, it may be crucial to bar all UEs that do not support it. Therefore, the importance of filtering out UEs that are not compatible with the applied techniques will only grow with time which may not be easily done using a single bit to distinguish for “NES-capable UEs” from “non-NES UEs”.

Figure 1 – Only UEs which do not support the specific applied technique need to be barred

During the study phase the need for cell barring was discussed based on barring legacy UEs from NES cells while letting NES-capable UEs to access the NES cell. This differentiation may not be granular enough. Cells may apply a set of NES techniques, while UEs may be able to support a different set of NES techniques.

As more NES techniques are expected to be added in future releases, it will become increasingly important to selectively block UEs that do not support the applied techniques in a cell, rather than barring all legacy UEs.

NES market may get even more fragmented with future releases. NES features will be introduced over time, in different releases.  For the foreseeable future networks and UEs with different NES capabilities will have to interact.

- However, with the constant introduction of new features and NES techniques, the current barring mechanism may require frequent redesigns. Therefore, a more flexible and a future-proof approach for barring UEs should be adopted to minimize redesign efforts and ensure efficient management of network resources.

RAN2 may consider a finer granularity for UEs to apply cell barring than to distinguish legacy UEs and NES supporting UEs only. Thus, there might be a better match of UEs being able to camp on cells applying NES techniques. Therefore, to be able to address each NES technique, having more than one bit is beneficiary.

Due to the introduction of NES features cells may need to be barred more frequently than they have been barred on the past. During initial access (initial cell search) a UE will need then to traverse more frequencies until it can find one which is not barred. Consequently, in order to prevent significant initial access delays for UEs, the process of receiving and decoding the barring information should be designed to be as quick as possible. Therefore, ideally NES-related barring should be directly on the MIB, which could provide quicker access to the information. However, only one bit is left in the MIB.

Ideally NES-related barring should be on MIB, to allow fast adaptations and not degrade initial access. However only 1 spare bit is left on MIB.

RAN 2 to discuss how to enable fast evaluation of NES-related barring.

# Conclusions

In this contribution, we discussed some Cell Selection and Cell Reselection aspects of Network Energy Savings. The following observations and proposals have been made:

Observation 1: During the study phase the need for cell barring was discussed based on barring legacy UEs from NES cells while letting NES-capable UEs to access the NES cell. This differentiation may not be granular enough. Cells may apply a set of NES techniques, while UEs may be able to support a different set of NES techniques.

Observation 2: NES market may get even more fragmented with future releases. NES features will be introduced over time, in different releases.  For the foreseeable future networks and UEs with different NES capabilities will have to interact.

- Observation 3: Ideally NES-related barring should be on MIB, to allow fast adaptations and not degrade initial access. However only 1 spare bit is left on MIB.

- Proposal 1: RAN2 may consider a finer granularity for UEs to apply cell barring than to distinguish legacy UEs and NES supporting UEs only. Thus, there might be a better match of UEs being able to camp on cells applying NES techniques. Therefore, to be able to address each NES technique, having more than one bit is beneficiary.

- Proposal 2: RAN 2 to discuss how to enable fast evaluation of NES-related barring.

# References

- RP-230914 “Status Report to TSG”, 3GPP TSG RAN meeting #100, Taipei, June, 2023