3GPP TSG RAN WG2 #123	                                R2-2308572

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.4.2.1

Source:	InterDigital, Inc.

Title:	RRC_INACTIVE and LTM.

Document for:	Discussion

# Introduction

The WI on further NR mobility enhancements [1] includes the following objectives for L1/2 based inter-cell mobility:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized - To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including: - Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and - Definition of corresponding RRM requirements [RAN4]; and - If necessary based on RAN4 outcome, definition of corresponding signalling support [RAN2]. Note 5: RAN4 will coordinate in due course with RAN2 to start the work. Note 6: R4-2220415 serves as baseline for future work in RAN4 Note 7: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.

In this contribution we discuss what happens with LTM configuration when UE goes to RRC_INACTIVE.

# Discussion

Currently, when the UE receives RRCRelease to move to RRC_INACTIVE, stored candidate cell configurations are released (e.g., conditional reconfiguration targets). However, since LTM is designed to achieve multiple subsequent cell switches without the need for RRC Reconfiguration, it is possible (likely) that LTM candidate configurations are still valid after resuming the connection, even if the UE has performed some cell reselection in the meantime (within a limited area). Since LTM is also designed to minimise RRC reconfiguration needed after cell switch, it would be beneficial if LTM candidate configurations can be stored when the connection is suspended, so that they are available without the need for RRC Reconfiguration again after resume to (re) configure LTM candidates – at least if the UE stays in the same cell, or within a relatively small area (E.g. within the coverage of configured LTM candidates).

Proposal 1: LTM candidate configuration is stored when entering RRC_INACTIVE so that LTM candidates can be available during and/or after RRC Resume.

If LTM candidates are stored when entering RRC_INACTIVE, then LTM can immediately be used after resuming the RRC Connection (E.g. after Msg5). It is possible to further improve the RRC Resume procedure itself if the stored candidate configuration for the current cell (i.e., the PCell) could be enabled by indicating the configuration ID in RRC Resume, or in other words, to trigger LTM towards the current cell in Msg4, by referencing the stored configuration of the current cell. This way, the already stored configuration is applied on RRCResume instead needing an explicit RRC configuration, as is needed in the current system.

Proposal 2: During RRC resume, the stored LTM configuration for the current PCell can be applied. FFS whether LTM candidate ID is indicated in RRCResume or MAC CE.

If early measurements are reported, then LTM could also be used to quickly setup SCG/SCells based on stored candidate configurations. In addition, it is still to be discussed exactly how the R18 early measurements for FR2 are reported and what to do with the reported measurements (e.g., how to report the measurement, what the content of the report is, and what to send from the NW in response). If these measurements can be reported during RRC resume then it might be possible to not only use LTM for setting up (resuming) the PCell, but it could also be used for fast SCG/SCell setup based on measurements of potential SCells.

Proposal 3: During RRC resume, LTM can be used setup SCG/SCells (e.g., if early measurements are reported, according to RAN4 progress). FFS whether indicated in RRCResume or MAC CE.

# Conclusion

In this paper we provide the following proposals:

Proposal 1: LTM candidates are stored when entering RRC_INACTIVE so that LTM candidates can be available during and/or after RRC Resume.

Proposal 2: During RRC resume, the stored LTM configuration for the current PCell can be applied. FFS whether LTM candidate ID is indicated in RRCResume or MAC CE.

Proposal 3: During RRC resume, LTM can be used setup SCG/SCells (e.g., if early measurements are reported, according to RAN4 progress). FFS whether indicated in RRCResume or MAC CE.

# References

- RP-223520,  “Revised WID on Further NR mobility enhancements”

- 38.133, “NR; Requirements for support of radio resource management”