3GPP TSG RAN WG2 #123	                                R2-2308573

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.4.2.1

Source:	InterDigital, Inc.

Title:	LTM execution upon RLF and HOF.

Document for:	Discussion

# Introduction

The WI on further NR mobility enhancements [1] includes the following objectives for L1/2 based inter-cell mobility:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In RAN2#121bis the following was agreed:

- At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

In this contribution we provide an overview of the procedure and high level open issues.

# Discussion

Figure 1: LTM baseline procedure ([3])

Figure 1 above shows the baseline LTM procedure (as captured in the running stage 2 CR [3]) broken down into 4 general phases:

- LTM preparation

- Early sync

- LTM execution

- LTM completion

For RLF execution, in addition to the MAC CE triggering LTM, it has been agreed that at RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

Rather than assuming that LTM can be triggered only after an RLF or HOF, RAN2 could consider how to measure and select a suitable candidate more quickly – instead of waiting for RLF detection if there is a suitable LTM candidate available when out-of-sync is detected, then LTM could be triggered before T310 expires allowing for a faster recovery and minimise the interruption, which would be significant compared to waiting for T310 to expire and performing a cell selection before triggering LTM (e.g. as per CHO).

Proposal 1: Instead of waiting for RLF (T310 expiry), if there is a suitable LTM candidate available before T310 expires then LTM is triggered.

A suitable LTM candidate can be considered as an LTM candidate which meets an RSRP threshold. However, existing thresholds may not be suitable (E.g. Qrxlevmin may not be considered good enough to execute LTM). Hence a new threshold should be introduced to allow UE to trigger LTM early before T310 expiry.

Proposal 2: Introduce an RSRP threshold for allowing UE to trigger LTM to a candidate cell which has a measured RSRP above threshold, if T310 is running.

For the HOF case (LTM execution failure) it is less clear that any suitable “early” trigger to another candidate is possible, since we expect the LTM supervision timer must expire first, to allow sufficient time to attempt LTM to the indicated candidate.

Proposal 3: As a baseline, LTM can be triggered to a suitable candidate upon HOF (new LTM supervisor timer expiry). RAN2 to discuss whether a faster recovery can be supported for HOF (e.g., before expiry of the new LTM supervisor timer, if there is a suitable alternative candidate cell).

# Conclusion

In this paper we provide the following proposals.

Proposal 1: Instead of waiting for RLF (T310 expiry), if there is a suitable LTM candidate available before T310 expires then LTM is triggered.

Proposal 2: Introduce an RSRP threshold for allowing UE to trigger LTM to a candidate cell which has a measured RSRP above threshold, if T310 is running.

Proposal 3: As a baseline, LTM can be triggered to a suitable candidate upon HOF (new LTM supervisor timer expiry). RAN2 to discuss whether a faster recovery can be supported for HOF e.g. before expiry of the new LTM supervisor timer expiry, if there is a suitable alternative candidate cell.

# References

- RP-221799,  “Revised WID on Further NR mobility enhancements”

- 38.133, “NR; Requirements for support of radio resource management”

- R2-2302039, “38.300 running CR for introduction of NR further mobility enhancements”, Mediatek Inc.

- R2-2304214, “Summary of [AT121bis-e][018][eMob] Procedure Consolidation”, Huawei