3GPP TSG RAN WG2 #123	                                R2-2308574

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.4.2.2

Source:	InterDigital, Inc.

Title:	RRC Measurements when LTM is Configured.

Document for:	Discussion

# Introduction

The WI on further NR mobility enhancements [1] includes the following objectives for L1/2 based inter-cell mobility:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

Currently RAN1 are discussing L1 measurement enhancements for support of LTM execution. However, some consideration is needed from a RAN2 perspective in terms of candidate set maintenance and in terms of how LTM and L3 mobility interact.

# Discussion

For LTM candidate set initial configuration and maintenance/reconfiguration we assume that this would be supported by using RRC measurements and reporting since the RRC configuration needs to come from the CU. However, it’s not clear that existing measurement events are suitable. For example, it is not clear how to compare an LTM candidate (non-serving) with a non-candidate neighbour using the existing measurement events, in order to replace one of the candidates. Some consideration is needed to determine how to perform the LTM candidate set configuration and maintenance.

Proposal 1: A new RRC event is needed to compare configured LTM candidate cells with neighbour cells to support LTM candidate set maintenance (addition/removal/replacement of LTM candidates).

Since R18 LTM does not support inter-CU handover, we need to rely on L3 mobility for at least this case. In fact, a legacy handover (or CHO) procedure including the RRC measurements and reporting needs to be supported in case of handover to any cell outside of the configured LTM candidate set. Hence, even when LTM is in use, the UE will still have to perform L3 measurements and RRC measurement reporting in parallel.

One issue with configuring LTM and L3 mobility in parallel is a potential race condition between the 2 approaches. LTM cell changes will occur more frequently due to lower latency and shorter measurement evaluation times, so there will be some situations in which the L3 (e.g., inter-CU) handover is triggered too early or too late. For example, if an LTM cell change occurs during time-to-trigger for a L3 measurement event, then the event would not be triggered, and the event evaluation would need to re-start based on L3 measurements configured for the new serving cell, which could result in too late inter-CU handover and ultimately RLF. Some consideration on how to perform a L3 measurement event evaluation while LTM is in use is necessary.

A straightforward approach would be to use s-measure to control whether LTM is enabled or not. Currently s-measure is only used to allow UE not to measure on neighbour carriers/cells when the serving cell quality is suitably high. If we use s-measure to control LTM then as long as the serving cell’s signal level is suitably high, then only LTM needs to be performed (L3 measurements need not be performed or reported). If the serving cell is below s-measure, then L3 measurements are enabled (as today) and this also implies that no LTM candidate cell has a good enough quality (assuming UE is always on the best LTM candidate cell) and a L3 handover needs to be performed (E.g. to a cell on another CU), hence in this case LTM needs to be disabled to avoid interfering with RRC measurement evaluation.

One exception would of course be any new event intended to support LTM set maintenance.

Proposal 2: s-measure controls whether LTM or RRC measurements are performed.

- If the serving cell quality is above s-measure, then LTM measurements are performed and RRC measurements are disabled.

- If the serving cell quality is below s-measure, then LTM is disabled and RRC measurements are performed.

# Conclusion

In this paper we provide the following proposals regarding potential solution directions to consider in the design of L1/2 triggered handover in Release-18.

Proposal 1: A new RRC event is needed to compare configured LTM candidates with neighbour cells.

Proposal 2: s-measure controls whether LTM or RRC measurements are performed.

- If the serving cell quality is above s-measure then LTM measurements are performed and RRC measurements are disabled.

- If the serving cell quality is below s-measure then LTM is disabled and RRC measurements are performed.

# References

- RP-221799,  “Revised WID on Further NR mobility enhancements”

- 38.133, “NR; Requirements for support of radio resource management”