3GPP TSG RAN WG2 #123	                                R2-2308575

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.4.2.2

Source:	InterDigital, Inc.

Title:	RLC and PDCP reset.

Document for:	Discussion

# Introduction

The WI on further NR mobility enhancements [1] includes the following objectives for L1/2 based inter-cell mobility:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In the previous meetings, the following agreements were made.

- Can use legacy behaviour: - P2: RAN2 assumes that network implementation allows speedy data recovery for RLC AM bearer at intra-DU LTM cell switch without specification impact. - P3: The PDCP data recovery procedure can be applied to the RLC AM bearers for inter-DU LTM cell switch. - Will not support HARQ continue at LTM cell switch in this release. - For non-TA parts, we do MAC reset, which overrides earlier agreements on partial MAC reset. As earlier agreed RLC-AM can continue at LTM cell switch (intended for intra-DU).

It’s still FFS on whether the LTM-CandidateNoResetL2-List field should include separate reset flags for MAC, RLC, and PDCP recovery.

# Discussion

Currently the RRC running CR contains the following:ltm-NoResetID This field indicate whether the UE should perform no L2 reset for an LTM candidate cell upon an LTM cell switch procedure. If the value of ltm-NoResetID in the LTM candidate cell is the same as the value of ltm-ServingCellNoResetID in the serving cell of a cell group, then the UE shall not perform any L2 reset during an LTM cell switch procedure.

Since it was agreed in the previous meeting that MAC reset is not performed, the “ltm-NoResetID” applies to RLC and PDCP. The following shows an example CU-DU split architecture.

Figure 1: CU-DU split with RLC and PDCP.

As can be seen above, the main case for RLC reset in Rel-18 is inter-DU LTM. For an intra-DU cell switch, no RLC reset is required because RLC resides in the DU so it can be continued.

For inter-DU LTM, it’s agreed that PDCP data recovery procedure can be applied for AM bearers. For PDCP re-establishment, the main case would be inter-CU cell switch which is not supported in R18 (but likely in R19). However, it may be necessary to support PDCP re-establishment in some cases per NW configuration.

For R18, it should at least be possible to reset RLC but not PDCP (e.g. in the inter-DU, intra-CU, cell switch).  Hence, the proposed field “ltm-NoResetID” should be split into separate IEs for RLC and PDCP. Alternatively, it can be clarified that the field “ltm-NoResetID” refers to reset of RLC, and PDCP re-establishment can be revisited in Rel-19.

Proposal 1: The field “ltm-NoResetID” should be split into separate IDs for RLC and PDCP.

# Conclusion

In this paper we provide the following proposal:

Proposal 1: The field “ltm-NoResetID” should be split into separate IDs for RLC and PDCP.

# References

- RP-221799,  “Revised WID on Further NR mobility enhancements”

- 38.133, “NR; Requirements for support of radio resource management”