3GPP TSG-RAN WG2 Meeting #123	R2-2308608

Toulouse, France, 21st-25th August 2023

Agenda item:	7.3.5

Source: 	Fujitsu

Title: 	Discussion on Connected mode mobility for network energy savings

Document for:	Discussion and decision

Introduction

In RAN2#122 meeting, RAN2 agreed the followings on the connected mode mobility [1].

Agreements:

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

In this contribution, we would like to discuss the CHO enhancements for network energy savings.

Discussion

General requirement

Some efficient NES techniques were not included in the WI scope of Rel-18 due to the TU limitation. But as proposed in Rel-19 workshop, we think enhanced techniques would be supported in the future release to archive more NES efficiency. Based on this aspect, it is important to consider a forward compatible solution for CHO enhancements in Rel-18. In other words, NES technique dependent solution should be avoided (e.g., define different triggering by different NES techniques).

Observation 1: NES techniques may be additionally supported in a future release.

Proposal 1:	RAN2 should consider a forward compatibility for NES CHO enhancements solution.

For the source cell case

The motivation of consideration of NES mode in a source cell is to handover the UE faster for efficient power saving. If the traffic/load in the source cell becomes lower, the NES technique(s) can be applied to reduce the network power consumptions. However, the UE may not know when the cell decides to start an energy saving operation, and if the UEs require enough radio resources, the network wants to move the UEs to the neighboring cell before NES mode is changed.

In the previous meeting, RAN2 agreed 1) Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario, and 2) we have a specific NES CHO execution condition based on source cell NES mode. It implies it relays on the serving/neighbour cell quality whether an entering condition of NES CHO execution is fulfilled or not. Different from the legacy CHO, the quality of the serving/neighbour cell may not fulfill the normal execution condition before the network turns on the NES mode. In order to facilitate the conditional event execution, it is reasonable to introduce new NES specific offset or parameter sets corresponding to NES mode.

Observation 2: Conditional Event A3, A4 or A5 for normal mode cell may not be fulfilled before the network turns on the NES mode.

Proposal 2:	NES mode specific offset or parameter sets is introduced in Conditional Event A3, A4 or A5 to facilitate NES specific CHO execution condition.

Given relaxed value compared with the normal event is used, if NES mode specific offset or parameter sets is immediately used for an evaluation of conditional event A3, A4 or A5 after RRC configuration, a handover may be not intentionally executed (i.e., too early HO). It may reduce the CHO efficiency and benefits. Further, as RAN1 agreed to introduce an L1 signalling to activate/deactivate Cell DTX/DRX in the last meeting [3], it is difficult for the UE itself to determine the NES mode beforehand because it can be dynamically switched based on the cell traffic/load. Only the network can determine the NES mode switch timing, it could be simple that the gNB indicates to apply the NES specific condition to the UE. However, if RRC signalling is used to indicate the NES mode switch timing, it will cause significant signalling overheads. Meanwhile, an indication by L1 signalling will require further additional RAN1 work. One option is reusing L1 signalling for Cell DTX/DRX activation/deactivation as the indication, but it is clearly NES technique dependent solution and not a forward compatible. Another possible option is that the gNB indicates start and end time to apply the NES specific CHO condition. Such timing information can be additionally configured in the NES specific CHO condition. The UE receives and stores the NES specific CHO condition, but it is not valid until the indicated time. At the indicated start time, the UE considers the NES specific CHO condition is valid and starts the evaluation based on the configured condition. The end time indicates the deadline for handover to the candidate target cell. At the indicated end time, the UE would initiate handover procedure to the target cell even if the radio quality is not fulfilled. Otherwise, the gNB cannot turn on the NES mode or the UE may fail the DL synchronization especially in case of cell-off or carrier-off.

Proposal 3:	NES mode specific offset or parameter sets can be applied based on timing information, e.g., start time.

Proposal 4:	If NES specific CHO execution condition is not fulfilled by the indicated time, the UE will initiate the NES specific CHO procedure.

There was no consensus, but time-based approach (NTN condEventT1) was also discussed in RAN2#122. In our understanding, as the condEventT1 need to indicate absolute UTC time (i.e., t1-Threshold), it implies it can apply to the scheduling cell-off or carrier-off scenarios. It is useful to handover the UEs to the target candidate cell at a pre-scheduled time because the condEventT1 is not relative to the radio quality. Based on the current UE capability, the condEventT1 is defined as a per band and supported for NTN band only. In order to extend it for TN bands, it can be simply done by removing such NTN band restriction from the specification. It seems there are low specification impacts for introducing the timer-based approach.

Observation 3:	A condEventT1 can be useful for the case where the NES mode of the cell is scheduled and semi-statically changed.

Proposal 5:	RAN2 to address a condEventT1 for the case where the NES mode of the cell is scheduled and semi-statically changed.

When the condEventT1 is configured, the gNB has to configure condEventA3, condEventA4 or condEventA5 for the same candidate cell as a second triggering event described in TS 38.331 to support the radio quality based handover. Therefore, if the condEventT1 is configured for NES CHO execution, it is natural to configure condEventA3, condEventA4 or condEventA5 with NES mode specific parameter or parameter sets.

condExecutionCond

The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for CHO, CPA, intra-SN CPC without MN involvement or MN initiated inter-SN CPC. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. For CHO, if network configures condEventD1 or condEventT1 for a candidate cell network configures a second triggering event condEventA3, condEventA4 or condEventA5 for the same candidate cell. Network does not configure both condEventD1 and condEventT1 for the same candidate cell.

Observation 4:	In Rel-17, condEventT1 requires to configure condEventA3, condEventA4 or condEventA5 as a second triggering event for the same candidate cell.

Proposal 6:	If condEventT1 is enhanced for NES specific CHO execution, the gNB also configures condEventA3, condEventA4 or condEventA5 as a second triggering event for the same candidate cell.

For the target cell case

For the target cell case, different from the source cell case, the NES mode of candidate target cell should be known during the CHO evaluation, otherwise the UE may handover to the cell which cannot meet the UE QoS requirements. But according to the current procedure, the UE does not have to read any system information from the target cell until a random access procedure is successfully completed. Furthermore, the NES technique could be adaptively activated/deactivated in the target cell, then it is hard to identify the current mode of the target cell from the system information. Therefore, the UE does not have to obtain the NES mode of the cell from the target cell.

Proposal 7: 	RAN2 to confirm the UE does not have to obtain the NES mode of the cell from the target cell.

Considering the service continuity, it is useful for the UE to prioritize the cell with the same NES mode between the source cell and target cell. If the UE is in normal mode cell, the normal mode cell should be prioritized for CHO candidates, and vice versa. As summarized in [2], there are two major options to consider the NES mode of target cell from the source cell.

Proposal 8: RAN2 to discuss following two options to ensure that the UE can find a suitable target cell:

-	Option 1: [11/ 25] NW implementation to reconfigure candidate cells, i.e., no spec impact

-	Option 2: [9/ 25] Network provides additional prioritization for candidate cells that UE can take into consideration.

The option 1 is very simple solution as there is no enhancement. If the network preliminarily knows the NES mode of target cell when the CHO configuration is provided to the UE and the mode of cell is not dynamically changed, the UE can select a suitable target cell according to the CHO configuration. On the other hand, the option 2 is preferable in case that if multiple CHO candidate cells fulfill the condition simultaneously. Because the CHO evaluation is triggered due to NES mode change even if the UE is located near the center of the cell.

Therefore, in this case, the UE may find the multiple candidate cells with good quality. Hence, it is necessary to provide priority information to select a suitable target cell. The priority information can indicate which CHO candidate cell is high priority.

Proposal 8: 	The priority information is additionally provided by the source cell to select a suitable target cell.

Proposal 9: 	The UE selects the CHO candidate cell indicated as a high priority if multiple CHO candidate cells fulfill the condition and the priority information is provided.

Conclusion

In this contribution, we have some discussions on connected mode mobility enhancements for NES and the following observations and proposals are made:

General requirement:

Observation 1: NES techniques may be additionally supported in a future release.

Proposal 1:	RAN2 should consider a forward compatibility for NES CHO enhancements solution.

Source cell case:

Observation 2: Conditional Event A3, A4 or A5 for normal mode cell may not be fulfilled before the network activates the NES mode.

Proposal 2:	NES mode specific offset or parameter sets are introduced in Conditional Event A3, A4 or A5 for NES specific CHO execution condition.

Proposal 3:	NES mode specific offset or parameter sets is applied based on relative timing information.

Proposal 4:	If NES specific CHO execution condition is not fulfilled by the indicated time, the UE will initiate the NES specific CHO procedure.

Observation 3:	A condEventT1 can be useful for the case where the NES mode of the cell is scheduled and semi-statically changed.

Proposal 5:	RAN2 to address a condEventT1 for the case where the NES mode of the cell is scheduled and semi-statically changed.

Observation 4:	A condEventT1 requires to configure condEventA3, condEventA4 or condEventA5 as a second triggering event for the same candidate cell.

Proposal 6:	If a condEventT1 is configured for NES CHO execution, the gNB also configures condEventA3, condEventA4 or condEventA5 as a second triggering event for the same candidate cell.

Target cell case:

Proposal 7: 	RAN2 to confirm the UE does not have to obtain the NES mode of the cell from the target cell.

Proposal 8: 	The priority information is additionally provided by the source cell to select a suitable target cell.

Proposal 9: 	The UE selects the CHO candidate cell indicated as a high priority if multiple CHO candidate cells fulfill the condition and the priority information is provided.

References

- R2-2306543

- R2-2304353

- R1-2306246