3GPP TSG RAN2 #123										R2-2308642

Toulouse, France, 21 – 25 August, 2023

Agenda Item:		5.4.3

Source:			Huawei, HiSilicon

Title:		Discussion on location configuration for WLAN, BT and sensor for SON and MDT features

Document for:		Discussion and Decision

1 Introduction

At RAN2#122 meeting, RAN2 discussed location configuration and made the following progress:

R2-2306096	Discussion on location configuration for SON and MDT features	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

Agreements:

1	RAN2 to confirm understanding#1 for both LTE SON/MDT and NR SON/MDT. If agreeable, there are no specification impacts.

Understanding#1: For measurement reports and other reports (i.e. RLF report/SCG failure report), the NR can configure IE (2) only if NR configures IE (1)

FFS:

-	NR can configure IE btNameList, only if NR configures IE includeBT-Meas for one or more measurements (measurement reports)

-	NR can configure IE wlanNameList, only if NR configures IE includeWLAN-Meas for one or more measurements (measurement reports)

-	NR can configure IE sensorNameList, only if NR configures IE includeSensor-Meas for one or more measurements (measurement reports)

Proposal 3: If proposal 2 is agreeable, it is proposed to capture them in Rel-17 36.331 and 38.331 specifications, and the changes can be early implementable.

Proposal 4: If both lists are present (i.e. includeBT-Meas and btNameList, or includeWLAN-Meas and wlanNameList, or includeSensor-Meas and sensorNameList), the UE only applies btNameList/wlanNameList/ sensorNameList for both immediate MDT and RLF/SCGFailureInformation reporting.

Proposal 5: If proposal 4 is agreeable, it is proposed RAN2 to discuss whether/how to capture in the TS 36.331/TS 38.331.

For BT/WLAN/sensor related location configurations, RAN2 did not make much progress due to lack of time. In this paper, we focus on location configurations for Bluetooth, WLAN and Sensor.

It is noted that these issues have been found in both LTE and NR specifications, and we would like to address them in RAN2 and discuss how to solve it.

2 Discussion

For Bluetooth/WLAN/Sensor location info, the network can indicate some configurations to UE in order to get such information. There are the following ways:

- The IEs includeBT-Meas-r16/includeWLAN-Meas-r16/includeSensor-Meas-r16 can be used to get location information for immediate MDT. These Ies are similar to the usage of the IE includeCommonLocationInfo-r16

- The IEs btNameList-r16/wlanNameList-r16/sensorNameList-r16 can be used to request the UE to attempt to have valid location information for immediate MDT, RLF report and SCGFailureInformation. These Ies are similar to the usage of the IE obtainCommonLocation-r16

Details can be found as below.

The BT/WLAN/Sensor location configuration info in Measurement Reporting

EventTriggerConfig/PeriodicalReportConfig includeCommonLocationInfo-r16 ENUMERATED {true} OPTIONAL, -- Need R includeBT-Meas-r16 SetupRelease {BT-NameList-r16} OPTIONAL, -- Need M includeWLAN-Meas-r16 SetupRelease {WLAN-NameList-r16} OPTIONAL, -- Need M includeSensor-Meas-r16 SetupRelease {Sensor-NameList-r16} OPTIONAL -- Need M

1> if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the wlan-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the LogMeasResultWLAN, in order of decreasing RSSI for WLAN APs; 1> if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the BT-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the LogMeasResultBT, in order of decreasing RSSI for Bluetooth beacons; 1> if the includeSensor-Meas is configured in the corresponding reportConfig for this measId, set the sensor-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the sensor-MeasurementInformation; 2> if available, include the sensor-MotionInformation;

The BT/WLAN/Sensor location configuration info in OtherConfig

OtherConfig-v1610 btNameList-r16 SetupRelease {BT-NameList-r16} OPTIONAL, -- Need M wlanNameList-r16 SetupRelease {WLAN-NameList-r16} OPTIONAL, -- Need M sensorNameList-r16 SetupRelease {Sensor-NameList-r16} OPTIONAL, -- Need M obtainCommonLocation-r16 ENUMERATED {true} OPTIONAL, -- Need R

1> if the received otherConfig includes the btNameList: 2> if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; 1> if the received otherConfig includes the wlanNameList: 2> if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; 1> if the received otherConfig includes the sensorNameList: 2> if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation; NOTE 2: The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

For BT/WLAN/Sensor location info, the configuration part and UE behaviours are similar to common location. However, there is no restriction like “NR configures the field if includeCommonLocationInfo is configured for one or more measurements.”.

In our opinon, the understanding#1 for common location should be followed here, i.e. understanding#1 (i.e. Understanding#1: For measurement reports and other reports (i.e. RLF report/SCG failure report), the NR can configure IE (2) only if NR configures IE (1)). If we allow “activate BT/WLAN/Sensor” and “include BT/WLAN/Sensor” can be spearately enabled, it may introduce some flexiblities but it will also introduce some issues, e.g. user consent handling at network side.

In general, we prefer to have uniform NW/UE behaviours for location information collection, i.e. both UE and NW should check the IEs includeBT-Meas/includeWLAN-Meas/includeSensor-Meas before applying the IEs btNameList/wlanNameList/sensorNameList.

Proposal 1: It is proposed RAN2 to agree on the following:

- NR can configure IE btNameList, only if NR configures IE includeBT-Meas for one or more measurements (measurement reports)

- NR can configure IE wlanNameList, only if NR configures IE includeWLAN-Meas for one or more measurements (measurement reports)

- NR can configure IE sensorNameList, only if NR configures IE includeSensor-Meas for one or more measurements (measurement reports)

If P1 is agreeable, we suggest to capture it in LTE and NR specifications in order to have clear NW/UE behaviours.

Proposal 2: If proposal 1 is agreeable, it is proposed to capture them in Rel-17 36.331 and 38.331 specifications, and the changes can be early implementable.

For BT/WLAN/Sensor related location reporting, if the UE receives both includeBT-Meas and btNameList Ies, the UE will have two lists for BT measurements (WLAN/Sensor related location functionalities also have similar configurations). For the relation between the two lists, our understandings are as below:

- For network, both lists are independent

- For UE, if both lists are present, the UE only applies btNameList/wlanNameList/sensorNameList for both immediate MDT and RLF/SCGFailureInformation reporting

In summary, we have the following proposal:

Proposal 3: If both lists are present (i.e. includeBT-Meas and btNameList, or includeWLAN-Meas and wlanNameList, or includeSensor-Meas and sensorNameList), the UE only applies btNameList/wlanNameList/ sensorNameList for both immediate MDT and RLF/SCGFailureInformation reporting.

If proposal 4 is agreeable, we propose RAN2 to discuss whether/how to capture it in the TS 36.331/TS 38.331.

Proposal 4: If proposal 3 is agreeable, it is proposed RAN2 to discuss whether/how to capture in the TS 36.331/TS 38.331.

3 Conclusion

In this paper, we discuss the location configuration for SON and MDT features, and the location information includes common location, Bluetooth, WLAN and sensor features. Based on our analysis, we propose:

Proposal 1: It is proposed RAN2 to agree on the following:

- NR can configure IE btNameList, only if NR configures IE includeBT-Meas for one or more measurements (measurement reports)

- NR can configure IE wlanNameList, only if NR configures IE includeWLAN-Meas for one or more measurements (measurement reports)

- NR can configure IE sensorNameList, only if NR configures IE includeSensor-Meas for one or more measurements (measurement reports)

Proposal 2: If proposal 1 is agreeable, it is proposed to capture them in Rel-17 36.331 and 38.331 specifications, and the changes can be early implementable.

Proposal 3: If both lists are present (i.e. includeBT-Meas and btNameList, or includeWLAN-Meas and wlanNameList, or includeSensor-Meas and sensorNameList), the UE only applies btNameList/wlanNameList/ sensorNameList for both immediate MDT and RLF/SCGFailureInformation reporting.

Proposal 4: If proposal 3 is agreeable, it is proposed RAN2 to discuss whether/how to capture in the TS 36.331/TS 38.331.

4 References

[1] 	R2-125005, Report of 3GPP TSG RAN WG2 meeting #79