3GPP TSG-RAN WG2 Meeting #123	R2-2308649

Toulouse, France, August 21-25, 2023

Agenda item:	7.11.2.1

Source:	Sharp

Title:	MCCH Monitoring and Configuration of UE with Multicast reception in RRC_INACTIVE

WID/SID:	NR_MBS_enh-Core

Document for:	Discussion and Decision

1	Introduction

In this contribution, we discussed the issue of when UE starts to monitor multicast MCCH and the following FFS in the editor’s note captured in current running CR [1]:

Editor’s Note: FFS if “configured with MBS multicast reception in RRC_INACTIVE” should be replaced with other description or terminology.

2 Discussion

2.1 MCCH Monitoring

Upon receiving a paging message including a TMGI that UE has joint the multicast session indicated by the TMGI and allowed to be received in RRC_INACTIVE, if UE does not have the PTM configuration of the multicast session, UE should acquire the PTM configuration from multicast MCCH. But if the PTM configuration has already been configured in RRCRelease message or acquired from multicast MCCH before, then UE can start to receive the multicast session without reading the multicast MCCH.

Proposal 1 Upon receiving a paging message including a TMGI which indicates a multicast session allowed to be received in RRC_INACTIVE in the paging message, UE starts to receive the multicast session without reading the multicast MCCH if it has the PTM configuration of the multicast session.

Currently, UE acquires Multicast MCCH for PTM configuration, multicast deactivation and PTM configuration change notification. For these purpose, following two alternatives are proposed by companies about when UE starts to monitor multicast MCCH:

Alt.1 is UE starts to monitor multicast MCCH when it is released to RRC_INACTIVE;

Alt.2 is UE starts to monitor multicast MCCH when any jointed Multicast session is activated;

For Alt.1, companies have concerns on the power efficiency of the UE. For Alt.2, we need to consider how to handle the MRBs/PTM configuration for multicast has already configured in RRCRelease or acquired from multicast MCCH before. With alt.2, UE acquires the PTM configuration and monitor the deactivation of a multicast session only when at least one multicast session indicated as activated. But for notification of multicast configuration change, it is not clear if the PTM configuration can be changed when the multicast is deactivated.

Proposal 2 RAN2 to discuss if the PTM configuration can be changed when the multicast is deactivated.

If the change of PTM configuration can only happened for the multicast indicated as activated, Alt.2 is preferred. Otherwise, UE needs to monitor the notification of configuration change even if the multicast is deactivated.

Proposal 3 RAN2 to discuss when UE starts to monitor multicast MCCH.

2.2 Configuration of UE with Multicast reception in RRC_INACTIVE

According to current running CR [1], UE needs to determine whether it is configured with MBS multicast reception in RRC_INACTIVE in the following three cases:

Case 1: upon receiving SIB1, UE performs RRC Resume procedure for multicast reception if UE configured with MBS multicast reception in RRC_INACTIVE for at least one active MBS session.

Case 2: upon receiving paging message, UE may performs RRC Resume procedure for multicast reception if UE configured with MBS multicast reception in RRC_INACTIVE.

Case 3: UE applies the multicast MCCH information acquisition procedure to acquire the MBS multicast configuration information from the network if it is configured to receive MBS multicast services in RRC_INACTIVE.

Then, how UE to determine whether it is configured with MBS multicast reception in RRC_INACTIVE? At least following two solutions can be considered:

Option 1: configuring UE with MBS multicast reception in RRC_INACTIVE explicitly via RRC signalling.

In RAN2#121, RAN2 has agreed [3]:

When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration. Other dedicated RRC messages will not be used to provide PTM configuration for MBS multicast for INACTIVE.

So, RRCRelease message can also be used to configure UE with MBS multicast reception in RRC_INACTIVE. For all the three cases, UE can determine whether it is configured with MBS multicast reception in RRC_INACTIVE based on the received RRCRelease message.

Option 2: configuring UE with MBS multicast reception in RRC_INACTIVE implicitly.

In this option, for Case 1, UE may perform RRC Resume for multicast reception purpose only if there is at least one active MBS session. So, UE can consider it is configured with MBS multicast reception in RRC_INACTIVE if there is at least one active MBS session.

For Case 2, upon receiving a paging message which includes a TMGI that UE has joined and inactiveReceptionAllowed is included for the TMGI, UE can consider it is configured with MBS multicast reception in RRC_INACTIVE and start to receive multicast session if the PTM configuration is available for the multicast session indicated by the TMGI.

For Case 3, it is related to when UE need to acquire multicast MCCH information. If Alt.1 is used, explicit configuration is needed, otherwise, it does not need.

The drawbacks of option 2 are that UE capable of multicast reception in RRC_INACTIVE should always monitor the multicast MCCH for multicast session that it has joined even if the multicast session will not be allowed to be received in RRC_INACTIVE which is not power efficient.

In addition, in RAN2#121bis meeting, RAN2 had following agreements for the special UE[2]:

- A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation.

- UE-specific paging (i.e. PagingRecordList) can be used to move specific MBS multicast UE(s) to RRC_CONNECTED (i.e. legacy UE behavior).

For such type of UE, it is never be allowed to receive the multicast session in RRC_INACTIVE. For special UE, monitors multicast of MCCH is not power efficient. So, it is beneficial to configure UE with MBS multicast reception in RRC_INACTIVE explicitly. It is reasonable that Whether a UE is a special UE should be per multicast session. So,

Proposal 4 UE is configured per TMGI about whether the multicast indicated by the TMGI can be received in RRC_INACTIVE in RRCRelease message.

3 Summary

In this section, we summarized our proposals as showed below:

Proposal 1 Upon receiving a paging message including a TMGI which indicates a multicast session allowed to be received in RRC_INACTIVE in the paging message, UE starts to receive the multicast session without reading the multicast MCCH if it has the PTM configuration of the multicast session.

Proposal 2 RAN2 to discuss if the PTM configuration can be changed when the multicast is deactivated.

Proposal 3 RAN2 to discuss when UE starts to monitor multicast MCCH.

Proposal 4 UE is configured per TMGI about whether the multicast indicated by the TMGI can be received in RRC_INACTIVE in RRCRelease message.

4 References

[1] R2-2306855 RRC running CR for eMBS.

[2] RAN2#121bis chairman’s notes.

[3] RAN2#121 chairman’s notes.