3GPP TSG-RAN WG2 Meeting #123                                   R2-2308695

Toulouse, France, 21st August – 25th August, 2023

Agenda item:		7.24.2

Source:				Samsung

Title:					Discussion on positioning support for L2 U2N remote UE

Document for:		Discussion & Decision

1	Introduction

In the late RAN2#122, the following CRs were agreed in principle under R18 TEI to support positioning for L2 U2N remote UE.

- LPP CR for TS 37.355 (R2-2305854) [1].
- RRC CR for TS 38.331 (R2-2306839) [2].
- UE Capability CR for TS 38.306 (R2-2306828)
- State2 CR for TS 38.305 (R2-2305852)

The changes in LPP and RRC CRs can be summariez as below.
- In LPP CR
  * Remote UE indicator is added in CommonIEsProvideCapabilities IE
  * Remote UE indicator is added in TargetDeviceErrorCauses IEs for the GNSS, NR E-CID, NR DL-TDOA, NR DL-AoD, and NR Multi-RTT
- In RRC CR
  * An SFN-DFN offset is added to the RRCReconfigurationSidelink message sent from the L2 U2N relay UE
  * A requested posSIB list is added to the RemoteUEInformationSidelink message.
  * An indication of support for the SFN-DFN offset/forwarding the posSIB by the remote UE is added to the UECapabilityInformationSidelink message.

In this contribution, we discuss some remaining issues on positioning of L2 U2N remote UE based on the aforementioned AIP CRs.

2	Discussion

2.1	SFN-DFN offset in RRC_CONNECTED

According to the AIP RRC CR [2], the remote UE is allowed to request an SFN-DFN offset information from the connected relay UE by sending RemoteUEInformationSidelink. Then, the relay UE can provide an SFN-DFN offset information with the remote UE via RRCReconfigurationSidelink message. However, with the current AIP RRC CR, the request procedure for SFN-DFN offset is not supported when the remote UE is in RRC_CONNECTED state.

Observation 1: With the AIP RRC CR (R2-2306839), the remote UE can not request an SFN-DFN offset from the relay UE in RRC_CONNECTED state.

Meanwhile, the L2 U2N remote UE can receive positioning assistance data in RRC_CONNECTED state. Specifically, in RRC_CONNECTED state, the L2 U2N remote UE state can request posSIB(s) via DedicatedSIBRequest message and acquire the requested posSIB(s) by receiving the dedicatedSystemInformationDelivery IE included in RRCReconfiguration. The L2 U2N remote UE can also get the required positioning assistance information using the legacy LPP Assistance Data Delivery procedure. Therefore, we understand that the remote UE should be able to have an SFN-DFN offset in RRC_CONNECTED as well to understand the positioning assistance data provided with SFN timeline.

Observation 2: The remote UE can acquire positioning assistance data with SFN timeline also in RRC_CONNECTED state.

Based on the observations above, we would like to propose the following.

Proposal 1: Allow L2 U2N remote UE in RRC_CONNECTED to request an SFN-DFN from the connected L2 U2N relay UE via RemoteUEInformationSidelink message as in the TP to 38.331 in Annex.

2.2	Remote UE indication

According to the AIP LPP CR [1], the LMF can request the target device to indicate if it operates as a L2 U2N Remote UE by setting remoteUE-IndicationReq-r18 in CommonIEsRequestCapabilities. Then, the target device can include remoteUE-Indication-r18 in CommonIEsProvideCapabilities if it operates as a U2N Remote UE.

From our understanding, the intention of introducing remoteUE-IndicationReq-r18 in RequestCapabilities message is to allow the target device to include remoteUE-Indication-r18 in ProvideCapabilities messase only when the LMF requested it before. However, according to the conditional presence code for remoteUE-Indication-r18 in the AIP LPP CR [1], the target device can optionally include remoteUE-Indication-r18 regardless of the request from the LMF (i.e., presence of remoteUE-IndicationReq-r18 in CommonIEsRequestCapabilities). Thus, in some cases, the target device can provide the LMF with the remote UE indication although the LMF can not understand/use the indication and has not requested it before. This can lead unnecessary overhead on both the target device and the LMF.

Observation 3: With the AIP LPP CR (R2-2305854), the remote UE can include remoteUE-Indication-r18 in ProvideCapabilities message regardless of presence of remoteUE-IndicationReq-r18 in RequestCapabilities message.

Based on the observations above, we would like to propose the following.

Proposal 2: Clarify that L2 U2N remote UE can include remoteUE-Indication-r18 in ProvideCapabilities message only when it has been requested by the LMF before as in the TP to 37.355 in Annex.

2.3	Coverage indication

Based on the remote UE indication (remoteUE-Indication-r18 in AIP LPP CR [1]), the LMF can know that the target UE is a remote UE, and choose a proper positioning method/configuration. Specifically, the LMF can be aware that the target device has some restrictions on the use of pre-configured MG/PPW, semi-persistent/aperiodic SRS as mentioned in [3]. Also, the LMF could assume that the target device is located out of the cell coverage and do not use NR (E-)CID method or any other positioning methods that need the serving cell location.

On the other hand, there can be the case where the target is acting as a L2 U2N remote UE, but still located within the coverage of the serving cell (i.e., the serving cell of the relay UE).

Observation 4: There can be the case where the target device is acting as a L2 U2N remote UE, but still located within the coverage of the serving cell (i.e., the serving cell of the relay UE).

In that case, if the LMF can be aware that the target device operating as a remote UE is located within the serving cell coverage, it may use positioning methods requiring the serving cell’s location. For example, the LMF can determine to use NR DL-TDOA for that target UE and select the target TRPs based on the serving cell’s location. Considering the aforementioned case, we see the benefit of introducing a coverage indication for the remote UE in addition to the existing remote UE indication.

Observation 5: With a coverage indication, the LMF can determine to use the positioning methods requiring the serving cell’s location even for the target device acting as a L2 U2N remote UE.

Based on the observations above, we would like to propose the following.

Proposal 3: Introduce a coverage indication to indicate whether the target device operating as a L2 U2N remote UE is located within the coverage of the serving cell (i.e., the serving cell of the relay UE) as in the TP to 37.355 in Annex.

2.4	PrimaryCellID for L2 U2N remote UE

With the current LPP spec.[4], the target device should include its primary cell ID (i.e., primaryCellID-r15) in CommonIEsRequestAssistanceData when it requests positioning assistance data from the LMF as below.

–	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Cond Segmentation

]],

[[

periodicAssistanceDataReq-r15

PeriodicAssistanceDataControlParameters-r15

OPTIONAL,	-- Cond PerADreq

primaryCellID-r15			NCGI-r15					OPTIONAL	-- Cond NR

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NR | The field is mandatory present for NR access. The field shall be omitted for non-NR user plane support. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device.

Observation 6: With the current LPP, the target device should include its primary cell ID (i.e., primaryCellID-r15) in CommonIEsRequestAssistanceData when it requests positioning assistance data from the LMF.

However, the target device acting as a L2 U2N remote UE does not have a primary cell and can’t not properly set primaryCellID-r15, which will lead to failure of LPP assistance data transfer procedure.

Observation 7: The target device acting as a L2 U2N remote UE does not have a primary cell and can’t not properly set the primaryCellID-r15 in CommonIEsRequestAssistanceData.

To resolve this issue, we can update the field description for primaryCellID-r15 to allow the target device acting as a remote UE to set the field with the identity of the current primary cell/camping cell for the L2 U2N Relay UE.

Proposal 4: Update the field description for primaryCellID-r15 to allow the target device operating as a L2 U2N remote UE to report the identity of the current primary cell/camping cell for the L2 U2N Relay UE as in the TP to 37.355 in Annex.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

SFN-DFN offset in RRC_CONNECTED

Observation 1: With the AIP RRC CR (R2-2306839), the remote UE can not request an SFN-DFN offset from the relay UE in RRC_CONNECTED state.

Observation 2: The remote UE can acquire positioning assistance data with SFN timeline also in RRC_CONNECTED state.

Proposal 1: Allow L2 U2N remote UE in RRC_CONNECTED to request an SFN-DFN from the connected L2 U2N relay UE via RemoteUEInformationSidelink message as in the TP to 38.331 in Annex.

Remote UE indication

Observation 3: With the AIP LPP CR (R2-2305854), the remote UE can include remoteUE-Indication-r18 in ProvideCapabilities message regardless of presence of remoteUE-IndicationReq-r18 in RequestCapabilities message.

Proposal 2: Clarify that L2 U2N remote UE can include remoteUE-Indication-r18 in ProvideCapabilities message only when it has been requested by the LMF before as in the TP to 37.355 in Annex.

Coverage indication

Observation 4: There can be the case where the target device is acting as a L2 U2N remote UE, but still located within the coverage of the serving cell (i.e., the serving cell of the relay UE).

Observation 5: With a coverage indication, the LMF can determine to use the positioning methods requiring the serving cell’s location even for the target device acting as a L2 U2N remote UE.

Proposal 3: Introduce a coverage indication to indicate whether the target device operating as a L2 U2N remote UE is located within the coverage of the serving cell (i.e., the serving cell of the relay UE) as in the TP to 37.355 in Annex.

PrimaryCellID for L2 U2N remote UE

Observation 6: With the current LPP, the target device should include its primary cell ID (i.e., primaryCellID-r15) in CommonIEsRequestAssistanceData when it requests positioning assistance data from the LMF.

Observation 7: The target device acting as a L2 U2N remote UE does not have a primary cell and can’t not properly set the primaryCellID-r15 in CommonIEsRequestAssistanceData.

Proposal 4: Update the field description for primaryCellID-r15 to allow the target device operating as a L2 U2N remote UE to report the identity of the current primary cell/camping cell for the L2 U2N Relay UE as in the TP to 37.355 in Annex.

4	Reference

- R2-2305854, “Support positioning of L2 UE-to-network remote UEs [PosL2RemoteUE]”, CR#0444 to 37.355, MediaTek Inc., CATT, Huawei, HiSilicon, Qualcomm Incorporated, Xiaomi, Intel Corporation, vivo, Ericsson.

- R2-2306839, “Downlink positioning support and posSIB request for L2 UE-to-network remote UE [PosL2RemoteUE]”, CR#0444 to 38.331, MediaTek Inc., CATT, Huawei, HiSilicon, Qualcomm Incorporated, Xiaomi, Intel Corporation, vivo, Ericsson.

- R2-2210367, “On Positioning Support for L2 UE-to-Network Remote UEs”, Qualcomm Incorporated.

- 3GPP TS 37.355 v17.4.0, “LTE Positioning Protocol (LPP)”.

Annex: Text Proposals

TP to 38.331 (changes on top of the AIP RRC CR, R2-2306839)

##### 5.8.9.8.1	General

Figure 5.8.9.8.1-1: Remote UE information

This procedure is used by the L2 U2N Remote UE in RRC_IDLE/RRC_INACTIVE to inform about the required SIB(s)/posSIB(s), provide Paging related information to the connected L2 U2N Relay UE, and request the SFN-DFN offset from the connected L2 U2N Relay UE.

This procedure is used by the L2 U2N Remote UE in RRC_CONNECTED to request the SFN-DFN offset from the connected L2 U2N Relay UE.

NOTE:	MIB is not required by a L2 U2N Remote UE.

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall:

1>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before:

2>	include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has not stored a valid version, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) that the UE requires for a positioning operation, and the requested posSIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before, and the connected L2 U2N relay UE set posSIB-ForwardingSupported to true:

2>	include sl-RequestedPosSIB-List in the RemoteUEInformationSidelink to indicate the requested posSIB(s);

1>	if the UE needs the SFN-DFN offset based on the request from upper layers and the connected L2 U2N relay UE set sfn-DFN-OffsetSupported to true:

2>	set sl-SFN-DFN-OffsetRequested to true;

1>	if the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the L2 U2N Relay UE before, set sl-PagingInfo-RemoteUE as follows:

2>	if the L2 U2N Remote UE is in RRC_IDLE:

3>	include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer;

2>	else if the L2 U2N Remote UE is in RRC_INACTIVE:

3>	include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer,

4>	set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC);

3>	else:

4>	set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

When entering RRC_CONNECTED, if L2 U2N remote UE had sent sl-RequestedSIB-List, sl-RequestedPosSIB-List, and/or sl-PagingInfo-RemoteUE, the L2 U2N Remote UE shall:

1>	set the sl-RequestedSIB-List to the value release if requested before;

1>	set the sl-RequestedPosSIB-List to the value release if requested before;

1>	set the sl-PagingInfo-RemoteUE to the value release if sent before;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

Upon any change in the need of SFN-DFN offset while in RRC_CONNECTED, the L2 U2N Remote UE shall:

1>  if the UE needs the SFN-DFN offset based on the request from upper layers and the connected L2 U2N relay UE set sfn-DFN-OffsetSupported to true:

2>  set sl-SFN-DFN-OffsetRequested to true;

1> submit the RemoteUEInformationSidelink message to lower layers for transmission;

TP to 37.355 (changes on top of the AIP LPP CR, R2-2305854)

### 6.4.2	Common Positioning

#### […]

#### –	CommonIEsRequestCapabilities

The CommonIEsRequestCapabilities carries common IEs for a Request Capabilities LPP message Type.

-- ASN1START

CommonIEsRequestCapabilities ::= SEQUENCE {

...,

[[

lpp-message-segmentation-req-r14	BIT STRING {	serverToTarget	(0),

targetToServer	(1) }	OPTIONAL -- Need ON

]],

[[

remoteUE-IndicationReq-r18          ENUMERATED { true }                     OPTIONAL -- Need ON

]]

}

-- ASN1STOP

CommonIEsRequestCapabilities field descriptions

lpp-message-segmentation-req This field, if present, indicates that the target device is requested to provide its LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the server is able to send segmented LPP messages to the target device; if bit 0 is set to value 0 it indicates that the server is not able to send segmented LPP messages to the target device. If bit 1 is set to value 1, it indicates that the server is able to receive segmented LPP messages from the target device; if bit 1 is set to value 0 it indicates that the server is not able to receive segmented LPP messages from the target device.

remoteUE-IndicationReq This field, if present, indicates that the target device is requested to indicate if it operates as a L2 U2N Remote UE.

#### –	CommonIEsProvideCapabilities

The CommonIEsProvideCapabilities carries common IEs for a Provide Capabilities LPP message Type.

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

...,

[[

segmentationInfo-r14			SegmentationInfo-r14			OPTIONAL,	-- Cond Segmentation

lpp-message-segmentation-r14	BIT STRING { serverToTarget	(0),

targetToServer	(1) }	OPTIONAL

]],

[[

remoteUE-Indication-r18		BOOLEAN                             OPTIONAL,   -- Cond NR

inCoverageIndication-r18	BOOLEAN								OPTIONAL	-- Cond Remote

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| NR | This field is optionally present, need OR, for NR access if remoteUE-IndicationReq has been received from the location server in this loction session. Otherwise it is not present. |
| Remote | This field is optionally present, need OR, if remoteUE-Indication is present. Otherwise it is not present. |

CommonIEsProvideCapabilities field descriptions

segmentationInfo This field indicates whether this ProvideCapabilities message is one of many segments, as specified in clause 4.3.5.

lpp-message-segmentation This field, if present, indicates the target device's LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the target device supports receiving segmented LPP messages; if bit 0 is set to value 0 it indicates that the target device does not support receiving segmented LPP messages. If bit 1 is set to value 1, it indicates that the target device supports sending segmented LPP messages; if bit 1 is set to value 0 it indicates that the target device does not support sending segmented LPP messages.

remoteUE-Indication This field indicates whether the target device in NR access is configured as a L2 U2N Remote UE. The target device in NR access may transmit a ProvideCapabilities message with an appropriate value of this field when it starts or stops operation as a U2N Remote UE.

inCoverageIndication This field indicates whether the target device operating as a L2 U2N Remote UE in NR access is in coverage of the L2 U2N Relay UE's PCell/camping cell. The target device may transmit a ProvideCapabilities message with an appropriate value of this field when it enters or leaves the coverage of the L2 U2N Relay UE's PCell/camping cell.

–	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Cond Segmentation

]],

[[

periodicAssistanceDataReq-r15

PeriodicAssistanceDataControlParameters-r15

OPTIONAL,	-- Cond PerADreq

primaryCellID-r15			NCGI-r15					OPTIONAL	-- Cond NR

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EUTRA | The field is mandatory present for E-UTRA or NB-IoT access. The field shall be omitted for non-EUTRA and non-NB-IoT user plane support. |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| PerADreq | The field is mandatory present if the target device requests periodic assistance data delivery. Otherwise it is not present. |
| NR | The field is mandatory present for NR access. The field shall be omitted for non-NR user plane support. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device. If the target device is operating as a L2 U2N remote UE, this parameter identifies the current primary cell/camping cell for the connected L2 U2N Relay UE.

segmentationInfo This field indicates whether this RequestAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceDataReq This field indicates a request for periodic assistance data delivery, as specified in clause 5.2.1a.