3GPP TSG-RAN WG2 Meeting #123                                            R2-2308700

AUG 21 – AUG 25, 2023

Source:	CAICT

Title:	Discussion on NTN-NTN cell reselection enhancements

Agenda Item:	7.7.4.1.2

Document for:	Discussion and Decision

Introduction

RAN2 122 has discussed the NTN-NTN cell reselection enhancements and make some agreement as following:

Agreements in RAN2 121bis:

- RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements.

- For earth-moving cell, new IE is introduced to indicate the reference location of serving cell.

- For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

- For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case.

- Time-based cell reselection criteria is not pursued in R18.

Agreements in RAN2 122:

- Re-use epochTime-r17 in ntn-Config IE to provide the time reference for an Earth moving cell reference location.

- Re-use t-Service-r17 format for the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch. FFS whether we reuse exactly the same IE name as in R17 (updating the field description) or a new one

In the last meeting, RAN2 has agree a distance threshold is introduced for location-based measurement initiation and a new IE is introduced to indicate the reference location of serving cell for cell (re)selection in earth-moving system.

Discussion

Serving cell reference location can be derived by satellite’s ephemeris and epochTime, but this is an extremely rough estimate. LEO satellite is between 300-1500 km, that the satellite moving speed is much different from the serving cell moving speed. UE can get the speed of satellite only by the satellite’s ephemeris. The trajectory of satellite represented by ephemeris is an ellipse which is not parallel to the earth, meaning that the velocity of satellite does not equal to the velocity of its projection (e.g., cell reference location) on earth.

For satellites with multiple beams, it is not possible to directly derive the reference positions of other cells based solely on the ephemeris and epoch time. The reference locations of these cells are not sub satellite points. However, only the trajectories of the reference location that coincides with the sub satellite points can be obtained by using the satellite's ephemeris and epoch time.

To get more accurate derivation of the trajectory of reference location, additional information is needed to derive the serving cell reference location based on the sub-satellite point derived by the satellite’s ephemeris and epoch time. In our understanding, the cell reference location relative to serving cell, its moving speed and direction would be a straightforward approach for the derivation of the trajectory of reference location.

Proposal 1: It is suggested that system information should include additional information to allow more accurate measurements., such as reference location with epochTime, cell radius, cell movement speed, cell movement direction.

Conclusion

According to the discussion, we get below proposals:

Proposal 1: It is suggested that system information should include additional information to allow more accurate measurements., such as reference location with epochTime, cell radius, cell movement speed, cell movement direction.

Reference

- RP-221819	Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, TALES

- R2-2303140	Report of [Post121][106][NR NTN enh] NTN-NTN cell reselection (ZTE)	ZTE Corporation, Sanechips	discussion	Rel-18