3GPP TSG-RAN WG2 Meeting #123	R2-2308705

Toulouse, France, 21 August – 25 August 2023               (revision of R2-2306218)

Agenda item:	7.8.5

Source: 	Huawei, HiSilicon

Title: 	Further discussion on UAV remote identification broadcast

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #121-bis, the following agreements have been reached regarding UAV ID broadcast for NR UAVs:

Agreements:

- DAA can be supported using the same framework as used for BRID transmission over the LTE and NR PC5 interface, without any specific enhancements.

- LTE PC5 Mode-4 resource allocation is supported, and LTE PC5 Mode-3 is not supported

- NR PC5 mode-1 is not supported

- For LTE PC5, we will follow the NR PC5 framework agreements, unless explicitly identified e.g. a strong technical reason

- RAN2 assumes that BRID and DAA services will be delivered on a frequency designated by regulators

- As a baseline, we will use the existing V2X QoS framework.  FFS whether different resource pools are needed for UAV services

- No further enhancement on PC5 range for A2X broadcast services will be pursued in this release

- We will not investigate interference for BRID and DAA broadcast

- Send an LS to SA2 to:

- inform them as a result of RAN Plenary decision to re-use BRID RAN2 will only support PC5 broadcast for deconfliction in RAN in release 18.

- ask SA2 whether BRID and DAA broadcast over LTE and NR PC5 requires new QoS requirements and parameters not supported within the ranges supported for V2X

In RAN2 meeting #122, no agreements were achieved due to the time limitation. In this paper, we will discuss the SA2 LS replies.

- Discussion

In SA2 #157's meeting, SA2 replied to RAN2’s LS. For the RAN2 question "whether BRID and DAA broadcast over LTE and NR PC5 require new QoS requirements and parameters not supported within the ranges supported for V2X". The replies were as follows:

SA2 discussed that defining new PQI(PC5 5QI) value(s) is beneficial for BRID and DAA over NR PC5 so that the transport layers can distinguish A2X services from V2X services if differentiated handling is needed. The related CR work will be progressed in the future meeting. Please note that using the existing PQI values for BRID and DAA over NR PC5 is not restricted. For BRID and DAA over LTE PC5, it is considered that the existing QoS framework can be used.

We can see that SA2 will define new PQI value(s) for BRID and DAA over NR PC5. It is used to distinguish A2X services from V2X services. It is clear that there is no PC5 impact for the new PQI for A2X. In addition, SA2 has defined an A2X layer for aircraft communication by leveraging the V2X mechanism [1]. There are also no further requirements. Thus, no PC5 enhancement is needed for A2X communication.

Proposal 1: No PC5 enhancement is needed for A2X communication.

SA2 also replies to the agreement that DAA service delivery only uses broadcast. The replies were as follows:

SA2 specified DAA mechanism using broadcast, as well as DAA mechanism using unicast over NR PC5 by reusing the unicast mode V2X communication as specified in TS 23.287. Because the contents related to DAA transmitted over PC5 reference point are transparent not only to the A2X layer but also to the AS layer, SA2's understanding is that DAA specific enhancements to the existing unicast communication are not needed and the work carried out in RAN2 supports what is defined in SA2.

SA2 points out that the DAA mechanism can use unicast over NR PC5 by reusing the unicast mode of V2X communication. The contents of the DAA message are transparent to the A2X and AS layers and there is no RAN2 impact or work load. Thus, based on the SA2 reply, the unicast DAA via PC5 can be supported only if no PC5 enhancement is needed.

Proposal 2: Based on the SA2 reply, the unicast DAA via PC5 can be supported if no PC5 enhancement is needed.

- Conclusion

Proposal 1: No PC5 enhancement is needed for A2X communication.

Proposal 2: Based on the SA2 reply, the unicast DAA via PC5 can be supported if no PC5 enhancement is needed.

- Reference

- 3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".