3GPP TSG-RAN WG2 Meeting #123                                    R2-2308708

Toulouse, France, August 21-25, 2023

Agenda item:		7.17.4

Source:				Samsung

Title:					Further discussion on MUSIM gap priorities

Document for:		Discussion & Decision

1	Introduction

This contribution discusses remaining issues on MUSIM gap priorities.

2	Discussion

RAN2 has discussed how UE indicates its periodic MUSIM gap priority preference but could not make any conclusion yet.

Proposals needed to be discussed online: Proposal 3: RAN2 to discuss how UE indicates periodic MUSIM gap priority preference - Option 1: UE indicates an absolute priority for all or a subset periodic MUSIM gaps by taking into account of the Type-2 MG gap priority - Option 2: UE indicates a relative priority for all or a subset periodic MUSIM gaps, i.e. the priority is relative just among the MUSIM gaps - Option 3: wait RAN4 progress/feedback No consensus on P3 and P5.

Network will configure the UE with periodic MUSIM gaps in accordance with UE assistance information and Type-2 MG(s) without UE assistance information. But network still has no idea about the purpose of each requested MUSIM gap (e.g. paging, RRM measurements, etc). Therefore, it is questionable whether network will identify if UE prefers to be configured with a certain periodic MUSIM gap with a higher gap priority than the gap priority of Type-2 MG gap. We think that at least there should be a method of informing network of UE's preference on a certain periodic MUSIM gap with a higher gap priority than the gap priority of Type-2 MG gap.

Proposal 1: UE can indicate its preference on a certain periodic MUSIM gap with a higher gap priority than the gap priority of Type-2 MG gap to network. FFS on details.

RAN4 is still discussing how to handle aperiodic MUSIM gap priority preference/configuration as per below.

- Option 1: - The priority level of aperiodic MUSIM gap can be configured by NW A - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A - Option 2: - Aperiodic MUSIM gap by default has the highest priority level. - The gap priority level is not explicitly configured by the NW - Option 3: The aperiodic MUSIM gap by default has the highest priority level. The priority level of aperiodic MUSIM gap can be configured by NW A: Recommendations: Option 1 and 2 are agreements from RAN4 106bis. Suggest to down-select from option 1 and 2.

Thus, we don't need to have duplicated discussion.

Proposal 2: RAN2 to follow RAN4 decision on handling of gap priority for aperiodic MUSIM gap.

RAN4 is also discussing how to address collision between different MUSIM gaps as per RAN4 agreements below:

Issue 2-2-2: Solutions for collision between different MUSIM gaps - Agreements - Define two solutions for collision handling between different MUSIM gaps - 1) Priority based solution (i.e., network controls the MUSIM gaps priority) - 2) “Keep” solution (i.e., keep all collided MUSIM gaps) - FFS on the mechanism to select and/or switch between the solutions - Tentative agreement - Note: It is a common understanding that UE may request MUSIM gaps with different priorities (i.e., equal priorities are not supported)

Note that it is still under discussion how/when to use "Keep" solution i.e. whether equal priority between different MUSIM gaps is allowed or not. Without further RAN4 progress on this topic, it is a bit premature to discuss detailed signalling design of MUSIM gap priority preference/configuration.

Proposal 3: RAN2 to wait further RAN4 progress for signalling details of MUSIM gap priority preference/configuration.

3	Conclusion

RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: UE can indicate its preference on a certain periodic MUSIM gap with a higher gap priority than the gap priority of Type-2 MG gap to network. FFS on details.

Proposal 2: RAN2 to follow RAN4 decision on handling of gap priority for aperiodic MUSIM gap.

Proposal 3: RAN2 to wait further RAN4 progress for signalling details of MUSIM gap priority preference/configuration.

4	Reference