3GPP TSG-RAN WG2 Meeting #123	R2-2308710

# Toulouse, France, August 21-25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1652 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL transmissions during measurement gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the current spec, SL transmissions and receptions on PSCCH, PSSCH, PSBCH are not affected by measurement gaps. However, current spec also specifies that during measurment gaps, the UE does not perform the transmission of HARQ feedback, SR, and CSI. It could lead to ambiguity on whether SL transmissions on PSFCH for HARQ feedback is affected by measurement gaps or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE does not perform the transmission of HARQ feedback, SR, and CSI on UL during a measurement gap and SL transmisisons for HARQ feedback are not affected by the measurement gap. Impact analysis Impacted functionality: Measurement gaps This CR is mandatory for sidelink functionality to avoid unnecessary SL transmissions interruption during measurement gaps. Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no inter-operablity issue. - If the network is implemented according to this CR but the UE is not, there is no inter-operablity issue. - If one UE is implemented according to this CR while the other UE is not, the UE not implemented according to the CR could stop SL transmissions for HARQ feedback to the one UE during measurement gaps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SL transmissions for HARQ feedback may be skipped during measurement gaps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### Start of change

## 5.14	Handling of measurement gaps

During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI on UL;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running:

2>	monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH;

2>	not receive on DL-SCH.

##### End of change