3GPP TSG-RAN WG2 Meeting #123                                                        R2-2308730

Toulouse, France, 21st - 25th August, 2023

Source:              CATT

Title:	             CHO procedure enhancements

Agenda Item:	7.3.5

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting [1], the following agreements were achieved on CHO enhancements for NES:

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

In this contribution, we discuss the next level of details of the CHO enhancements based on the above agreements.

Discussion

We discuss the two NES scenarios where the network aims at handing over UEs before a NES action (in the source cell):

- Cell switch-off

- Cell DTX/DRX

  - CHO enhancements for Cell switch-off

In RAN2#122 meeting, operators clarified during on-line discussions that the typical scenario for Cell switch-off is:

- Multi-layer cells deployment (no coverage issue)

- Scenario:

  - one or more cells from that multi-layer are always switched-off at a scheduled time e.g. 10PM, whatever the number of UEs, UEs’ on-going traffic, etc at that time

  - all UEs must HO at that time to the leftover cells of the multi-layer (no coverage issue expected)

In such case, CondEvent A4 (Conditional reconfiguration candidate becomes better than absolute threshold) can be used with a low-enough threshold combined with an absolute time (not a timer) when to execute the HO.

Observation 1: For Cell switch-off scenario, CondEventA4 can be used with a low-enough threshold combined with an absolute time (not a timer) when to execute the HO.

Given the use of EventA4 for NES was already agreed in RAN2#122, it is left to study the “time based mechanism”, per the same RAN2 agreement. In our view, for the above Cell switch-off scenario, such absolute time is already made available by the legacy CondEventT1 [2]:

CondEvent T1: Time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration;

where:

condEventT1-r17 SEQUENCE { t1-Threshold-r17 INTEGER (0..549755813887), duration-r17 INTEGER (1..6000) }

and:

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

So it is clear that this t1-Threshold is exactly what we aim at in observation #1 when considering the absolute time condition.

Proposal 1: For Cell switch-off, legacy CondEventA4 and CondEventT1 address operators’ scenario/requirements and can be used to execute CHO. Nothing new is needed.

  - CHO enhancements for Cell DTX/DRX

First, we observe that, similar to Cell switch-off, the CondEvent A4 can be used with a low-enough threshold to force UEs to handoff to a choice of candidates cells. So we discuss here the associated “time-based” mechanism.

From RAN2 agreements #1 and 2 (Section 1), it is clear that the NES mode plays a role in the CHO execution. It leaves to define what “NES mode” exactly means, but for Cell DTX/DRX, as a minimum, it should reflect the activation of the Cell DTX/DRX operation. As a result, for Cell DTX/DRX, these agreements mean that such “NES mode” dependent CHO can only apply to those UEs configured with Cell DTX/DRX in first place. Because the CHO configuration must include a new CondEvent related to the Cell DTX/DRX, so it must be configured after the Cell DTX/DRX is configured.

Observation 2: For Cell DTX/DRX, a CHO which execution condition is based on source cell NES mode, can only apply to those UEs configured with Cell DTX/DRX in first place.

From the above, the procedure can only be:

- Network RRC-configures Cell DTX/DRX

- Network RRC-configures CHO

- Network activates Cell DTX/DRX

  - if CHO cond is met => trigger the HO execution

Observation 3: A CHO execution condition based on Cell DTX/DRX mode of source cell requires that Cell DTX/DRX configuration and activation are separate events.

Moreover, in RAN1#113, RAN1 made the following agreements [3]:

RAN1 has made the following agreements: - Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disabling the feature) from RAN1 perspective. - RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback. - subject to UE capability - RAN1 asks RAN2 to consider the additional support of a MAC CE based indication. - For the group common L1 signalling using PDCCH for cell DTX/DRX activation and deactivation - Based on new DCI format 2_X - DCI size budget is not increased - Number of required BDs is not increased - FFS: PDCCH monitoring configuration for the new DCI format is identical to PDCCH monitoring configuration for DCI format 2_6 if the UE monitors both DCI formats - FFS new RNTI is used

Observation 4: RAN1 agreed to support L1 signaling at least for activation/deactivation of a cell DTX.

At the same time, RAN2 agreed: “We will not introduce new L1 signalling for the purpose of CHO”. However, we understand these two agreements are not conflicting since the RAN1-agreed L1 signalling is not introduced for the purpose of CHO in first place.

Observation 5:  The agreement “We will not introduce new L1 signalling for the purpose of CHO” does not rule out that L1 Cell DTX/DRX activation (agreed in RAN1) can be used as CHO trigger.

On the contrary, since, in order to meet the CHO NES-mode-dependent execution, the Cell DTX/DRX configuration and activation have to be separate events, it makes sense that Cell DTX/DRX activation triggering the CHO is a L1 signalling, thus avoiding multiple RRC messages for the configuration and the activation.

Proposal 2: L1 Cell DTX/DRX activation (agreed in RAN1) can be used as CHO trigger

Alternately, RAN2 agreed in RAN2#121bis-e “As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released”. However, once activated by RRC (i.e. in absence of L1 Cell DTX/DRX activation), the problematic interval for non-suitable UEs is not the on-duration, where UEs are served normally but the non-active period.

Observation 6:  When Cell DTX/DRX is activated, the problematic interval for non-suitable UEs is not the on-duration, where UEs are served normally but the non-active period.

Therefore, also considering that UEs should be kept as long as possible in the source cell (because the QoE is satisfied), an implicit CHO execution time after Cell DTX/DRX has been RRC-activated should be the time the Cell DTX/DRX enters non-active period.

Figure 1: Implicit CHO execution time = start of non-active period

Proposal 3: For Cell DTX/DRX, one possible (implicit) NES CHO execution condition based on source cell NES mode is is the time when the Cell DTX/DRX enters non-active period.

Conclusion

According to the analysis in section 2, it is observed and proposed:

Observation 1: For Cell switch-off scenario, CondEventA4 can be used with a low-enough threshold combined with an absolute time (not a timer) when to execute the HO.

Proposal 1: For Cell switch-off, legacy CondEventA4 and CondEventT1 address operators’ scenario/requirements and can be used to execute CHO. Nothing new is needed.

Observation 2: For Cell DTX/DRX, a CHO which execution condition is based on source cell NES mode, can only apply to those UEs configured with Cell DTX/DRX in first place.

Observation 3: A CHO execution condition based on Cell DTX/DRX mode of source cell requires that Cell DTX/DRX configuration and activation are separate events.

Observation 4: RAN1 agreed to support L1 signaling at least for activation/deactivation of a cell DTX.

Observation 5:  The agreement “We will not introduce new L1 signalling for the purpose of CHO” does not rule out that L1 Cell DTX/DRX activation (agreed in RAN1) can be used as CHO trigger.

Proposal 2: L1 Cell DTX/DRX activation (agreed in RAN1) can be used as CHO trigger

Observation 6:  When Cell DTX/DRX is activated, the problematic interval for non-suitable UEs is not the on-duration, where UEs are served normally but the non-active period.

Proposal 3: For Cell DTX/DRX, one possible (implicit) NES CHO execution condition based on source cell NES mode is the time when the Cell DTX/DRX enters non-active period.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #122

- TS38.331, Radio Resource Control (RRC) protocol specification (Release 17), 3GPP

- R1-2306246, Reply LS on Cell DTX/DRX activation/deactivation RAN1