3GPP TSG-RAN WG2 Meeting #123                                                                  R2-2308736

Toulouse, France, 21st – 25th Aug 2023

Source:	CATT

Title:	Discussion on UE capability reporting (with TP to TS38.306 BLCR)

Agenda Item:	7.25.2‎

Document for:	Discussion and Decision

- Introduction

For Rel-18 UL Tx switching, the agreed-in-principle BLCRs have been finished via the post email discussion[1] after RAN2#122 meeting[2].

This contribution will discuss corrections to the BLCR of 38.306 for UE capability[3] and provide corresponding TP.

- Discussion

As discussed in our contribution[4], although the new IE switchingPeriodFor1T is introduced in 38.331 for signaling the 1Tx-2Tx/1Tx-1Tx switching period for Rel-18 in ULTxSwitchingBandPair, the value of this field can be same as that of uplinkTxSwitchingPeriod in 38.331 for signaling the switching period for Rel-16. If that is the case, UE may not signal switchingPeriodFor1T for indicating 1Tx-2Tx/1Tx-1Tx switching period for Rel-18 because the switching period for Rel-16 reported by UE can be applied for that band pair.

The same principle, if the value of 2Tx-2Tx switching period for Rel-18 is the same as that of 2Tx-2Tx switching period for Rel-17, UE may not signal switchingPeriodFor2T introduced in 38.331 for indicating 2Tx-2Tx switching period for Rel-18 because the 2Tx-2Tx switching period reported by UE for Rel-17 can be applied for that band pair. This way can help saving the signaling overhead for UE capability reporting.

Therefore, it should be clarified that if 2Tx-2Tx switching period for Rel-18 is not signalled, the reported 2Tx-2Tx switching period for Rel-17 is applied as the length of 2Tx-2Tx switching gap duration. And if 1Tx switching period for Rel-18 is not signalled, the reported switching period for Rel-16 is applied as the length of 2Tx-2Tx and/or 1Tx-1Tx switching.

Proposal 1: Specify that if 2Tx-2Tx switching period for Rel-18 is not signalled, the reported 2Tx-2Tx switching period for Rel-17 is applied as the length of 2Tx-2Tx switching gap duration.

Proposal 2: Specify that if 1Tx switching period for Rel-18 is not signalled, the reported switching period for Rel-16 is applied as the length of 2Tx-2Tx and/or 1Tx-1Tx switching.

Proposal 3: Adopt the TP to the 38.306 BLCR in section 5.

- Conclusion

In this contribution, we discuss corrections to the 38.306 BLCR for UE capability reporting, based on which we have the following proposals.

Proposal 1: Specify that if 2Tx-2Tx switching period for Rel-18 is not signalled, the reported 2Tx-2Tx switching period for Rel-17 is applied as the length of 2Tx-2Tx switching gap duration.

Proposal 2: Specify that if 1Tx switching period for Rel-18 is not signalled, the reported switching period for Rel-16 is applied as the length of 2Tx-2Tx and/or 1Tx-1Tx switching.

Proposal 3: Adopt the TP to the 38.306 BLCR in section 5.

- Reference

- [Post122][025][MCE] MCE TX switching CRs (Huawei)

- Minutes report, RAN2 Chairman, RAN2#122

- R2-2306913, UE capabilities of Rel-18 UL Tx switching enhancements, Huawei, NTT DOCOMO, RAN2#122

- R2-2308735, Discussion on UE capability reporting (with TP to TS38.331 BLCR), CATT, RAN2#123

- Text Proposals

START OF CHANGES

#### 4.2.7.1	BandCombinationList parameters

Unrelated part is skipped

| ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700, ULTxSwitchingBandPair-v18xy Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching and indicate support for 2-layer UL MIMO capabilities on both bands in each FeatureSet entry supporting UL 2T-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod-r16 indicates the length of UL Tx switching period of 1Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. UE shall not report the value n210us for EN-DC band combinations. n35us represents 35 us, n140us represents 140us, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod2T2T-r17 indicates the length of UL Tx switching period of 2Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. n35us represents 35 us, n140us represents 140us, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitching-DL-Interruption-r16 indicates that DL interruption on the band will occur during UL Tx switching, as specified in TS 38.133 [5] and in TS 36.133 [27]. UE is not allowed to set this field for the band combination of SUL band+TDD band, for which no DL interruption is allowed. Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5] and in TS 36.133 [27]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. The capability is not applicable to the following band combinations, in which DL reception interruption is not allowed: - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern Indicates UE supports R18 dynamic UL Tx switching across up to 4 bands as defined in TS 38.214 [12] and TS 38.101-1 [2]. The capability signaling further comprises of the following parameters: - For a band pair indicated by bandIndexUL1-r16 and bandIndexUL2-r16, UE shall indicate support of 2-layer UL MIMO in FeatureSet on both bands for 2T-2Tx switching, or indicate support of 2-layer UL MIMO on one band and 1-layer MIMO on the other band for 1Tx-2Tx switching, or indicate support of 1-layer UL MIMO on both bands for 1Tx-1Tx switching. - uplinkTxSwitchingOptionForBandPair-r18 indicates whether switchedUL or dualUL or both switching options is supported for a given band pair as specified in TS 38.214 [12]. - uplinkTxSwitchingMaintainedUL-Trans-r18 indicates that UL transmission with the number of Tx chain unchanged is allowed on the band during UL Tx switching between two other bands when UL Tx switching is configured as specified in 38.101-1 [2]. Absence of this field means there is interruption in all UL bands during the switching. - Field encoded as a bit map, where bit N is set to "1" if UL transmission on nth UL band is allowed. The leading / leftmost bit corresponds to the first UL band of this band combination excluding the two bands of the band pair, the next bit corresponds to the second UL band of this band combination excluding the two bands of the band pair and so on. - switchingPeriodFor2T-r18 indicates the length of 2Tx-2Tx switching period, switchingPeriodFor1T-r18 indicates the length of 1Tx-2Tx switching and/or 1Tx-1Tx switching period, as specified in TS 38.214 [12], TS 38.101-1 [2]. n35us represents 35 us, n140us represents 140us, and so on, as specified in TS 38.101-1 [2]. If switchingPeriodFor2T-r18 is absent, uplinkTxSwitchingPeriod2T2T-r17 is applied as the length of 2Tx-2Tx switching peirod. If switchingPeriodFor1T-r18 is absent, uplinkTxSwitchingPeriod-r16 is applied as the length of 2Tx-2Tx and/or 1Tx-1Tx switching. | BC | FD | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| uplinkTxSwitchingMinimumSeparationTime-r18 Indicates the minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots as specified in TS 38.214 [12]. The field is mandatory when UE supports dynamic UL Tx switching across more than two bands. | BC | CY | N/A | FR1 only |
| UplinkTxSwitchingAdditionalPeriodDualUL-r18 Indicates the UL Tx switching period for switching between a band pair and another band pair or another band, when R18 UL Tx switching is configured by uplinkTxSwitchingMoreBands-r18. If the capability is not reported, the switching period reported in switchingPeriodFor2T-r18 or switchingPeriodFor1T-r18 applies, as specified in TS 38.214 [12] and TS 38.101-1 [2]. - bandPairIndex1-r18/bandPairIndex2-r18 xx refers to the xxth band pair entry in the band pair list indicated by ULTxSwitchingBandPair-r16. - bandIndex-r18 xx refers to the xxth band entry in this band combination. - switchingAdditionalPeriodDualUL-r18 indicateds the length of switching period for switching between one band pair indicated by bandPairIndex1-r18 and another band pair indicated by bandPairIndex2-r18 or another band indicated by bandIndex-r18. - n35us represents 35 us, n140us represents 140us, and so on, as specified in TS 38.101-1 [2]. A UE supporting this feature shall also indicate the support of dualUL switching option for the band pair(s) indicated in bandPairIndex1-r18/bandPairIndex2-r18. | BC | No | N/A | FR1 only |
| uplinkTxSwitching-OptionSupport-r16 Indicates which option is supported for dynamic UL 1Tx-2Tx switching for inter-band UL CA and (NG)EN-DC. switchedUL represents option 1 as specified in TS 38.214 [12], dualUL represents option 2 as specified in TS 38.214 [12], both represents both option 1 and option2 as specified in TS 38.214 [12]. UE shall not report the value both for (NG)EN-DC case. The field is mandatory for inter-band UL CA and (NG)EN-DC case where UE supports dynamic UL 1Tx-2Tx switching. | BC | CY | N/A | FR1 only |
| uplinkTxSwitching-OptionSupport2T2T-r17 Indicates which option is supported for dynamic UL 2Tx-2Tx switching for inter-band UL CA. switchedUL represents option 1 as specified in TS 38.214 [12], dualUL represents option 2 as specified in TS 38.214 [12], both represents both option 1 and option2 as specified in TS 38.214 [12]. The field is mandatory for inter-band UL CA cases where UE supports dynamic UL 2Tx-2Tx switching. The UE indicating support of this feature shall indicate support of at least one common switching option between uplinkTxSwitching-OptionSupport2T2T-r17 and uplinkTxSwitching-OptionSupport-r16. | BC | CY | N/A | FR1 only |
| uplinkTxSwitching-PowerBoosting-r16 Indicates the support of 3dB boosting on the maximum output power for UE transmission under the operation state in which 2-port transmission can be supported on carrier2 in case of inter-band UL CA case where UE supports dynamic UL Tx switching. A UE shall only indicate this capability in case the UE supports power class 3 for inter-band UL CA for the band combination as defined in TS 38.101-1 [2]. | BC | No | N/A | FR1 only |
| UplinkTxSwitchingBandParameters-v1700 Contains the UL Tx switching specific band parameters for a given band combination. The capability signalling comprises of the following parameters: - bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination. bandIndex xx refers to the xxth band entry in the band combination. - uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 indicates support of the uplink codebook subset for the carrier(s) on a band capable of two antenna connectors on which UE supports dynamic UL 2Tx-2Tx switching with another band in the band combination. UE indicating support of full coherent codebook subset shall also support non-coherent codebook subset. If this field is absent, - When 2Tx-2Tx switching between two bands is configured by uplinkTxSwitching-2T-Mode-r17, the per BC UE capability reported in uplinkTxSwitching-PUSCH-TransCoherence-r16 is applied, and if this field and uplinkTxSwitching-PUSCH-TransCoherence-r16 are both absent, the UE capability reported in pusch-TransCoherence is applied when uplink Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. - When R18 dynamic UL Tx switching is configured by uplinkTxSwitchingMoreBands-r18, the UE capability reported in pusch-TransCoherence is applied when uplink Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. | BC | No | N/A | FR1 only |
| uplinkTxSwitching-PUSCH-TransCoherence-r16 Indicates support of the uplink codebook subset when uplink 1Tx-2Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. UE indicating support of full coherent codebook subset shall also support non-coherent codebook subset. If the field is absent, the supported uplink codebook subset indicated by pusch-TransCoherence applies when the uplink switching is triggered between last transmitted SRS and scheduled transmission. | BC | No | N/A | FR1 only |

END OF CHANGES