3GPP TSG-RAN WG2 Meeting #123	R2-2308737

Toulouse, France, 21st Aug – 25th Aug, 2023

Agenda Item:	7.3.2

Source:	vivo

Title:	Remaining issues for cell DTX-DRX

Document for:	Discussion and Decision

Introduction

In R2#122, the following agreements were made [1]:

Agreements: 1 UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy. 2 UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy. 3 Working assumption: When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX. 4 Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX). The behavior is captured in stage 2 spec 5 When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy). This includes DL HARQ feedback.

In this contribution, we will discuss the remaining issues for Cell DTX/DRX.

Discussion

Cell DTX/DRX activation/deactivation

Background

In R2#121bis, the following agreements were made [2], and an LS was sent to RAN1 [3]:

Agreements 4. As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released. 5. From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details. Ask about feasibility and reliability of using L1 signaling. Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration. Extract a few key benefits of dynamic signaling from email discussion and online discussions

In R1#113, the following agreements were made in the Reply LS [4]:

With respect to questions asked by RAN2, RAN1 has made the following agreements: - Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disabling the feature) from RAN1 perspective. - RAN1 supports the group common L1 signaling using PDCCH for Cell DTX/DRX activation and deactivation without HARQ feedback. - subject to UE capability - RAN1 asks RAN2 to consider the additional support of a MAC CE based indication.

MAC CE based A/D signaling

According to the RAN1 Reply LS, group common L1 A/D signaling without HARQ feedback is supported, and RAN2 is asked to consider the additional support of a MAC CE based indication. From our understanding, group common L1 A/D signaling is beneficial for reducing signaling overhead and A/D latency. However, HARQ feedback is not supported for it, and hence the problem of misaligned understanding of Cell DTX/DRX activation/deactivation status between the gNB and the UE may happen, e.g. the gNB transmits group common L1 A/D signaling and supposes all the feature-supported UEs will have entered cell-DTX/DRX-activated status after reception of the signaling, but some UEs may mis-detect the group common L1 A/D signaling and remain cell-DTX/DRX-deactivated status. This is the reason why RAN1 asks RAN2 to consider support of L2 A/D signaling.

The misaligned understanding problem will cause the following impacts:

- UE may waste energy for PDCCH monitoring during cell DTX non-active period, and SR/CG transmission during cell DRX non-active period, if the UE assume Cell DTX/DRX is still deactivated while the gNB has actually already activated it;

- UE may miss PDCCH/SPS detection during cell DTX non-active period, if the UE assume Cell DTX/DRX is still activated while the gNB has actually already deactivated it;

Observation 1: Without MAC CE based A/D signaling for Cell DTX/DRX, the gNB and the UE may have misaligned understanding of Cell DTX/DRX activation/deactivation status when Cell DTX/DRX is activated/deactivated by group common L1 signaling, which may cause the UE wasting energy or missing PDCCH/SPS detection.

Therefore, as a supplementary solution for activation/deactivation with group common L1 signaling, we propose RAN2 to additionally support MAC CE based A/D signaling for Cell DTX/DRX.

Proposal 1: RAN2 to confirm support of MAC CE based A/D signaling for Cell DTX/DRX.

If MAC CE based A/D signaling for Cell DTX/DRX is supported, one may be confused about how the gNB detects the misaligned understanding between the gNB and the UE, and how the gNB decides to use group common L1 signaling or MAC CE based signaling. From our observation, these can be up to the gNB implementation, and examples are shown as below:

- If the amount of CONNECTED UEs is limited, the gNB can just use MAC CE based signaling;

- For the UEs capable of handling Cell DTX/DRX A/D by group common L1 signaling, the gNB uses group common L1 signaling. The gNB uses MAC CE based signaling for the rest UEs;

- For the UEs capable of handling Cell DTX/DRX A/D by group common L1 signaling, the gNB first uses group common L1 signaling for A/D, but keeps SR/CG reception ON for a while. If the gNB finds some UEs capable of L1 signalling are still transmitting SR/CG on the cell DRX non-active period, the gNB knows those UEs have not entered cell-DTX/DRX-activated status, and thus sends Cell DTX/DRX A/D MAC CE to them.

Proposal 2: It is up to the gNB implementation on how to detect misaligned understanding of the Cell DTX/DRX activation/deactivation status between the gNB and the UE.

Proposal 3: If RAN2 confirms to support MAC CE based A/D signaling for Cell DTX/DRX, it is up to the gNB implementation how to use group common L1 signaling or MAC CE based signaling for Cell DTX/DRX activation/deactivation.

Cell DTX/DRX activation/deactivation type

In existing configured grant mechanism, 2 types of configured grant are supported, i.e. PUSCH transmission is semi-statically configured by RRC without the detection of an UL grant in a DCI in Type 1, or semi-persistently scheduled by an UL grant in a valid activation DCI in Type 2 [5][6].

In comparison, it remains unclear and confusing whether Cell DTX/DRX also needs to support 2 types of activation, considering RAN1 progress on introducing group common L1 signalling, i.e. Cell DTX/DRX is implicitly activated once configured by RRC as type 1, and Cell DTX/DRX can be firstly deactivated when configured by RRC but later activated by L1 signalling as type 2. According to the previous RAN2 agreements, at least type 1 is supported. The only difference between the two types is whether Cell DTX/DRX can be explicitly deactivated when configured by RRC. Therefore, we propose RAN2 to discuss whether type 2 is also supported.

Proposal 4: Besides the agreed baseline to support Cell DTX/DRX being implicitly activated once configured by RRC, RAN2 to discuss whether Cell DTX/DRX can be explicitly deactivated when configured by RRC, i.e. in this case, c.

Behaviors during Cell DTX/DRX non-active period

PDCCH for retransmission during cell DTX non-active period

Case 1: UE C-DRX is configured

In R2#122, the following WA was made [1]:

Agreements: 3 Working assumption: When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

Allowing retransmission for dynamic scheduling during cell DTX non-active time does not cause severe NES gain loss as it is corner case that the gNB schedules PDCCH for new transmission at the end of cell DTX on-duration and the UE happens to fail in decoding the PDCCH. Also, the gNB can rely on implementation to avoid to schedule dynamic grants/assignments with PDCCH for new transmission at the end of cell DTX on-duration to avoid potential retransmissions during cell DTX non-active period. Therefore, we propose RAN2 to confirm the WA.

Proposal 5: RAN2 to confirm the following working assumption: When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

Case 2: UE C-DRX is not configured

In existing mechanism, if UE C-DRX is not configured, there is no such HARQ-RTT-Timer/retransmissionTimer, and the UE should monitor PDCCH on all the PDCCH monitoring occasions, regardless whether it is monitoring PDCCH for new transmission or retransmission. According to the working assumption made for case 1, it is reasonable for the UE not configured with C-DRX to also monitor PDCCH (for retransmission) at the cell DTX non-active period, if necessary. For instance, if the UE responses HARQ NACK at the end of cell DTX active period, the UE should monitor PDCCH (for retransmission) at the following cell DTX non-active period. From the NW side, it is still up to the NW whether it schedules retransmissions out of the Cell DTX active period.

Proposal 6: If UE is not configured with C-DRX, it is up to the network whether it schedules retransmissions out of the Cell DTX active period.

If P5 is agreeable, the only question left is whether to introduce cell DTX HARQ-RTT-Timer/retransmissionTimer for cell DTX when C-DRX is not configured. As a baseline, the UE should monitor PDCCH (for retransmission) at the following cell DTX non-active period if necessary. If the cell DTX HARQ-RTT-Timer/retransmissionTimer are introduced, the UE can avoid some unnecessary PDCCH monitoring occasion during cell DTX non-active period, and the UE behavior will be similar to the case when UE C-DRX is configured, but further discussion is needed on how cell DTX HARQ-RTT-Timer/retransmissionTimer works if C-DRX is also configured. An easy way forward is cell DTX HARQ-RTT-Timer/retransmissionTimer can only be configured when C-DRX is not configured. Therefore, we propose RAN2 to discuss how UE monitors PDCCH for retransmission out of Cell DTX active period.

Proposal 7: If proposal 6 is agreeable, RAN2 to further discuss how UE monitors PDCCH for retransmission out of Cell DTX active period:

- Opt. 1: If UE is not configured with C-DRX, the UE should monitor PDCCH for retransmission if necessary, e.g. the UE responses HARQ NACK at the end of the cell DTX active period, the UE should monitor PDCCH (for retransmission) out of cell DTX active period;

- Opt. 2: If and only if UE is not configured with C-DRX, the NW configures cell DTX HARQ-RTT-Timer/retransmissionTimer to the UE. When the cell DTX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

Conclusion

Based on the discussion, we have the following observation and proposals:

Observation 1: Without MAC CE based A/D signaling for Cell DTX/DRX, the gNB and the UE may have misaligned understanding of Cell DTX/DRX activation/deactivation status when Cell DTX/DRX is activated/deactivated by group common L1 signaling, which may cause the UE wasting energy or missing PDCCH/SPS detection.

Proposal 1: RAN2 to confirm support of MAC CE based A/D signaling for Cell DTX/DRX.

Proposal 2: It is up to the gNB implementation on how to detect misaligned understanding of the Cell DTX/DRX activation/deactivation status between the gNB and the UE.

Proposal 3: If RAN2 confirms to support MAC CE based A/D signaling for Cell DTX/DRX, it is up to the gNB implementation how to use group common L1 signaling or MAC CE based signaling for Cell DTX/DRX activation/deactivation.

Proposal 4: Besides the agreed baseline to support Cell DTX/DRX being implicitly activated once configured by RRC, RAN2 to discuss whether Cell DTX/DRX can be explicitly deactivated when configured by RRC.

Proposal 5: RAN2 to confirm the following working assumption: When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

Proposal 6: If UE is not configured with C-DRX, it is up to the network whether it schedules retransmissions out of the Cell DTX active period.

Proposal 7: If proposal 6 is agreeable, RAN2 to further discuss how UE monitors PDCCH for retransmission out of Cell DTX active period:

- Opt. 1: If UE is not configured with C-DRX, the UE should monitor PDCCH for retransmission if necessary, e.g. the UE responses HARQ NACK at the end of the cell DTX active period, the UE should monitor PDCCH (for retransmission) out of cell DTX active period;

- Opt. 2: If and only if UE is not configured with C-DRX, the NW configures cell DTX HARQ-RTT-Timer/retransmissionTimer to the UE. When the cell DTX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

References

- R2#122 meeting minutes

- R2#121bis meeting minutes

- R2-2304357 Draft LS on Cell DTX/DRX activation/deactivation

- R1-2306169 Draft Reply LS on Cell DTX/DRX activation/deactivation

- TS 38.214, V17.6.0

- TS 38.321, V17.5.0