3GPP TSG-RAN WG2 Meeting #123	R2-2308738

Toulouse, France, 21st Aug – 25th Aug, 2023

Agenda Item:	7.3.2

Source:	vivo, Fraunhofer IIS, Huawei, HiSilicon, LG Electronics Inc., MediaTek Inc., NEC, Xiaomi

Title:	UE C-DRX configuration used upon cell DTX activation

Document for:	Discussion and Decision

Introduction

During the R18 NES SI phase, RAN2 agreed that it is beneficial to align UE DRX with network DTX and DRX alignment among multiple UEs [1]. In this contribution, we will continue to discuss about the UE DRX alignment issue.

Discussion

Background

At R2#119bis, RAN2 reached the following agreement on UE DRX alignment issue for cell DTX/DRX [1]:

Agreements => It is beneficial to align UE DRX with network DTX and DRX alignment among multiple UEs. Details are FFS, including UE transmission/reception behavior during DTX. RAN2 to study the alignment.

At R2#121bis, RAN2 sent an LS to RAN1 to ask about the feasibility and reliability of using L1 signalling for Cell DTX/DRX activation and deactivation, as companies saw a benefit with using group common signalling to reduce the signalling overhead caused by multiple dedicated RRC messages [2]. After that, RAN1 replied at R1#113 as below [3]:

With respect to questions asked by RAN2, RAN1 has made the following agreements: - Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disabling the feature) from RAN1 perspective. - RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback. - subject to UE capability - RAN1 asks RAN2 to consider the additional support of a MAC CE based indication.

UE C-DRX configuration used upon cell DTX activation

With the agreements made for cell DTX so far, we are now able to illustrate an example process of how cell DTX is configured and activated/deactivated from the UE’s perspective (since cell DTX and cell DRX can be configured and operated separately, we only mention cell DTX which is related to the UE DRX alignment issue for simplicity), as show in Fig. 1:

- Step 1: RRC CONNECTED UEs are configured with cell DTX, and cell DTX is implicitly activated upon configuration (as agreed at R2#121bis [4]);

- Step 2a: If cell DTX configuration is released by RRC, cell DTX is deactivated once the RRC configuration is released;

- Step 2b: If cell DTX configuration is not released by RRC, cell DTX is deactivated upon reception of L1 deactivation signaling.

- Step 3 (after Step 2b): cell DTX is activated again, upon reception of RRC reconfiguration with updated cell DTX configuration, or L1 activation signaling.

- Step 4 (after Step 3): GoTo Step 2a/2b;

Fig. 1: example process of Cell DTX activation/deactivation

As mentioned in the background section, RAN1 has agreed on support of group common L1 signaling for cell DTX/DRX activation and deactivation, with the benefit of saving signalling overhead caused by multiple dedicated RRC messages, i.e. the gNB does not have to reconfigure the cell DTX configuration of the UEs one by one for cell DTX/DRX activation and deactivation.

Observation 1: Support of group common L1 signaling for cell DTX/DRX activation and deactivation is beneficial for reducing signalling overhead caused by multiple dedicated RRC messages for cell DTX/DRX activation and deactivation.

After the last RAN2 meeting, the Rapporteur organized a post-meeting email for discussion on several issues of cell DTX/DRX [5]. For the discussion of UE DRX alignment issue, we observe from the comments that the majority companies prefer to respect the agreement made during the SI phase that it is beneficial to align UE DRX with cell DTX. Meanwhile, the Rapporteur pointed out the existing mechanism is to reconfigure C-DRX of UEs by UE-specific RRC messages, and this procedure may cause high signalling overhead when cell DTX is activated/deactivated. Besides, the signaling delay is much larger than L1 signaling for cell DTX activation/deactivation. Therefore, how to efficiently change the C-DRX of multiple UEs should be discussed by RAN2.

Observation 2: To change the C-DRX of multiple UEs when cell DTX is activated/deactivated, the legacy mechanism of reconfiguring C-DRX of UEs by UE-specific RRC messages causes high signalling overhead and larger signaling delay than L1 signaling for cell DTX activation/deactivation, which weakens the benefit from introducing group common L1 signaling for cell DTX activation/deactivation.

A simple solution for reducing C-DRX reconfiguration signalling overhead is that the gNB configures an additional C-DRX configuration containing the C-DRX parameters used upon cell DTX activation for the UE along with the cell DTX configuration. In this way, the gNB can avoid reconfiguring the UE C-DRX every time when cell DTX is activated/deactivated. This solution is consistent with the motivation of introducing group common L1 signalling for reducing signalling overhead.

Observation 3: With an additional C-DRX configuration used upon cell DTX activation configured together with cell DTX configuration, the gNB can reduce signalling overhead and latency caused by multiple RRC reconfiguration messages for aligning UE C-DRX with cell DTX.

The existing UE C-DRX configuration is shown in Annex 5.1 [5]. Apart from shortDRX, all the other C-DRX parameter fields are mandatory in the existing C-DRX configuration. However, for the purpose of aligning C-DRX with cell DTX, some companies point out that the NW does not always need to reconfigure all the mandatory C-DRX parameters in the legacy DRX-config, unless it wants to change those parameters, i.e. the mandatory parameter fields in legacy C-DRX configuration can be optional in the additional C-DRX configuration used upon cell DTX activation. For instance, if the gNB only wants to change the C-DRX startOffset of a UE, the C-DRX configuration used upon cell DTX activation can include only the C-DRX startOffset.

Observation 4: For alignment between UE C-DRX and cell DTX, the gNB does not always need to reconfigure all the C-DRX parameters.

Some companies also comment that if C-DRX of a UE is already aligned with the to-be-activated cell DTX, the NW can avoid to reconfigure C-DRX for such UE, which is also true. Therefore, the IE of the additional C-DRX configuration used upon cell DTX activation can also be optional.

In conclusion, we propose that for alignment between UE C-DRX and cell DTX, the gNB can configure an additional UE C-DRX configuration used upon cell DTX activation together with cell DTX/DRX parameters, which contains optional C-DRX parameters that the UE shall use when cell DTX is activated.

Proposal 1: For alignment between UE C-DRX and cell DTX, the gNB can configure an additional UE C-DRX configuration together with cell DTX/DRX parameters, which is enabled implicitly upon cell DTX activation. The additional C-DRX configuration contains optional C-DRX parameters which the UE shall use when cell DTX is activated.

Conclusion

Based on the discussion, we have the following observations and proposal:

Observation 1: Support of group common L1 signaling for cell DTX/DRX activation and deactivation is beneficial for reducing signalling overhead caused by multiple dedicated RRC messages for cell DTX/DRX activation and deactivation.

Observation 2: To change the C-DRX of multiple UEs when cell DTX is activated/deactivated, the legacy mechanism of reconfiguring C-DRX of UEs by UE-specific RRC messages causes high signalling overhead and larger signaling delay than L1 signaling for cell DTX activation/deactivation, which weakens the benefit from introducing group common L1 signaling for cell DTX activation/deactivation.

Observation 3: With an additional C-DRX configuration used upon cell DTX activation configured together with cell DTX configuration, the gNB can reduce signalling overhead and latency caused by multiple RRC reconfiguration messages for aligning UE C-DRX with cell DTX.

Observation 4: For alignment between UE C-DRX and cell DTX, the gNB does not always need to reconfigure all the C-DRX parameters.

Proposal 1: For alignment between UE C-DRX and cell DTX, the gNB can configure an additional UE C-DRX configuration together with cell DTX/DRX parameters, which is enabled implicitly upon cell DTX activation. The additional C-DRX configuration contains optional C-DRX parameters which the UE shall use when cell DTX is activated.

References

- R2#119bis meeting minutes

- R2-2304568 LS on Cell DTX/DRX activation/deactivation

- R1-2306246 Reply LS on Cell DTX/DRX activation/deactivation

- R2#121bis meeting minutes

- TS 38.331, V17.5.0

Annex

Existing C-DRX configuration IE

–	DRX-Config

The IE DRX-Config is used to configure DRX related parameters.

DRX-Config information element

-- ASN1START

-- TAG-DRX-CONFIG-START

DRX-Config ::=                      SEQUENCE {

drx-onDurationTimer                 CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                 ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL                INTEGER (0..56),

drx-HARQ-RTT-TimerUL                INTEGER (0..56),

drx-RetransmissionTimerDL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

}

DRX-ConfigExt-v1700 ::=                 SEQUENCE {

drx-HARQ-RTT-TimerDL-r17                INTEGER (0..448),

drx-HARQ-RTT-TimerUL-r17                INTEGER (0..448)

}

-- TAG-DRX-CONFIG-STOP

-- ASN1STOP