3GPP TSG-RAN WG2 Meeting #123	R2-2308748

Toulouse, France, 21– 25 August 2023	R2-2306312

Agenda item:	7.22.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Low-power WUS in RRC_CONNECTED

WID/SID:	FS_NR_LPWUS - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The SI on LP-WUS was approved in [RP-222644] with the following objective related to RAN2:

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

RAN1 agreed that both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI in RAN1#110bis-e, and made some further progress for the RRC_CONNECTED as follows:

RAN1#112bis-e - For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID - indication to wake-up to PDCCH monitoring. - Other information candidates are not precluded - Study pros and cons of including above information to LP-WUS. - Note: the information may be explicitly or implicitly indicated. - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques.

This contribution presents our view on possible area that RAN2 can discuss for RRC_CONNECTED mode.

# 2	Discussion

## 2.1	LP-WUS and PDCCH monitoring

For use of LP-WUS, the start condition needs to be defined. One simple way would be that the gNB indicates LP-WUS monitoring to the UE. The other way would be to provide a condition to a UE and the UE starts the LP-WUS monitoring when the condition is satisfied. For instance, in RRC_CONNECTED, the UE may use RSRP as one criterion to determine whether to monitor the LP-WUS or not to ensure successful reception of LP-WUS. If the UE starts LP-WUS monitoring based on a condition, it is important that the UE and the gNB have the same understanding whether the LP-WUS is used or not in the UE side. Thus, RAN2 need to discuss how to synchronize between the gNB and the UE, and what can be the start point of LP-WUS monitoring.

Proposal 1: RAN2 to discuss how the UE starts LP-WUS monitoring.

After receiving the LP-WUS, the UE begins monitoring the PDCCH. However, at some point, the UE needs to switch back to LP-WUS monitoring. There would be various factors to consider when to end the PDCCH monitoring and switch back to LP-WUS monitoring. For example, if the UE does not expect further scheduling or the link quality is not sufficient to receive LP-WUS well, it may switch back, or it may switch back periodically. The way of switching back may depend on whether DRX is configured or not, as DRX already has mechanisms to determine whether scheduling is less likely. For instance, the DRX cycle and DRX timers for active time can indicate that scheduling is less expected. Therefore, RAN2 should take DRX into account when discussing how to switch back to LP-WUS monitoring.

Proposal 2: RAN2 to discuss how to switch back to PDCCH monitoring from LP-WUS monitoring by considering the DRX, but not limited to DRX.

## 2.2	LP-WUS and DRX

LP-WUS and DRX are both used to conserve power, so it is important to know how they interact. In DRX, PDCCH monitoring is controlled by the active time, and the active time is determined by various DRX timers, such as the drx-onDurationTimer and drx-InactivityTimer. In this regards, RAN2 need discuss how LP-WUS affects the active time and DRX timers in DRX operation. For instance, we should consider how the LP-WUS monitoring period is considered in the DRX active time and how LP-WUS monitoring affects the start of DRX timers.

Proposal 3: RAN2 to discuss the LP-WUS impact on the DRX active time and DRX timers.

## 2.3	UL synchronization

LP-WUS allows the main radio to be in ultra-low power saving mode and wakes-up the main radio when scheduling is expected. However, since the UE cannot predict when the UE will receive the LP-WUS, it would be desirable for the UE to be ready for data transfer at any time in order not to sacrifice the latency. For instance, the UE needs to be in UL synchronized state as otherwise RA procedure would be initiated when uplink transmission is required. In RRC_CONNECTED, periodic CSI report can be one way that the gNB estimates the UL timing. However, periodic CSI report is mainly used for proper scheduling decision, and we think, for UL synchronization, more sporadic CSI report would be fine.

Proposal 4: RAN2 to discuss how to manage the uplink synchronization status while the UE monitors the LP-WUS in RRC_CONNECTED.

## 2.4	UE assistance for LP-WUS

The UE may prefer certain LP-WUS configuration or in some cases the UE may prefer or not prefer to monitor LP-WUS. In some cases, the UE may be out of coverage area of LP-WUS, and LP-WUS monitoring is not preferred.

Proposal 5: RAN2 to discuss UE Assistance Information for LP-WUS.

## 2.5	SPS and CG

For RRC_CONNECTED, RAN1 agreed that LP-WUS indicates to wake-up PDCCH monitoring. It was further agreed that RLM/BFD/CSI as well as RRM measurements are performed by the main radio. However, it is unclear whether the main radio keeps the SPS and CG resources activated while monitoring the LP-WUS. If SPS and CG are assumed deactivated while monitoring LP-WUS, the gNB needs to send the LP-WUS to wake up the main radio and then activates the SPS/CG via DCI, i.e., the activation command. This would introduce latency and power consumption because the UE waits for receiving the activation command after starting the PDCCH monitoring. Moreover, the gNB may intend to rely on SPS/CG scheduling instead of dynamic scheduling in main radio, in which cases further PDCCH monitoring after SPS/CG activation would be needlessly consuming power. Thus, RAN2 needs to study how to reduce unnecessary PDCCH monitoring when using SPS/CG. For instance, we can consider that the LP-WUS indicates SPS/CG activation.

Proposal 6: RAN2 to discuss the LP-WUS impact on SPS and CG Type 2.

## 2.6	LP-WUS resource management

RAN1 is currently discussing how to group the UEs targeted by the LP-WUS, which would provide some benefit from overhead perspective, and agreed in RAN1#112bis-e that:

For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID - indication to wake-up to PDCCH monitoring.

When multiple UEs are grouped for LP-WUS, it should be flexible to assign LP-WUS resources and take them back to/from the LP-WUS group because the LP-WUS resources are limited. Considering the gain from LP-WUS, it would be generally preferred by the UE to use LP-WUS instead of eDRX, or there can be a UE which cannot maintain its battery life longer with eDRX, or which needs to join the LP-WUS group immediately with any urgent reason. Given that there can be a maximum number of UEs that can share the LP-WUS resource, RAN2 need to discuss how to assign/distribute the LP-WUS resources to a group of UEs for a case e.g., if a new UE needs to join the group while the LP-WUS resources are already served to the maximum number of UEs. For instance, the new UE can indicate how urgent it is to use the LP-WUS, and the gNB can collect preference from the UEs which already joined the LP-WUS group.

Proposal 7: RAN2 to discuss how to allocate the LP-WUS resources flexibly, especially considering the case where the group of UEs share the LP-WUS resources.

## 2.7	LP-WUS and MBS

In SA2#150-e, SA2 asked RAN1 in [S2-2203020] whether it is useful for NG-RAN to receive from 5GC information on NR UE capabilities (e.g. RedCap) of the target recipients of MBS data in MBS broadcast mode. It was further discussed and concluded in RAN#96 that Rel-17 specifications do not prevent any UE, including RedCap UEs, to support MBS [RP-221861]. Therefore, it would be reasonable to see how MBS can be benefit from LP-WUS.

For current MBS operation, even if a UE has successfully received an initial transmission for a MBS multicast service, it does not know, if the next scheduling DCI is either:

- another new transmission

- a retransmission on behalf of other UEs using that same MBS multicast service but which NACKed the initial transmission

until the main radio has decoded the New Data Field sent in the scheduling DCI.

This means the main radio must be fully ON to decode the New Data Field in the scheduling DCI, even though, that specific UE already receives the initial transmission successfully and the next scheduling DCI just indicates a retransmission for other UEs. If the UE can know in advance whether the next scheduling DCI is for another initial transmission or retransmission, the UE can stay in lower power state for longer. For instance, if the UE successfully receives the initial transmission and LP-WUS indicates the next scheduling DCI is for retransmission, i.e., not for another initial transmission, the UE stays in a lower power state and does not need to wake up the main radio.

Proposal 8:  RAN2 to discuss how the LP-WUS can help UE not to wake-up the main radio unnecessarily by considering the MBS operation.

# 3	Conclusion

This document has proposed the followings:

Proposal 1: RAN2 to discuss how the UE starts LP-WUS monitoring.

Proposal 2: RAN2 to discuss how to switch back to PDCCH monitoring from LP-WUS monitoring by considering the DRX, but not limited to DRX.

Proposal 3: RAN2 to discuss the LP-WUS impact on the DRX active time and DRX timers.

Proposal 4: RAN2 to study how to manage the uplink synchronization status while the UE monitors the LP-WUS in RRC_CONNECTED.

Proposal 5: RAN2 to discuss UE Assistance Information for LP-WUS.

Proposal 6: RAN2 to discuss the LP-WUS impact on SPS and CG Type 2.

Proposal 7: RAN2 to discuss how to allocate the group LP-WUS resources flexibly, especially considering the case where the group LP-WUS resources are already served by maximum number of UEs.

Proposal 8:  RAN2 to discuss how the LP-WUS can help UE not to wake-up the main radio unnecessarily by considering the MBS operation.