3GPP TSG-RAN WG2 #123                                                                                            R2-2308750

Toulouse, France, 21st Aug. – 25th Aug. 2023                                         Revision of R2-2306297

Agenda Item:	    7.4.4

Source: 	    MediaTek Inc.

Title:		    Remaining issues for CHO with candidate SCG

Document for:	    Discussion and decision

# 1 Introduction

In this contribution, we discuss below objective #4 from mobility enhancement work item [1].

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

R2 has made the following agreements related to objective #4

In RAN2#119

- Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17.

- CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported.

- FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

Chair: NOTE that the above agreements are NOT intended to describe the Stage3 signalling details.

In RAN2#120

- Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation)

In RAN2#121

- RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18

- The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above

In RAN2#121bis

For the CHO+CPC case:

- When both CHO and CPC conditions are met, both CHO and CPC cell change is executed.

- Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen).

- Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

In RAN2#122

- P3: The CHO execution conditions (for candidate PCells) and CPA/CPC execution conditions (for candidate PSCells) are provided based on the source MeasConfig.

- P4: For CHO execution conditions, the source MN determines the execution conditions on candidate PCells, based on the source MCG MeasConfig.

- P5: For CPA/CPC execution conditions, the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g. event A4 threshold).

- P6: The candidate MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions (e.g. event A4 threshold). According to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE.

- FFS how, if to support event A3/A5.

- P8: For CHO with candidate SCGs for CPA/CPC, the RRCReconfigurtaion message in one CHO container includes one MCG configuration and one SCG configuration (i.e. similar to Rel-17 CHO with SCG configuration).

- P9: The execution conditions associated with one CHO container includes both CHO execution condition(s) and CPA/CPC execution condition(s), i.e. triggering conditions on both candidate PCell and candidate PSCell.

- P10: If there are multiple candidate PSCells associated with one candidate PCell, the NW can provide multiple CHO configurations for the same candidate PCell, i.e. each one contains one MCG configuration (for the same candidate PCell) and one SCG configuration (for different candidate PSCell).

- P12: When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions.

- For CHO+CPC we only consider execution when BOTH conditions are met.

(When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

We would like to discuss some remaining open issues for CHO with candidate SCG in this contribution.

# 2 Discussion

## 2.1 Indicator for selected conditional reconfiguration

For CHO with candidate SCG, it is possible that one PSCell is associated with multiple candidate PSCell (multiple SN). Therefore, there is a need to how to indicate the selected target SCG to the target MN. Similar to CPA and inter-SN CPC, the UE will indicate the selected conditional reconfiguration ID in selectedCondRRCReconfig. However in this CHO with candidate SCG procedure, the selectedCondRRCReconfig-r17 is generated by source MN and target MN is not aware of it. So, some mechanism is needed than simply reuse old signalling.

There could be two different options

- Option 1: NOT reuse selectedCondRRCReconfig-r17 is not reused. The UE sends some other information (e.g. target PSCell PCID) to target MN.

- Option 2: Reuse selectedCondRRCReconfig-r17, assuming RAN3 defines the inter-node coordination to align the CondReconfigId between the source MN and each candidate MN.

Both options are fine but we think that option 2 is preferred. It seems a little bit wired to have another RRC indicator that is similar to selectedCondRRCReconfig. If maximum number of conditional reconfigurations should already be coordinated between network notes, it should be possible to indicate the select indexes to candidate MN.

Proposal 1: For CHO with candidate SCG, reuse to selectedCondRRCReconfig to indicate the ID of the selected conditional reconfiguration. Assuming RAN3 defines the inter-node coordination to align the CondReconfigId between the source MN and each candidate.

## 2.2 Supporting of event A3/A5

It is still FFS on whether to support event A3/A5 based on MCG measConfig for CHO with candidate SCG. In this procedure, the PCell is changed and we think the source PSCell may not be a DC pair with target PCell. Therefore, evaluating on source PSCell is not really needed and event A4 should be sufficient. It is also our understanding, that in most deployment, multiple SNs are associated with one MN, not multiple MNs to be associated with one SN. We consider event A3/A5 a further controlling flexibility but not really essential. Thus we suggest not to support A3/A5 event.

Proposal 2: For CHO with candidate SCG, do NOT support condEventA3 or condEventA5 for the execution conditions for candidate PSCells.

## 2.3 Use CHO with candidate SCG in failure recovery

It is not concluded yet whether we want to use CHO with candidate SCG in CHO failure recovery procedure.

In general (starting from Rel-16 CHO), the UE can execute CHO upon two separate events:

- Event 1: Regular CHO execution (i.e., condExecutionCond becomes fulfilled)

- Event 2: CHO recovery (i.e., execute CHO instead of legacy re-est signalling in case attemptCondReconfig is configured and suitable cell found for re-est matches with PCell in condRRCReconfig for CHO)

For both Event 1 and Event 2, the UE performs some evaluation of the target PCell signal level/quality:

- Event 1: The evaluation is as per meas IDs defined by condExecutionCond

- Event 2: The evaluation is by leveraging suitable cell criteria (cell selection criterion S as per 38.304)

In Rel-17, support for CHO with SCG configuration was added. And those CHO configurations are considered also for Event 2.

- For Rel-17 CHO with SCG, only PCell criteria is evaluated by the UE, for both events 1 and 2.

- This means that UE might not hear the PSCell at all, or might hear it poorly, but anyway Event 1 and Event 2 apply similar level of evaluation (PCell evaluation only).

For Rel-18 CHO with candidate SCGs, the UE evaluates both PCell and PSCell criteria (condExecutionCond and condExecutionCondPSCell, respectively) for Event 1. If Rel-18 CHO configurations with candidate SCGs were used for CHO recovery (i.e., Event 2), the Event 1 and Event 2 evaluations would no longer be ~equivalent with each other, since the UE only evaluates PCell quality (suitable cell criteria) in Event 2. For this reason, we think Rel-18 CHO configurations with candidate SCGs should not be used in CHO recovery.

Proposal 3: Rel-18 CHO configurations with candidate SCGs should NOT be used in CHO recovery.

It could be captured for example as shown in Annex (based on running CR ASN.1).

# 3 Conclusions

Based on the discussion in section 2, we have following proposals.

Proposal 1: For CHO with candidate SCG, reuse to selectedCondRRCReconfig to indicate the ID of the selected conditional reconfiguration. Assuming RAN3 defines the inter-node coordination to align the CondReconfigId between the source MN and each candidate.

Proposal 2: For CHO with candidate SCG, do NOT support condEventA3 or condEventA5 for the execution conditions for candidate PSCells.

Proposal 3: Rel-18 CHO configurations with candidate SCGs should NOT be used in CHO recovery.

# 4 References

[1] RP-222332, “Revised WID on Further NR mobility enhancements”, MediaTek

# Annex: RRC Change for not supporting CHO with candidate SCG in CHO recovery procedure

5.3.7.3            Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>  ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>  stop timer T311;

1>  if T390 is running:

2>  stop timer T390 for all access categories;

2>  perform the actions as specified in 5.3.14.4;

1>  stop the relay (re)selection procedure, if ongoing;

1>  if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG or mobility from NR failure, and

1>  if attemptCondReconfig is configured; and

1>  if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>  if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>  if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>  apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

Editor’s note: FFS whether the legacy CHO recovery mechanism applies to the configuration for CHO including target MCG and candidate SCGs.

NOTE 1:    It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.