3GPP TSG-RAN WG2 Meeting #123	R2-2308791

Toulouse, France, 21 - 25 August 2023

Agenda item:	7.17.2

Source: 	Qualcomm Incorporated

Title:  	Procedures for Dual-Active MUSIM

Document for:	Discussion and decision

# Introduction

The main objective of the Rel-18 MUSIM WI ([1]) is for the UE to signal changes to its capability when it is in Connected mode in another network and thus has to split its resources between the two active connections.

RAN2#119bis-e has discussed the scenarios and signaling for dual-active MUSIM and made several preliminary agreements. RAN2#121 and 121bis-e have made good progress on the signaling for the temporary UE capability restrictions as well as the necessary procedures ([2], [3]).

In this contribution, we discuss the open issues related to the signaling and procedures for temporary UE capability changes for dual-active MUSIM.

# Discussion

RAN2#121 has discussed for the UE to report capability restrictions before configured with UAI. This was for the scenario when the UE is moving from Idle/Inactive to Connected mode on NW A while already being in Connected mode in NW B. The problem was further discussed in RAN2#121bis-e with the following agreements:

Support “early indication” from UE to network during RRC connection setup/resume procedure.

FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

Similar to UAI, the UE behavior here should be configured by the NW. Since the UE is coming from Idle or Inactive mode, such configuration should be provided before the UE initiates the connection setup/resume.

Proposal 1: The NW can allow “early indication” via dedicated (e.g. RRC Release for Inactive mode) or broadcast signalling (e.g. in SIB1).

The first FFS from the above RAN2#121bis-e agreement is which message to use for the early indication. This should be signaled no later than msg5, i.e. when the UE moves to Connected mode. Then the choice would be between msg3 and msg5. Even though it can be argued that an earlier indication can be advantageous, using msg3 is very difficult from ASN.1 perspective due to the lack of spare bits.

Observation 1: There are not sufficient spare bits to signal “early indication” in msg3.

The goal of the early indication is for the NW not to assume full capability in scheduling the UE. In the current deployments, we haven’t observed NWs doing this before msg5.

Observation 2: An earlier indication than msg5 is not needed for the existing NR deployments.

Proposal 2: The UE signals the “early indication” for temporary UE capability restriction in msg5.

RAN2 has not yet discussed on the type of information to be sent as part of the “early indication”. At the very least, this shoud be a Boolean “flag”. Other types of additional information (e.g. max MIMO layers) can further be considered in order to help NW predict UE behavior and also schedule the UE accordingly.

Proposal 3: The “early indication” includes a Boolean flag (e.g. TRUE means that the capability is restricted). Other types of additional information (e.g. MIMO layers) are FFS.

RAN2#121bis-e has also agreed on “proactive” approach with the following agreement:

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

Our understanding is that the main difference between “proactive” and “reactive” approaches is that in the former case, the UE can indicate restrictions to its capabilities independent of its current RRC configuration. This is also reflected in the RAN2 agreement above. For example, the UE can indicate that it can not support SCells in a particular band even if it is not currently configured in that band. The rest of the UE reporting should be same between proactive and reactive approaches, i.e. the same set of UE capabilities should be allowed to be restricted. However, the signaling could be different since the UE can refer to the existing capabilities differently. For example, release of serving cells can be done via using a cell index in the reactive approach while ARFCN or band info can be used in the proactive approach.

Propsoal 4: In principle, the same set of UE capabilities for temporary restrictions are applicable to both proactive and reactive approaches. ASN.1 may differ between the two in some cases and can be FFS.

There can be two options on configuring “proactive” approach. We can consider this as a separate optional feature and enable accordingly. The second option would be to always allow this as part of MUSIM capability restrictions and thus we don’t need to distinguish between two approaches.

Proposal 5: RAN2 should discuss whether “proactive” approach is an additional/optional behavior for UE capability restriction or is always allowed as part of dual-active MUSIM feature.

Irrespective of whether it is a separate feature or not, “proactive” approach should be used when it is allowed by the NW, similar to other UAI behavior.

Proposal 6: “Proactive” approach is allowed based on NW configuration, similar to other UAI reporting.

When the UE reports its capability restrictions, the expected behavior from the NW is to use RRC reconfiguration to re-configure the UE accordingly. There were papers in previous RAN2 meetings which discussed what the UE should do if it does not get such reconfiguration.

In Rel-17 MUSIM, when the UE requests to leave NW A due to conflicts with NW B, the UE starts a timer upon transmission of the request and upon expiry, the UE can leave the NW A connection. The justification for thiwas was that the activity on NW B could be more important and/or time critical.

For dual-active MUSIM, the problem is similar. If the UE has to absolutely reduce its capabilities on NW A (e.g. NW B is E-UTRA where changes to UE capabilities is not possible), then we should discuss if a default behavior by the UE when NW A response is delayed can be applicable.

In some cases, the UE can know at which time the capability restrictions will have to be applied. For example, when the UE starts a connection request on NW B or gets a reconfiguration on NW B, it can predict when that will cause a restriction on NW A. Thus, it is necessary for the UE to receive the NW A response before that time.

Proposal 7: RAN2 to acknowledge the scenario where the UE capability restrictions have to happen before a certain time and discuss solutions when NW A response does not happen before that time.

In RAN2#121bis-e, there is an FFS which was repeated for all UE capabilties considered to be restricted. This was whether the signaling is “only for NW A or also NW B”.

RAN2 specifications define the UE behavior from a UE connected to a single NW. Therefore, it is not possible to specify UE behavior across two networks. NW A and NW B are only used in discussions and agreements but not in the specifcations. We use these names only as a label. This principle was also followed in Rel-17 MUSIM.

Observation 3: RAN2 specifications define the UE behavior only from one NW perspective.

Proposal 8: As in Rel-17 MUSIM, RAN2 will introduce new Rel-18 UE behavior from only NW A perspective.

# Conclusion

In this document, we discussed the procedures for UE capability restrictions for dual-active MUSIM further and propose the following:

Proposal 1: The NW can allow “early indication” via dedicated (e.g. RRC Release for Inactive mode) or broadcast signalling (e.g. in SIB1).

Observation 1: There are not sufficient spare bits to signal “early indication” in msg3.

Observation 2: An earlier indication than msg5 is not needed for the existing NR deployments.

Proposal 2: The UE signals the “early indication” for temporary UE capability restriction in msg5.

Proposal 3: The “early indication” includes a Boolean flag (e.g. TRUE means that the capability is restricted). Other types of additional information (e.g. MIMO layers) are FFS.

Propsoal 4: In principle, the same set of UE capabilities for temporary restrictions are applicable to both proactive and reactive approaches. ASN.1 may differ between the two in some cases and can be FFS.

Proposal 5: RAN2 should discuss whether “proactive” approach is an additional/optional behavior for UE capability restriction or is always allowed as part of dual-active MUSIM feature.

Proposal 6: “Proactive” approach is allowed based on NW configuration, similar to other UAI reporting.

Proposal 7: RAN2 to acknowledge the scenario where the UE capability restrictions have to happen before a certain time and discuss solutions when NW A response does not happen before that time.

Observation 3: RAN2 specifications define the UE behavior only from one NW perspective.

Proposal 8: As in Rel-17 MUSIM, RAN2 will introduce new Rel-18 UE behavior from only NW A perspective.

# References

- RP-223492, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo

- R2-2301902, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)

- RAN2#121bis-e Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)