3GPP TSG-RAN WG2 Meeting #123	Tdoc R2-2308794

Toulouse, France, August 21 – 25, 2023

Agenda Item:	7.3.2

Source:	Ericsson

Title:	Further aspects on cell DTX/DRX

Document for:	Discussion, Decision

# 1	Introduction

A new work item (WI) on Network energy savings for NR was approved at RAN#98 [1], and the following objectives were specified:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]. - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]. - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains: - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]. - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals. - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements. - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]. - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]. - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4].

This contribution was revised to take into account the latest discussions related to cell DTX/DRX.

# 2	Discussion

## 2.1	Cell DTX/DRX Configuration, Activation and Deactivation

The parameters such as periodicity, start slot/offset, and on-duration allow for a basic functionality of the Cell DTX/DRX. However, they do not ensure that the ongoing packet deliveries to the UE are not disrupted by cell DTX/DRX non-active periods. Ensuring this is, however, essential since interrupting the on-going packet delivery will lead to unnecessary latency at the UE side, and it also does not bring in NW energy savings, as the NW has to ultimately transmit this data. Therefore, following the nominal on duration and off duration times may lead to inability to maintain responsive traffic patterns or meet relevant QoS/QoE requirements for individual UEs. For this reason, we think that there is a need for mechanisms that allow for extending Cell DTX and Cell DRX on duration times, which in turn would allow for accommodating the ongoing packet deliveries, transmissions, and retransmissions.

NW energy saving friendly mechanisms for extending Cell DTX and Cell DRX on duration times are needed to ensure that the ongoing packet deliveries to the UE are not disrupted, to maintain responsive traffic patterns and to meet relevant QoS/QoE requirements for individual UEs.

There are different solutions that one can consider to achieve the objective summarized in Observation 1, e.g. a Cell DTX/DRX inactivity timer that the NW can configure per UE as a part of Cell DTX/DRX RRC configuration, or use dynamic signalling for extending Cell DTX and Cell DRX on duration times. For dynamic signalling, it is something to be discussed in RAN1 given the last LS RAN2 have sent to RAN1 (R2-2304568). Moreover, while RAN2 stated in this LS possible benefits from a lower layer signalling, it is not clear what usage RAN1 may conclude for L1 signalling. We should thus focus on the configuration aspect that would allow for extension of Cell DTX and Cell DRX on duration times. For this purpose, it would be beneficial to introduce a Cell DTX/DRX inactivity timer.

This timer could be UE-specific, meaning that when the Cell DTX/DRX on duration time is extended for the purpose of serving one UE, the extension does not need to be communicated to the other UEs (i.e., the cell can be considered as active from one UE’s perspective and as non-active from another UE’s perspective). Furthermore, to additionally support a NW energy saving friendly design of Cell DTX/DRX inactivity timer, the NW can choose to configure UEs with different Cell DTX/DRX inactivity timer values that are, for example, shorter than typical UE C-DRX inactivity timer values or the NW can choose to not configure Cell DTX/DRX timer for certain UEs. How and whether to configure the Cell DTX/DRX inactivity timer, the NW can decide for each UE individually (e.g., based on QoS/QoE requirements of a particular UE, NW energy saving objectives or feedback from the UE).

UE specific inactivity timer is introduced to extend on duration time of Cell DTX/DRX, i.e. after expiration of the timer, the UE considers the Cell DTX/DRX to be in non-active period.

Since there has been a considerable progress now on how a single Cell DTX/DRX pattern can be configured for a serving cell, it should also be discussed whether multiple configurations per serving cell should be allowed. Similarly, as discussed above (e.g. to better serve different UEs with different QoS requirements), configuration of multiple patterns should be allowed. We think the specification impact of allowing multiple patterns would not be big, as long as a single pattern is active at a time.

Specification impact of multiple patterns can be minimized if there is at maximum a single pattern active at a time, for a given serving cell.

This could also bring flexibility for the NW and additional sleeping opportunities, e.g. if no UEs require at a certain moment a longer on duration time, the pattern could be changed faster on those UEs such that the NW could be able to sleep longer.

Multiple patterns can facilitate a faster switching to sleeping mode in cases where a longer on duration is not required in the cell.

To this end, having two patterns that can be switched between them would already allow some flexibility and would also limit the complexity on how many patterns for the UE to be aware. Therefore, it can be assumed to support at least two patterns. Whether to support anything beyond two patterns could be further discussed.

The UE can be provided with at least two Cell DTX/DRX patterns. Only one pattern is active at a time.

Since RAN2 agreed as a baseline that cell DTX/DRX is activated/deactivated implicitly by RRC signalling (i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released), having multiple patterns implies that the NW also informs the active pattern. An indicator is only required if at least two patterns are configured. This could also apply to the common L1 signaling agreed by RAN1 for cell DTX/DRX activation/deactivation.

When at least two cell DTX/DRX configurations are provided to the UE, the UE is also provided with the indicator of the pattern that should be active.

Moreover, RAN1 also asks RAN2 to consider the additional support of a MAC CE based indication. This can be beneficial to timely and reliably change between cell DTX/DRX patterns when required.

If multiple cell DTX/DRX patterns are configured for a serving cell, MAC CE can be used to activate/deactivate the patterns. Only one pattern is active at a time.

## 2.2	Coexistence of Cell DTX and UE C-DRX

Another aspect that needs to be addressed is how Cell DTX/DRX and UE C-DRX mechanisms coexist together when the UE is configured with both Cell DTX/DRX and UE C-DRX at the same time. Regarding this aspect, we do not see the need to specify that the on-duration of C-DRX falls within Cell DTX on-duration nor the need to specify that the periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity. In our opinion, these would impose restrictions on the UE C-DRX and Cell DTX configurations that are unnecessary. The UE C-DRX on-duration times do not need to completely fall under Cell DTX on-duration times for the two features to function together. Instead, it is enough if the UE C-DRX on-duration times at least partially overlap with Cell DTX on-duration times, which can be achieved by the NW through the appropriate configuration of UE C-DRX and Cell DTX parameters. We understand that the NW needs to ensure some overlap, which can be confirmed in RAN2, but we do not think this implies in any specification impact.

A functional coexistence of UE C-DRX and Cell DTX can be ensured by the NW through the appropriate UE C-DRX and Cell DTX configurations. No specification impact.

## 2.3	Signalling Aspects

In RAN2#121-bis-e, the following remained as FFS:

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support high priority traffic

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

On the first FFS concerning SR configuration, we think allowing exceptional SR can also imply in monitoring of PDCCH for dynamic grants/assignments. But since it was agreed that the UE does not monitor PDCCH in this case, we do not see the need to make an exception for SR only.

RAN2 will not introduce a configuration of exceptional SR occasions allowed during Cell DRX non-active periods, i.e. the UE never sends SR during cell DRX.

## 2.4	CA/DC

Cell DTX/DRX is a per cell feature and thus nothing prevents it to be used in dual connectivity, i.e. MN and SN can decide to use it for its own cell group. We understand there is no impact of the use of cell DTX/DRX by both MN and SN, and no coordination is required since the cell patterns should be independent for each cell group.

Cell DTX/DRX can be configured for spCell. No coordination is required for cell DTX/DRX between MN and SN.

For carrier aggregation, it was discussed in SI phase that the feature could be defined per serving cell. If defined in this way, it should naturally be applicable to other cells as well. There is no inter-dependency expected between the use of the feature for each cell separately. In case cross carrier scheduling is being used or any other feature that creates dependency between cells, it should be up to the NW how to ensure that cell DTX/DRX is activated/deactivated and/or reconfigured such that these features can work together.

Cell DTX/DRX can be configured for SCells and each SCell may have its own DTX/DRX configuration.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	NW energy saving friendly mechanisms for extending Cell DTX and Cell DRX on duration times are needed to ensure that the ongoing packet deliveries to the UE are not disrupted, to maintain responsive traffic patterns and to meet relevant QoS/QoE requirements for individual UEs.

Observation 2	Specification impact of multiple patterns can be minimized if there is at maximum a single pattern active at a time, for a given serving cell.

Observation 3	Multiple patterns can facilitate a faster switching to sleeping mode in cases where a longer on duration is not required in the cell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE specific inactivity timer is introduced to extend on duration time of Cell DTX/DRX, i.e. after expiration of the timer, the UE considers the Cell DTX/DRX to be in non-active period.

Proposal 2	The UE can be provided with at least two Cell DTX/DRX patterns. Only one pattern is active at a time.

Proposal 3	If multiple cell DTX/DRX patterns are configured for a serving cell, MAC CE can be used to activate/deactivate the patterns. Only one pattern is active at a time.

Proposal 4	A functional coexistence of UE C-DRX and Cell DTX can be ensured by the NW through the appropriate UE C-DRX and Cell DTX configurations. No specification impact.

Proposal 5	RAN2 will not introduce a configuration of exceptional SR occasions allowed during Cell DRX non-active periods, i.e. the UE never sends SR during cell DRX.

Proposal 6	Cell DTX/DRX can be configured for spCell. No coordination is required for cell DTX/DRX between MN and SN.

Proposal 7	Cell DTX/DRX can be configured for SCells and each SCell may have its own DTX/DRX configuration.

# References

RP-223540, New WID: Network energy savings for NR

R2-2301903, 3GPP TSG-RAN WG2 Meeting #121, Athens, February 27 – March 3, 2023; Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

R2-23xxxx Outcome of [POST121][312][NES] DTX/DRX - Configuration/ activation/ deactivation and alignment (Huawei), Huawei

TR 38.864, Study on network energy savings for NR

R1-2302131, 3GPP TSG-RAN WG1 Meeting #112, Athens, February 27 – March 3, 2023; Discussion Summary #2 for enhancements on cell DTX/DRX mechanism

R2-23xxxx Report of [POST121][311][NES] DTX/DRX - gNB and UE behaviours, InterDigital