3GPP TSG-RAN WG2 Meeting #123	R2-2308806

Toulouse France, August 21st - 25th 2023

Agenda Item:	7.19.2

Source:	Ericsson

Title:	PTW configuration and fallback mechanism for RRC_INACTIVE eDRX

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has been discussing supporting reduced capability (RedCap) NR devices WI (RP-223544). The WID has the following objective on enhancement of extended DRX:

- Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement.

Discussion on eDRX enhancements in RRC_INACTIVE continued in RAN2#122 and the following agreements were made:

- UE can support Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t support Rel-17 INACTIVE eDRX. - A cell can allow Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t allow Rel-17 INACTIVE eDRX, but the cell must allow IDLE eDRX. - We confirm the working assumption: UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX. - A UE configured with Rel-18 INACTIVE eDRX will fallback to use INACTIVE RAN DRX if it is either not configured with Rel-17 INACTIVE eDRX or the cell does not allow Rel-18 INACTIVE eDRX and Rel-17 INACTIVE eDRX.

In this contribution, we continue to discuss the enhancements of eDRX functionality in RRC_INACTIVE. The contribution will address the remaining open issues:

- Whether the length of RAN PTW can be configured with a larger value than the length of CN PTW.

- What kind of information RAN should provide to the CN, i.e., RAN PTW length

# 2	PTW configuration for enhanced eDRX

In Rel-17, eDRX cycles up to 10.24 s for RRC_INACTIVE and up to 10485.76 s for RRC_IDLE were specified. The value range for eDRX cycles in RRC_INACTIVE is extended in Rel-18 so that it includes cycles greater than 10.24 s and up to 10485.76 s allowing RAN to configure longer eDRX cycles in RRC_INACTIVE compared to Rel-17. When a UE is configured with an eDRX cycle longer than 10.24 s in RRC_IDLE or RRC_INACTIVE mode, it monitors the paging within a paging time window (PTW). A UE in RRC_IDLE monitors for CN paging in CN PTW, while the UE in RRC_INACTIVE monitors for CN paging in CN PTW and RAN paging in the RAN PTW. The length of PTW ranges from 1.28 s to 40.96 s in steps of 1.28 s for both IDLE eDRX and INACTIVE eDRX.

Figure 1: Illustration of PTW configuration for INACTVE and IDLE eDRX

For a Rel-18 UE, there are two possible scenarios regarding the length of RAN PTW as shown in Fig. 1. In one case RAN PTW can be configured with a value smaller than or equal to the length of CN PTW and in the other case RAN PTW can be configured with a value larger than the length of CN PTW. The length of PTW is important from the perspective of missing paging occasion and synchronization between network nodes. Currently the value range of PTW allows RAN and CN to configure values up to 40.96 s and the question here is whether RAN PTW length should be allowed to be larger than the CN PTW length. Even though we expect that typically RAN PTW length is configured with a value smaller than or equal to the value of CN PTW, we do not see the need to introduce such limitation.

It should be possible for RAN to configure the length of RAN PTW with a value smaller, equal to, or larger than the length of CN PTW.

If proposal 1 is agreed, the following can be observed:

With a possibility of such PTW configuration, a UE can be paged within the RAN PTW and CN PTW, within CN PTW when it is outside RAN PTW, or within RAN PTW when it is outside CN PTW.

# 3	PTW Configuration from RAN to AMF

It has been agreed earlier that RAN provides the INACTIVE eDRX cycle to CN for handling MT data.

##### 5.31.7.2.4	Paging for extended DRX for RRC_INACTIVE in NR connected to 5GC

For NR, the NG-RAN may request the CN to handle mobile terminated (MT) communication for the UE configured with eDRX for RRC_INACTIVE state by means of the Connection Inactive procedure with CN based MT communication handling Procedure (see clause 4.8.1.1a of TS 23.502 [3]). This allows the CN to apply high latency communication functions as specified in clause 5.31.8. The NG-RAN provides the eDRX cycle value for RRC_INACTIVE to AMF (i.e. >10.24s). Based on the request from NG-RAN, the AMF responds to NG-RAN and informs other NFs (e.g. SMF and UPF) involved in downlink data or signalling handling and trigger the data buffering as specified in clause 4.8.1.1a of TS 23.502 [3].

When MT data or signalling arrives for a UE in RRC_INACTIVE state, the other NFs communicate with the AMF for delivery of MT data or signalling. The AMF calculates the UE reachability based on the eDRX cycle value for RRC_INACTIVE state provided by NG-RAN and triggers NG-RAN paging via an N2 message if the UE is considered reachable as specified in clause 4.8.2.2b of TS 23.502 [3], otherwise the AMF informs other NFs applying high latency communication functions as specified in clause 5.31.8 based on eDRX cycle for RRC_INACTIVE (e.g. an Estimated Maximum Wait Time is calculated based on eDRX cycle value for RRC_INACTIVE).

When the UE enters RRC_CONNECTED state if the NG-RAN had sent the indication for the CN to handle mobile terminated (MT) communication, NG-RAN informs the AMF that the UE is now in RRC_CONNECTED state as specified in clause 4.8.2.2 of TS 23.502 [3]. The AMF then informs other NFs that the UE is now reachable using the high latency communication functions as specified in clause 5.31.8 and MT data and signalling can be delivered to the UE.

Now the question is whether the RAN configured PTW should be provided to the CN or not. A UE operating in eDRX longer than 10.24 s, monitors POs during a periodic Paging Time Window (PTW) configured for the UE. The start position of PTW is determined following the recent agreement from RAN2 meeting i.e., RAN and CN will use the PTW start calculation based only on CN eDRX cycle. The only remaining thing is the length of PTW, which the CN is unaware of as it is configured by RAN for RAN PTW. Since, RAN and CN can configure different PTW length for RAN PTW and CN PTW accordingly, it makes sense to inform CN about the RAN PTW configuration e.g., when a UE is configured with long eDRX > 10.24 s, the CN shall send the NGAP request message for RAN Paging to NG-RAN based on the Paging eDRX Cycle and RAN PTW length. The information about RAN PTW length is important for cases where DL data arrives after PTW starts, then AMF may want to know if data notification can still be triggered to the UE before PTW ends and needs to know the RAN PTW. Otherwise, if CN is not aware of the RAN PTW length, the CN may need to send the NGAP message prior to the starting point of RAN PTW..

RAN informs CN about the RAN PTW length of a UE configured with extended eDRX cycles in RRC_INACTIVE.

# 3	Fallback mechanism

The signalling of Rel-17 and Rel-18 INACTIVE eDRX in a cell can be different for UEs supporting Release-17 (up to 10.24 s) and Release-18 (longer than 10.24s) and the fallback mechanism needs to be designed accordingly. RAN2 has previously agreed the following regarding the fallback:

“

- UEs configured with Rel-18 enhanced INACTIVE eDRX should apply Rel-18 enhanced INACTIVE eDRX if Rel-18 enhanced INACTIVE eDRX is allowed in the serving cell, regardless of whether Rel-17 INACTIVE eDRX is allowed in the serving cell.

- UEs configured with Rel-18 enhanced INACTIVE eDRX should apply INACTIVE DRX if both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX are not allowed in the serving cell.

- UE can support Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t support Rel-17 INACTIVE eDRX.

- A cell can allow Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t allow Rel-17 INACTIVE eDRX, but the cell must allow IDLE eDRX.

”

A UE can be configured with Rel-18 INACTIVE eDRX if it supports Rel-18 enhanced INACTIVE eDRX cycle and the cell allows Rel-18 INACTIVE eDRX UEs to camp on. Based on the above agreements it is not necessary for a UE to support Rel-17 INACTIVE eDRX to be configured with Rel-18 eDRX i.e., a UE that supports only Rel-18 INACTIVE eDRX provided that it is configured with IDLE eDRX values larger than the INACTIVE eDRX values.

Furthermore, as per earlier RAN2 agreements some invalid cases are identified as follows:

“

- Case 1: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle but not configured with the IDLE eDRX cycle.

- Case 2: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle longer than the IDLE eDRX cycle.

”

It is not possible for a UE to be configured with Rel-18 INACTIVE eDRX if it is not configured with an IDLE eDRX cycle larger than or equal to the value of INACTIVE eDRX cycle.

# 4	Conclusion

In this contribution, we continue the discussion on the relevant details for enhancements of eDRX. Based on the discussion in the previous sections we observe and propose the following:

Observation 1	With a possibility of such PTW configuration, a UE can be paged within the RAN PTW and CN PTW, within CN PTW when it is outside RAN PTW, or within RAN PTW when it is outside CN PTW.

Observation 2	It is not possible for a UE to be configured with Rel-18 INACTIVE eDRX if it is not configured with an IDLE eDRX cycle larger than or equal to the value of INACTIVE eDRX cycle.

Proposal 1	It should be possible for RAN to configure the length of RAN PTW with a value smaller, equal to, or larger than the length of CN PTW.

Proposal 2	RAN informs CN about the RAN PTW length of a UE configured with extended eDRX cycles in RRC_INACTIVE.

# References

RP-211574, “Revised WID on support of reduced capability NR devices”, Nokia, Ericsson, RAN#92e, Online, June 2021.

RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, RAN#98e, Online, December 2022.

TR 38.875 V17.0.0, “Study on support of reduced capability NR devices”, V17.0.0 (2021-03).

TS 38.300, “NR and NG-RAN Overall description; Stage-2”, V16.5.0 (2021-03).

TS 36.304, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”, V16.5.0 (2021-07).

TS 23.501, v16.8.0, “System architecture for the 5G system (5GS)”, 3GPP

TS 23.003, v16.7.0, “Digital cellular telecommunications system (Phase 2+) (GSM)” 3GPP

TS 38.304, “NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state” V16.5.0 (2021-06)

TS 38.331, “NR;Radio Resource Control (RRC) protocol specification” V16.5.0 (2021-06)

TS 38.213, “NR; Physical layer procedures for control” V16.6.0 (2021-06)