3GPP TSG-RAN WG2 #123	Tdoc R2-2308809

Toulouse, France, August 21st - 25th, 2023

Agenda Item:	7.22.2

Source:	Ericsson

Title:	LP-WUS/WUR for RRC Idle and Inactive

Document for:	Discussion, Decision

# 1	Introduction

For the ongoing RAN1 led SI ‘Study on low-power wake-up signal and receiver for NR’ [1] there are some aspects for RAN2 to study:

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

In RAN2#121bis-e it was agreed to collect SI TR input jointly in RAN2 and provide it to RAN1 [2][3]:

Aim to do every Q: Collect RAN2 text proposals in a single document during the following meeting(s) and send the document to RAN1 as the input to the TR 38.869.

In this contribution the RRC Idle/Inactive aspects of WUR will be discussed and a text proposal for the TR is provided.

# 2	Discussion

## 2.1	RRC state considerations

WUR performance in RRC Idle/Inactive is to be evaluated against a PEI baseline according to the SID, either using DRX with a cycle length of 1.28s or using eDRX, with a 20s or 61s cycle length, and 4 POs per PTW. The “traffic model” to be evaluated is a certain range of per UE paging probabilities RE = {1%, 0.1%, 0.01% or 0.001%}, determining the group paging probability as RG = 1 – (1 – RE)N , where N is the number of UEs sharing a WUS monitoring occasion (for the DRX case, a bit more complicated for the eDRX case).

For IDLE/INACTIVE, LP-WUR gains are evaluated compared to IDLE DRX and Rel-17 PEI baseline.

From our accompanying RAN1 paper on low-power WUS evaluations [4] it is seen that the energy consumption reduction using WUR can be over 90% compared to baseline and that it is the biggest for shorter duty-cycles. The gains compared to baseline becomes smaller with increasing duty-cycle length, and with increasing data activity, i.e., probability of being addressed such that the main receiver must be started.

The above concerns the reduction in energy consumption, but WUR latency should also be evaluated. Regarding the latency, it should be noted that the sleep state of the main receiver will dictate a lower bound for the DL latency. That is, with the main receiver in deep-sleep state the transition time is 400 ms (according to the evaluation assumptions agreed by RAN1) and, hence, the deep-sleep state is mainly relevant in Idle/Inactive.

## 2.2	Continuous WUR vs. Duty-cycled WUR

Related to the above discussion, RAN1 has agreed the following:

Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

That is, in general, WUR operation can either be duty-cycled, i.e., as for Rel-15/16 WUS for NB-IoT/LTE-M and Rel-17 PEI, with the WUR switching between an active and a sleep mode, or continuously monitoring the DL with WUR in an active mode. The main benefit of the ‘continuous-WUR’ is thought to be reduced DL latency. That is, the UE can be woken up at any time. However, as discussed above, the start-up time of the main receiver (400ms transition time from ultra-deep sleep and 20ms transition time for regular deep sleep) puts a lower bound on the achievable DL latency. Therefore, a duty-cycle considerably shorter than the main receiver transition time, e.g., 50ms when the main-receiver is in ultra-deep sleep with a 400ms transition time, only increases the device energy consumption without any noticeable improvement for the DL latency. Therefore, it may the difficult to motivate continuous-WUR in Idle/Inactive.

The main receiver transition time, 400 ms in ultra-deep sleep and 20 ms in deep sleep, is a lower bound for the achievable DL latency.

Continuous-WUR has insignificant latency improvement compared to duty-cycled WUR (for Idle sleep states with non-zero transition time).

Further, our initial evaluations for the energy consumption gain do not look very promising for Continuous-WUR (‘Always-on WUR’ in Figure 1). For these evaluations, a WUR decoding attempt is made every 1ms in both cases, and for duty-cycled WUR the receiver remains on for 20ms during wake-up, and the false alarm probability is Pfa=10-3. These evaluations were done using the regular deep sleep for the main receiver (1 power unit) and not the ultra-deep sleep state (0.015 power units), so more relevant for the comparison of Duty-cycled WUR and Continuous-WUR than for absolute WUR gains compared to baseline. Further, the active WUR power unit is 0.1, which is in the agreed evaluation range of [0.01, 0.05, 0.1, 0.2, 0.5, 1, 2, 4, 10, 20, 30], and the paging frequency is once per 100s.

Figure 1: WUR power saving gain compared to a DRX baseline, for Duty-cycled WUR and Continuous (Always-on) WUR.

First it can be seen that the WUR gain is the biggest for smaller DRX cycle lengths. This can be understood from the much larger contribution to the total consumed energy from the main receiver (MR). That is, for the DRX baseline the MR is started every DRX cycle and otherwise kept in the deep sleep state. So, when the DRX cycle is long the total energy is dominated by the time in between DRX wake-up, in which case the MR is in the deep sleep state both for baseline DRX and for WUR, and therefore the difference and additional WUR gain is small. If instead the DRX cycle is short, the MR must be woken up frequently to monitor for paging with the DRX baseline, which is energy consuming. Instead using WUR, the MR needs to be started only when there is paging for the UE, otherwise it can remain in the deep sleep state.

WUR UE energy consumption reduction gain compared to baseline is the biggest for short DRX cycles.

Continuous-WUR is independent of the DRX cycle length (hence a flat curve in Figure 1). The main problem with Continuous-WUR is that it, due to the continuous monitoring, it is very sensitive to false alarms, in which case the MR is unnecessarily started, which consumes a lot of energy. Therefore, the energy consumption for Continuous-WUR does not decrease with an increasing DRX cycle length in the same way as for the DRX baseline or the Duty-cycled WUR. As seen from Figure 1, due to this false-alarm problem the power saving of Continuous-WUR is always smaller than for Duty-cycled WUR, and even becomes smaller than the DRX baseline at longer DRX cycle lengths.

False-alarms are problematic for Continuous-WUR and cause energy consumption reduction to be smaller than expected compared to Duty-cycled WUR (and even smaller than for the DRX baseline for longer DRX cycles).

In addition, Continuous-WUR would likely have larger specification impact than Duty-cycled WUR. Duty-cycled WUR would require a similar addition to TS 38.304 as Rel-17 PEI (Section 7.2), or even reuse it. For Continuous-WUR, a UE in Camped Normally state should monitor PDCCH according to Section 7 on Paging:

### 5.2.5 Camped Normally state This state is applicable for RRC_IDLE and RRC_INACTIVE state. When camped normally, the UE shall perform the following tasks: - monitor the paging channel of the cell as specified in clause 7 according to information broadcast in SIB1; - monitor Short Messages transmitted with P-RNTI over DCI as specified in clause 6.5 in TS 38.331 [3]; - monitor relevant System Information as specified in TS 38.331 [3];

However, even though Section 7.1 states the UE “may” use DRX there is no alternative for continuous monitoring specified:

# 7 Paging ## 7.1 Discontinuous Reception for paging The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle.

Therefore, a new section and UE behaviour for continuous monitoring would have to be introduced. This may be straight forward, but could also lead to unexpected open issues e.g., related to configuring UEs to monitor different frequency resources to reduce the false paging rate (since UEs are no longer distributed over POs).

Continuous monitoring of paging is not supported in TS 38.304 currently, and Continuous-WUR may have larger RAN2 specification impact than Duty-cycled WUR (for which PEI text can be reused).

## 2.3	WUS payload

Regarding the payload carried by WUS, RAN1 has agreed the following and in principle all options ranging from carrying a common WUS sequence for all UEs (similar to Rel-15 WUS for LTE-M/NB-IoT) to carrying the full UE_ID are being considered (see the Options in the cropped agreement below):

Agreement For link-level simulation of LP-WUS, the following table is used as starting point, - FFS for other assumptions if any - Note: The assumptions are not intended to limit the scope of the study or the design. Table XX. Simulation assumptions for LP-WUS Attributes; Assumptions. Channel structure; - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence)..

Agreement - For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g. UE-group, -subgroup or -ID - FFS: cell information - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information - Other information candidates are not precluded - Study pros and cons of including above information to LP-WUS. - Note: the information may be explicitly or implicitly indicated.

As for the RAN1 impact, it is mainly the WUS coverage which will be affected by the size of the WUS payload . For example, full WUS coverage (see section below) in the cell will be difficult to achieve with a large WUS payload using the same amount of radio resources.

As for the RAN2 impact, the WUS payload can have an impact on the procedure triggered in the UE at WUS detection. That is, if there is a common WUS sequence for all UEs in the cell, the WUS would trigger all UEs sharing a paging occasion (PO) to turn on their main receiver and monitor paging according to the legacy procedure in the associated PO (see Figure 2). In this case, all UEs in the shared PO will be triggered, also those who are not paged, which is referred to as “false paging”.

Figure 2: UE procedure with group indication, or common sequence, in WUS.

To reduce the false paging, i.e., that a UE is unnecessarily woken up by paging intended for another UE, a WUS UE subgroup could be indicated using a few bits in the WUS payload or using more WUS sequences. For example, 3 bits would support 8 WUS UE groups and reduce false paging probability by 8. Such WUS UE groups were introduced in Rel-16 for NB-IoT/LTE-M WUS and supported already from the start for Rel-17 PEI. For devices supporting Rel-18 WUR, false paging reduces the potential WUR energy consumption gain since the main receiver would unnecessarily be started upon false paging. Note that the multi-UE “false paging” effect is not the same as “false alarm”, which is instead a RAN1 single-UE link-level effect for incorrectly detecting WUS when none has been transmitted. Therefore, the RAN2 agreement from RAN2#122 should be corrected as follows:

RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false paging alarms rate (depend on L1 capacity to carry payload or the number of WUSs supported).

Revise the agreement from RAN2#122 as follows: RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false paging alarms rate (depend on L1 capacity to carry payload or the number of WUSs supported).

False paging reduces the potential WUR power saving gain, and it is more important for Rel-18 LP-WUS/WUR than for PEI or WUS for NB-IoT/LTE-M to keep false paging rate low (since for the Rel-18 device the main receiver is unnecessarily started up).

Finally, if a full UE identifier could be included in the WUS payload the procedure triggered in the UE could be different. For example, as shown in Figure 3, if an individual UE can be addressed in the WUS itself, there is no need for the UE to trigger the legacy procedure of monitoring for paging in the associated PO to see if its paging record is included in the paging message.

Figure 3: UE procedure with full UE identifier in WUS

Therefore, the UE could, in this case, immediately trigger the random access procedure.

If full UE_ID is included in the WUS payload, monitoring of the legacy paging message in the PO can be omitted, i.e., the UE can trigger RA with no need to decode the paging message first.

The main motivation for WUR is UE energy consumption reduction, and this is achieved by allowing the main receiver to stay in an (ultra-)deep sleep state for extended periods of time. When the UE is paged it anyway must start up the main receiver (15000 units transition energy from ultra-deep sleep state) and there should therefore be no bigger gain associated with directly triggering RA instead of the legacy paging procedure. The main benefit of using the full UE_ID in WUS should instead be that false paging is reduced to zero. The main drawback is for course that the full UE_ID is considerably larger than a few bits for WUS UE group indication, e.g., 48 bits for 5G-S-TMSI, and it may not be feasible to have full WUS cell coverage with this solution (pending RAN1 evaluation, cf. Section 2.1).

If the full UE_ID is included in WUS payload, the false paging is reduced to zero, but full WUS cell coverage may not be possible due to the relatively large WUS payload.

## 2.4	Full or partial WUS coverage

One RAN1 aspect which will have very large impact on RAN2 procedures is if WUS can cover the entire cell (e.g., on par with legacy PDCCH coverage) or if there will only be partial coverage in the cell.

Figure 4: Illustration of A) full WUS coverage in cell vs. B) partial WUS coverage.

For full WUS cell coverage, as for Rel-15 WUS for NB-IoT/LTE-M or for Rel-17 PEI, there will be no additional impact. However, for partial coverage several open issues will be introduced which must be resolved, e.g., the following:

- When should the UE monitor legacy paging and when should it monitor WUS? (e.g., based on RSRP-threshold). I.e., procedures for when the UE moves in and out of WUS coverage in the cell.

- How should a gNB determine if it should transmit legacy paging or WUS to reach the UE?

- How to address a UE becoming unreachable in the DL if the UE incorrectly assumes that it is within WUR coverage (e.g., due to the large RSRP measurement inaccuracy of several dB in the UE).

A solution with partial WUS cell coverage has substantial impact on RAN2 specifications compared to a solution with full WUS cell coverage.

## 2.5	Mobility and RRM measurements

In RAN1#111 it was agreed to study ways to reduce the RRM contribution to the overall UE energy consumption to maximize the WUR gains:

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

In addition, the following agreement was reached in RAN1#112:

Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal

Serving cell measurements need to be performed most frequently by the UE and will therefore have the biggest impact on UE energy consumption. Intra- and inter-frequency neighbour cell measurements need only be performed when the UE’s serving cell is weak and the UE is searching for a new serving cell. That the UE constantly must perform inter-frequency neighbour cell measurements is therefore not the most representative scenario and it is the serving cell measurements that will have the biggest impact on UE energy consumption.

For evaluation of RRM measurement impact on UE energy consumption, serving cell measurements are considered.

Serving cell measurements are defined in TS 38.133 Clause 4.2.2.2, and an extract is given below:

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1.

SS-RSRP and SS-RSRQ are measured on SSS and in FR1 the Scaling Factor N1 is equal to 1. Therefore, a UE must perform serving cell measurements either every DRX cycle or every 2nd DRX cycle in FR1 (somewhat more relaxed in FR2).

In general, there are three alternative solutions for RRM measurements with LP-WUR operation (these options are also considered for evaluation by RAN1, see Appendix A1):

- WUR measurement on legacy SSB

- WUR measurement on a new WUR-specific reference signal (LP-SS)

- MR measurement on legacy SSB

Solution A is applicable for a WUR capable of receiving legacy SSS (i.e., a somewhat more capable WUR). Solution B requires the introduction of a new WUR-specific reference signal, i.e., the LP-SS which is captured in RAN1 agreements as seen in Appendix A1:

This requires the introduction of a new always-on broadcast signal, which would result in additional NW overhead and energy consumption. Avoiding new always-on signals is one of the key considerations from NW operation perspective.

Solution C means the main receiver is started-up periodically to perform the RRM measurements. As shown in Figure 5,  without RRM measurement relaxations (K=1) the MR must be started in every DRX cycle and there is very little gain from using WUR (the figure shows results for good coverage, i.e., 1 SSB is required for MR sync). Therefore, measurement relaxations are needed for LP-WUR to provide power savings gain compared to baseline with solution C.

Figure 5: Power saving achieved by WUR when main radio performs RRM measurements every K DRX cycle (duty-cycled WUR).

This is also in line with RAN1 observations from evaluations, i.e., to provide significant UE power saving gains either relaxations need to be substantial (K>16) or the WUR/LR must be used for RRM measurements:

For RRM with duty-cycled LP-WUS monitoring, the following observations are made with the assumption that - MR in ultra-deep sleep - Effective per UE paging arrival rate <=1% - LP-WUR duty cycle ratio <=2% - MR ramp-up time/transition energy option 1 (i.e., 400ms, 15000) - RRM relaxation is assumed for both serving and neighbouring cells Compared with i-DRX, LP-WUS operation with - No RRM relaxed - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([-301%~-569%]) - MR relaxed < 8 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([-10%~7%)) - 8 times<= MR relaxed <=16 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([31%~60%]) - RRM relaxed > 16 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([60~92%]) - RRM offload RRM to LR - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([76%~92%]) Note: The ‘Effective per UE paging arrival rate’ is defined as (without taking FAR into account) - Per UE paging probability RE if LP-WUS is per UE paging - Per group paging probability RG = 1 – (1 – RE)N, if LP-WUS is per group paging (N is the number of UEs in the group)

For NR no serving cell measurement relaxations have been introduced, but for NB-IoT and LTE-M relaxation of serving cell measurements of up to every 8th DRX cycle was introduced for stationary UEs (see parameter numDRX-CyclesRelaxed in TS 36.331). However, as seen from Figure 5 and the RAN1 observations above, not even that is enough to achieve larger power saving gains for WUR. In addition, the smaller WUR gain would only be applicable to the fraction of UEs which can fulfil the stationary requirement.

WUR power saving gain is significantly reduced by using the main receiver for RRM measurements.

As an alternative to using the main receiver or LP-SS, study RRM measurements by LP-WUR using existing OFDMA based signals (SSB).

Reusing SSB for RRM measurement purposes would have zero additional overhead from broadcast, unlike the new LP-SS. To limit the negative impact from additional LP-SS overhead, some companies in RAN1 have claimed that a ~1s periodicity would be sufficient for the LP-SS. In RAN1#112bis, the following LP-SS periodicities were agreement for evaluation: {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} (see Appendix A1). First, TS 38.133 states that UEs should filter over at least 2 samples per serving cell measurement: “The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements.”. This means an RRM measurement would take double the time of the LP-SS periodicity used. Further, as shown in our RAN1 contribution, WUR measuring on LP-SS (OOK-based measurement) may have worse RRM measurement accuracy and therefore averaging over more samples than 2 may be required to achieve the same measurement accuracy as in legacy. For example, if 4 samples are needed, with the LP-SS periodicities agreed for evaluation, a RRM measurement using LP-SS would take from 1.28s up to 40s.

Second, as shown above a UE must perform serving cell measurement once every or every 2nd DRX cycle. As seen in Figure 1, the WUR gain compared to DRX baseline is the biggest for shorter DRX cycles, i.e., shorter than 1s or even 0.5s. Such relatively short DRX cycles will therefore likely be what will be used for WUR in practice, but the agreed LP-SS periodicity agreed for evaluation cannot fulfil the existing TS 38.133 requirements of performing serving cell measurements every or every 2nd DRX cycle (see above) for such short DRX cycles. That is, with a 160ms DRX cycle, serving cell measurements would have to be done every 160ms or 320ms (in FR1), which is not possible even if the UE monitors all LP-SS occurrences since one measurement takes 2 samples and therefore 640ms to 20s.

LP-SS periodicity must be at least half of the shortest DRX cycle applied in a cell to fulfil the RAN4 serving cell measurement requirements in TS 38.133 (shorter than DRX/2 if measurement accuracy is worse for OOK-based signals).

The LP-SS periodicity range agreed for evaluation, from 320ms to 10240ms, may not be able to fulfil the RAN4 requirements in TS 38.133 on how often serving cell measurements should be performed by the UE.

We therefore propose the following:

Captured in SI TR that LP-SS periodicity must be at least half of the shortest DRX cycle applied in a cell to fulfil the RAN4 serving cell measurement requirements in TS 38.133 (shorter than DRX/2 if measurement accuracy is worse for OOK-based signals).

# 3	Text proposal

Based on the above discussion we propose the following text proposal for inclusion in the RAN2 input to TR 38.869.

Use the following text proposal as baseline for LP-WUS/WUR operation in Idle/Inactive for the RAN2 input to TR 38.869.

### 7.3.1 RRC_IDLE/INACTIVE mode #### 7.3.1.1 Idle mode procedures and RRC state In RRC_IDLE and RRC_INACTIVE states UE-specific configuration is not possible. Further, UEs shall perform idle/inactive mode procedures defined in TS 38.304 and TS 38.331, which specify how UEs shall monitor for paging, keep system information up to date, and perform cell selection and re-selection to ensure that UE is camping on the strongest cell. These procedures may need to be modified to accommodate operation using LP-WUS/WUR. To avoid unnecessary control signalling overhead, Idle/Inactive mode procedures should not rely on signalling between UE and gNB unless there is paging for the UE.. The same principle should apply for LP-WUS/WUR operation and it is therefore not feasible to signal e.g., any LP-WUS/WUR state change such as moving in and out of WUS coverage. In principle, a new state, RRC or other, for operation using LP-WUS/WUR could be introduced. A sub-state to RRC_IDLE like the one used for Mobile Initiated Communication Only (MICO), where both monitoring of the downlink and RRM measurements are stopped until there is a subsequent uplink transmission, is however not feasible for LP-WUS/WUR. This since the point with LP-WUS/WUR is to improve UE energy consumption while maintaining the downlink monitoring functionality, and this is not possible if either the UE stops monitoring in downlink or if the UE does not detect that it has moved into a new cell and therefore apply the incorrect configuration for downlink monitoring. According to TS 38.300 the functionality to be supported over Uu interface in RRC_IDLE and RRC_INACTIVE are PLMN selection, SI reception, cell re-selection mobility, and paging. Therefore, it is feasible to introduce LP-WUS/WUR as a power saving feature for RRC_IDLE and RRC_INACTIVE. #### 7.3.1.2 WUR mode and triggered procedure Paging reception in RRC_IDLE and RRC_INACTIVE is supported both with discontinuous reception (DRX), and from Rel-17 and Rel-18, respectively, with extended DRX (eDRX). LP-WUS/WUR could also be supported with or without duty-cycled operation, i.e., as ‘continuous WUR’ or as ‘duty-cycled WUR’. Since the LP-WUS/WUR feature is introduced to lower the UE energy consumption, it is for these two options feasible to introduce support for the WUR option which can introduce the greatest UE energy consumption reduction compared to baseline. If the gain is similar, the magnitude of the specification impact should be a determining factor for which WUR option to support. Currently there is no support for continuous monitoring of paging in Idle/Inactive and therefore ‘continuous WUR’ may have a larger specification impact to TS 38.304 than ‘duty-cycled WUR’ (for which PEI text can be reused). A potential benefit of ‘continuous WUR’ is shorter downlink latency. That is, since the UE monitors the downlink continuously the downlink latency could potentially be shorter. Due to the transition time to start up the main receiver however, WUR operation will always have worse latency performance than legacy, i.e., comparing to legacy continuous monitoring or DRX. ‘Continuous WUR’ could still provide shorter downlink latency compared to ‘duty-cycled WUR’, comparing at the same UE energy consumption reduction gain. This however depends on the UE procedure triggered by WUS and the WUS payload content and size. That is, if a unique UE identifier can be included in the WUS payload, e.g., the 48-bit 5G-S-TMSI or 24/40-bit I-RNTI, a UE need not monitor paging in the PO but can directly from the WUS reception determine that it is being paged and immediately trigger access establishment. For ‘continuous WUR’, the achievable downlink latency is in this case therefore determined by the PRACH resource periodicity, and if this periodicity is shorter than the periodicity for downlink monitoring applied for ‘duty-cycled WUR’, there could be latency gain proportional to this difference. However, including such a large payload in WUS can be challenging for WUS link performance and coverage. If a unique UE identifier cannot be included in the WUS payload, the UE cannot directly from the WUS reception determine that itself, and not another UE, is being paged and therefore WUS detection would have to trigger the legacy paging monitoring procedure. That is, if the UE detects WUS it would start up its main receiver to monitor legacy paging, i.e., the PDCCH scheduling of the paging message on PDSCH, and at first after finding its own PagingRecord in the paging message the UE could determine that it is being paged (similar to Rel-15 WUS for NB-IoT/LTE-M or Rel-17 PEI). In this case, the downlink latency will be determined by the periodicity of the legacy paging occasions (POs) and therefore ‘continuous WUR’ will not have a latency benefit compared to ‘duty-cycled WUR’. That is, even if WUS is immediately received using ‘continuous WUR’ the UE would anyway need to wait for the subsequent PO (i.e., same PO as if ‘duty-cycled WUR’ is used). In the case a unique UE identifier cannot be included in the WUS payload, the mapping of WUS occasion to PO must also be defined. In the simplest case, similar to Rel-17 PEI, a WUS monitoring occasion could be associated with each PO, and a occur with a certain time offset before the PO (which must be long enough to include the transition time to start up the main receiver). Alternatively, a WUS monitoring occasion could be configured to trigger the UE to monitor multiple POs, or even an entire paging time window (PTW) when using eDRX. Such mapping could increase the network flexibility and reduce signalling overhead when paging several UEs but could at the same time have a negative impact on UE energy consumption and the WUR gain since upon each WUS reception, false alarm, or false paging the UE would have to keep the main receiver on for a longer time. For reference, Rel-17 PEI supports mapping of up to 8 POs (configured the parameter po-NumPerPEI), and Rel-15 WUS and Rel-16 GWUS for NB-IoT/LTE-M supports mapping of up to 4 POs, but only when the UE is configured with eDRX. Further, if a unique UE identifier cannot be included in the WUS payload, either all UEs could be triggered to wake up to monitor paging in the associated PO, or just a subset of the UEs sharing the PO, so called WUS UE subgrouping. The latter can be used to reduce the negative impact from false paging, i.e., unnecessary energy consumption in the UE caused by paging intended for another UE. In the latter case, WUS UE subgrouping information corresponding to 2 bits or more would have to be included in the WUS payload or from the use of multiple WUS signals corresponding to different subgroups. For reference, both Rel-17 PEI and Rel-16 GWUS for NB-IoT/LTE-M support up 8 subgroups per PO (configured by the parameters subgroupsNumPerPO and GWUS-NumGroups, respectively). Both these features rely on DRX and UEs already being distributed over the different POs in the cell, and the UE subgrouping therefore refers to further subgrouping of the UEs already divided into the same PO. This would be the same for ‘duty-cycled WUR’ operation. However, for ‘continuous WUR’, false paging could apply to all UEs in the cell since all UEs are monitoring the downlink continuously. False paging would therefore be much more severe for ‘continuous WUR’ and a larger number of LP-WUS/WUR UE subgroups would be required to achieve the same false paging rate. A summary of the impact from carrying a unique UE identifier in the WUS payload or UE subgroup indication in the WUS is given in Table 1. Table 1: Impact from the inclusion of a unique UE_ID or UE subgroup information in the LP-WUS payload. Unique UE_ID in WUS payload:; UE subgroup indication in WUS:. Procedure triggered by WUS reception:; Random Access; Legacy monitoring of paging in PO. gNB transmission upon paging:; WUS; WUS and legacy paging (PDCCH+PDSCH).. False paging:; No false paging.; False paging among all UEs in the cell using ‘Continuous WUR’, and among all UEs sharing a PO using ‘duty-cycled WUR’. False paging is reduced by use of UE subgrouping and an increasing number of UE subgroups.. Latency impact:; DL latency determined by PRACH resource periodicity.; DL latency determined by DRX cycle length and PRACH periodicity.. #### 7.3.1.3 Full or partial LP-WUS coverage The Rel-17 PEI and Rel-15 WUS for NB-IoT/LTE-M procedure works for WUR if there is full LP-WUS coverage in the cell. However, if LP-WUS link performance is considerably worse than legacy physical channels (e.g., PDCCH or PUSCH) there would only be partial WUS coverage in the cell and the complexity of the solution will be considerably higher. That is, UE and gNB may need to keep track of when the UE moves in and out of LP-WUS coverage in the cell, this to be able to fall back to monitoring paging using the main receiver when the coverage using WUR is not sufficient. The UE could determine this from measured RSRP, e.g., compared to an RSRP threshold in the WUR configuration in system information, but measurements and trigger conditions would need to be defined both for using the WUR, for moving out of LP-WUS coverage, and for the main receiver, for moving into LP-WUS coverage. For the gNB, if a unique UE identifier (e.g., 48-bit 5G-S-TMSI or 24/40-bit I-RNTI) is not carried in the WUS payload, a gNB anyway always need to transmit both LP-WUS and the associated legacy PDCCH and PDSCH for the paging message. In this case, the gNB would in principle not need to consider whether the UE is within LP-WUS coverage in the cell or not, but in case it is not there would be some unnecessary control signalling overhead from the transmission of WUS. If instead a unique UE identifier is carried in the WUS payload, the gNB should either transmit LP-WUS or legacy paging depending on if the UE is within LP-WUS coverage or not (since the UE directly can trigger RA). However, without any signalling from the UE upon WUS coverage change, or from being in the cell or not (see above), this would have to be done blindly and would be left to the gNB implementation (similar to paging the UE in two different cells, i.e., up to trial and error). With partial coverage, the most severe error case would be if the UE from inaccurate measurements incorrectly concludes that it is in LP-WUS coverage when it is not, and therefore becomes unreachable in the downlink (the UE only monitors using WUR but is outside LP-WUS coverage). #### 7.3.1.4 Configuration Regarding the WUR configuration, both UE and gNB must have a common understanding of when WUR should be applied to avoid that UEs become unreachable in the downlink (see above). Rel-17 PEI and Rel-15 WUS for NB-IoT/LTE-M could be considered as baseline in which case both UE and gNB apply WUR monitoring for paging if both 1) the UE has been configured with WUS according to the radio paging capabilities included in the paging request from AMF to gNB, and 2) WUR has been configured in the cell according to system information broadcast. This would further determine the conditions for entering and leaving WUR operation as being tied to the monitoring of paging in a cell: A UE enables WUR monitoring of paging if it is configured with WUR and camp on a cell in RRC_IDLE or RRC_INACTIVE state in which WUR is configured, for example when re-selecting a cell in in which WUR is configured or being released from RRC_CONNECTED in a cell in which WUR is configured. Regarding the conditions leaving WUR operation, a UE would continue to apply WUR for monitoring of paging until it either is de-configured with WUR, the cell de-configures WUR according to system information broadcast, or the UE re-selects another cell in which WUR is not configured. Procedures in the cell not related to monitoring of paging, for example connection establishment and transfer to RRC_CONNECTED, would already be covered by existing procedures in TS 38.304 and the use of WUR would not apply to them (except for RRM measurements, see below). #### 7.3.1.5 System information update Reception of system information (SI) broadcast is another Idle mode procedure which must be supported with LP-WUS/WUR. For UE initiated acquisition of SI there is no need to involve the WUR. That is, when the UE initiates the acquisition of SI, for example when SI has become outdated or upon cell change, the UE anyway needs to start up the main receiver for the reception of the SI message(s). However, this is not the case for a gNB notification of SI update and it must be possible to notify also WUR UEs about an upcoming SI change in the cell. For reference, in Rel-16 GWUS for NB-IoT/LTE-M this is done by a subgroup common to all UEs (configured by the parameter commonSequence), whereas for Rel-17 PEI the gNB must transmit PEI to all subgroups separately and upon the reception of PEI UEs continue to, as in legacy operation, monitor for the systemInfoModification and systemInfoModification-eDRX in the Short Messages transmitted on PDCCH using P-RNTI. For LP-WUS/WUR, potential solutions are therefore the following: - Common UE subgroup in WUS: gNB can address all UEs in WUS, and UEs wake up to check the legacy systemInfoModification in Short Message using the main receiver. - SI change notification in WUS: gNB can indicate a system information change directly in the WUS payload. - No common indication in WUS: gNB need to address each UE subgroup individually with a separate WUS, and UEs wake up to check the legacy systemInfoModification in Short Message using the main receiver. For solution A, the common UE indication can be reused for anything where all UEs must be addressed, i.e., CMAS/ETWS and everything covered by the Short Message Service, also covering any future need (unlike solution B which is limited to WUR). For solution B, SI update notification is specifically covered in WUS, but for any other need to address all UEs the gNB would need to rely on the same solution as in solution C. The benefit of B compared to A is that UEs can directly acquire the updated SI without the need to monitor legacy paging and the Short Message, but since SI update is rare in practice this will likely not have noticeable impact on UE energy consumption. For solution C, the gNB would upon SI update need to address each UE subgroup with a separate WUS which increases the control signalling overhead, and for some configurations it may even not be possible to do so during one BCCH modification period. Note that this is for the case without a unique UE_ID in the WUS payload and using subgrouping. The case with a unique UE_ID in the WUS payload is not feasible for solution C since the gNB would have to individually address every UE in the entire network since it doesn’t know which Idle and Inactive UEs are currently in the cell. #### 7.3.1.6 Mobility measurements To ensure a UE in RRC_IDLE or RRC_INACTIVE is camping on the strongest cell it needs to perform mobility measurements as specified in TS 38.304. Mobility measurements include serving cell measurements and neighbour cell measurements, which can be intra-frequency or inter-frequency measurements. In general, the neighbour cell measurements however only need to be performed by the UE if the serving cell is not strong enough, i.e., when the intra-frequency cell re-selection criterion (Srxlev > SIntraSearchP and Squal > SIntraSearchQ) or the inter-frequency cell re-selection criterion (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) is not fulfilled. For the majority of UEs, and in a well deployed network without coverage holes, it is therefore the serving cell measurements that need to be performed the most frequently, and therefore they will also have the biggest impact on UE energy consumption. For a UE operating with WUR, infrequently starting up the main receiver (MR) for neighbour cell measurements is not expected to have any significant impact on UE energy consumption. However, using the MR for serving cell measurements has a significant impact on UE energy consumption and reduce the WUR gain according to the observations from the study evaluation results. This since according to Clause 4.2.2.2 in TS 38.133, serving cell measurements must be performed every or every second DRX cycle in FR1 (and somewhat more relaxed in FR2). Starting the main receiver for RRM measurements every DRX cycle removes a large part of the UE energy consumption reductions from WUR. There are three options for performing serving cell measurements using WUR, and the WUR UE energy consumption reduction will depend on which option is adopted: - WUR measurement on legacy SSB - WUR measurement on a new WUR-specific reference signal (LP-SS) - MR measurement on legacy SSB Using WUR for RRM measurements (Options A and B) has a considerable larger UE power saving compared to using the MR for RRM measurements, [-301%~-569%] compared to [76%~92%]. For option A, a somewhat more advanced WUR is required which is capable of legacy SSS reception, since SS-RSRP and SS-RSRQ are used for serving cell measurements (see TS 38.133). For option B, a new broadcast signal, here referred to as the low-power sync-signal (LP-SS), must constantly be transmitted in cell which support LP-WUS/WUR, which increases the control signalling overhead, increases the network implementation complexity for WUR, and makes the NR signalling less lean. For option C, the WUR gains would be insignificant without a considerable amount of serving cell measurement relaxations. Only with a RRM relaxations to more than 16 times the WUR gain ([60~92%]) start to be similar to using WUR for RRM measurements. Support for serving cell measurement relaxations has not been introduced in NR, but for reference NB-IoT and LTE-M support serving cell measurements relaxations of up to every 8th DRX cycle (see parameter numDRX-CyclesRelaxed in TS 36.331). For WUR even such large relaxation would still reduce the WUR gain significantly ([31%~60%]), and the serving cell relaxation would only apply to UEs fulfilling a ‘stationary’ relaxed measurement criterion, which further limits the WUR gain to only a fraction of all UEs. For the LP-SS the additional overhead will depend on the broadcast periodicity of the LP-SS. According to TS 38.133 “The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2“. If the UE is required to perform serving cell measurements every DRX cycle (according to TS 38.133 the UE must perform serving cell measurements every or every 2nd DRX cycle in FR1), which is likely the case for at least one UE in the cell, and 2 samples are required for each measurement, the LP-SS periodicity must be at least half of the DRX cycle length. In practice this means that to be compliant with TS 38.133 and current RAN4 requirements, the LP-SS periodicity must half of the shortest DRX cycle in use in the cell. The largest WUR UE power saving gains are achieved for shorter DRX cycles compared to baseline, i.e., shorter than around 1s. In addition, the RRM measurement accuracy may be worse for OOK-based measurements, which is a likely implementation of LP-SS. If so, more than 2 samples of LP-SS may be required for a RRM measurement to achieve the same accuracy as a measurement of legacy SSS, which means the LP-SS periodicity must be even shorter than half of the shortest DRX cycle length applied in the cell.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The main receiver transition time, 400 ms in ultra-deep sleep and 20 ms in deep sleep, is a lower bound for the achievable DL latency.

Observation 2	Continuous-WUR has insignificant latency improvement compared to duty-cycled WUR (for Idle sleep states with non-zero transition time).

Observation 3	WUR UE energy consumption reduction gain compared to baseline is the biggest for short DRX cycles.

Observation 4	False-alarms are problematic for Continuous-WUR and cause energy consumption reduction to be smaller than expected compared to Duty-cycled WUR (and even smaller than for the DRX baseline for longer DRX cycles).

Observation 5	Continuous monitoring of paging is not supported in TS 38.304 currently, and Continuous-WUR may have larger RAN2 specification impact than Duty-cycled WUR (for which PEI text can be reused).

Observation 6	False paging reduces the potential WUR power saving gain, and it is more important for Rel-18 LP-WUS/WUR than for PEI or WUS for NB-IoT/LTE-M to keep false paging rate low (since for the Rel-18 device the main receiver is unnecessarily started up).

Observation 7	If full UE_ID is included in the WUS payload, monitoring of the legacy paging message in the PO can be omitted, i.e., the UE can trigger RA with no need to decode the paging message first.

Observation 8	If the full UE_ID is included in WUS payload, the false paging is reduced to zero, but full WUS cell coverage may not be possible due to the relatively large WUS payload.

Observation 9	A solution with partial WUS cell coverage has substantial impact on RAN2 specifications compared to a solution with full WUS cell coverage.

Observation 10	WUR power saving gain is significantly reduced by using the main receiver for RRM measurements.

Observation 11	LP-SS periodicity must be at least half of the shortest DRX cycle applied in a cell to fulfil the RAN4 serving cell measurement requirements in TS 38.133 (shorter than DRX/2 if measurement accuracy is worse for OOK-based signals).

Observation 12	The LP-SS periodicity range agreed for evaluation, from 320ms to 10240ms, may not be able to fulfil the RAN4 requirements in TS 38.133 on how often serving cell measurements should be performed by the UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For IDLE/INACTIVE, LP-WUR gains are evaluated compared to IDLE DRX and Rel-17 PEI baseline.

Proposal 2	Revise the agreement from RAN2#122 as follows: RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false paging alarms rate (depend on L1 capacity to carry payload or the number of WUSs supported).

Proposal 3	For evaluation of RRM measurement impact on UE energy consumption, serving cell measurements are considered.

Proposal 4	As an alternative to using the main receiver or LP-SS, study RRM measurements by LP-WUR using existing OFDMA based signals (SSB).

Proposal 5	Captured in SI TR that LP-SS periodicity must be at least half of the shortest DRX cycle applied in a cell to fulfil the RAN4 serving cell measurement requirements in TS 38.133 (shorter than DRX/2 if measurement accuracy is worse for OOK-based signals).

Proposal 6	Use the following text proposal as baseline for LP-WUS/WUR operation in Idle/Inactive for the RAN2 input to TR 38.869.

# References

RP-222644, “Revised SID: Study on low-power wake-up signal and receiver for NR”, RAN#97e, September 12-16, 2022.

R1-2305954, “Draft TR 38.869 v020: Study on low-power wake up signal and receiver for NR”, RAN1#113, May 2023.

R2-2303462, “Update of TR 38.869 for LP-WUS WUR”, vivo (Rapporteur), 3GPP RAN2#121bis-e, April 2023.

R1-2305575, “Low power WUS evaluations”, Ericsson, RAN1#113, Incheon Korea, May 2023.

# Appendix

## A1	RAN1 RRM & LP-SS agreements

- The following was agreed in RAN1#112bis:

Agreement The period of synchronization signal that LP-WUR used for at least power evaluation can be - Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR - For evaluations assuming LP-SS - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded

- The following was agreed in RAN1#113:

Agreement For reference setting for further study on LP-SS performance and resource overhead (including sync and/or measurement), companies to report the following used in their evaluations - the number of slots or symbols per period - the periodicty - the functionality of the LP-SS

Agreement

Observations:

For RRM with duty-cycled LP-WUS monitoring, the following observations are made with the assumption that - MR in ultra-deep sleep - Effective per UE paging arrival rate <=1% - LP-WUR duty cycle ratio <=2% - MR ramp-up time/transition energy option 1 (i.e., 400ms, 15000) - RRM relaxation is assumed for both serving and neighbouring cells Compared with i-DRX, LP-WUS operation with - No RRM relaxed - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([-301%~-569%]) - MR relaxed < 8 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([-10%~7%)) - 8 times<= MR relaxed <=16 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([31%~60%]) - RRM relaxed > 16 times - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([60~92%]) - RRM offload RRM to LR - Compared with i-DRX with and without PEI, LP-WUS provide mean power saving gain ([76%~92%]) Note: The ‘Effective per UE paging arrival rate’ is defined as (without taking FAR into account) - Per UE paging probability RE if LP-WUS is per UE paging - Per group paging probability RG = 1 – (1 – RE)N, if LP-WUS is per group paging (N is the number of UEs in the group)

Agreement For the LP WUR architectures analysis, in addition to LP-WUS detection, consider the following functions when necessary: - Synchronization signal processing and time/frequency synchronization for LP-WUR - RRM measurement at least for the serving cell

Agreement - For at least RRM serving cell measurement performed by LP-WUR based on reference signal(s), RAN1 identified at least the following metrics for further study and evaluation (including feasibility, complexity, power consumption, etc) - LP-RSSI or Energy detection: linear average of total received power over a RSSI resource. - FFS RSSI resource. - LP-RSRP: linear average of received power of resource of reference signal(s) or signal(s) parts. - FFS resource of reference signal(s) or signal(s) parts - LP-SINR = LP-RSRP/(power of interference and noise) - FFS how to define “power of interference and noise” - LP-RSRQ= [N x] LP-RSRP/LP-RSSI, where N is the factor of resource size difference for evaluation LP-RSRP and LP-RSSI. - Accounting AGC accuracy, ADC of at least 4 bits is required. - Note: Reference signal for performing measurements can be e.g. SSB (PSS/SSS/PBCH DMRS), LP-WUS-waveform sequence, LP-SS Note: The definition of metrics could be further refined based on future study

Agreement The following observations are to be captured in the TR - At least for LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial for the following functionality. - RRM measurements by LP-WUR, if supported - at least coarse time synchronization of LP-WUR. - at least coarse frequency synchronization of LP-WUR. - Additional periodic LP-SS system overhead depends on LP-SS periodicity, system BW, # of beams, and resource required to fulfil the target functionality, etc. Periodic signal if used for coarse synchronization may reduce overhead of signal preceding LP-WUS, if any. LP-SS can be designed to be common among UE groups (cell-specific) and such further reduce system overhead. - For LP-WUR that can receive existing PSS/SSS potentially assisted by PBCH DMRS/TRS for synchronization, existing PSS/SSS potentially assisted by PBCH DMRS/TRS may be used for above functionality. - Periodic LP-SS coverage should be equal or better than that of LP-WUS. - For fine time and frequency synchronization, a signal (e.g. preamble) preceding or part of LP-WUS may be used. Agreement - For Idle/Inactive mode, study offloading of RRM measurements of serving cell to LP-WUR under certain conditions, if any, and relaxation of serving/neighboring cell RRM measurements in MR considering - Periodic reference signal(s) is/are used for LR measurements. - FFS: reference signal(s) to measure, e.g. PSS/SSS/PBCH DMRS, LP-WUS waveform sequence, LP-SS - FFS: periodicity, content - MR performs measurements - Alt2: with relaxed periodicity if RRM measurement in MR is relaxed. - FFS: Condition for relaxation if any - Can apply for both neighboring and serving cell - Alt3: only when reference signal(s) based measurements by LP-WUR satisfy certain condition(s), e.g. are below threshold. - FFS threshold. - Above MR measurement under certain conditions can apply for both neighboring and serving cell - Potentially with relaxation methods for MR neighboring cell measurement - Other alternatives are not precluded - FFS: Feasibility of RRM measurements of neighbor cells by LP-WUR

## A2	RAN1 agreements on WUR activation & deactivation

- The following was agreed in RAN1#113:

Agreement - For Idle/Inactive mode, following options for activation and deactivation of LP-WUS monitoring by LP-WUR for a UE can be considered for study - Alt 1a: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS WUS monitoring is up to UE implementation. - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 1b: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS monitoring is based on preconfigured criteria - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 2: - activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling. - Paging misdetection performance shall not be impacted.

WUS and legacy paging (PDCCH+PDSCH) in partial LP-WUS coverage.