3GPP TSG-RAN WG2 Meeting #123						  R2-23xxxxx

Online, 21st August – 25th August, 2023

Agenda item	: 7.22.2

Source		: Samsung

Title		: Impact on UE IDLE/INACTIVE Procedures to support LP-WUR

Document for	: Discussion & Decision

# 1	Introduction

- RAN2 expect that different coverage LR/MR may have RAN2 impact, e.g. UE need to stop using LP WUS when moving out of LR coverage, other aspects FFS. What to cover (if anything) in TS 38.304 is FFS.

- For UE in RRC_IDLE/RRC_INACTIVE state, it is FFS to what extent the network is or need to be aware of which receiver the UE uses MR/LR or both (for paging reception etc). A potential drawback of not knowing could be increased LP WUS load, a potential drawback of awareness is increased signalling.

- RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. 
(Other control methods not precluded)

- RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false alarms rate (depend on L1 capacity to carry payload).

In release 18, proposed study item for low-power Wake-up Signal and Receiver for NR [1], the following RAN2 Agreements were made:

This contribution discusses aspects related to impact to Layer 2 procedures to support low-power wake-up receiver, and in particular the impacts on IDLE/INACTIVE mode operations.

# 2	Discussion

The UE is IDLE/INACTIVE mode performs certain operations such as monitoring for Paging message (both for RAN/CN/Group paging), short message indication, neighbor cell measurement for mobility, receive system information. Few of the issues to handle at layer 2 to support use of LP-WUS in UE are discussed in this paper

## 2.1. Impact due to different coverage areas of LP-WUS and Paging Channel:

RAN1 is currently studying the coverage areas of LP-WUS and other reference NR channels. In the case where the coverage areas of LP-WUS is smaller than the paging channel, the UE needs to handle the situations where the UE is reachable via paging but not by LP-WUS by the cell. There needs to be conditions on when to use LR for monitoring wake-up indication (WI).

The network indicates the support of LP-WUS to the UE as part of System Information message and/or dedicated signaling. The UE can uses the LR for monitoring the WI if the current cell indicates the support of LP-WUS and has a valid LP-WUS configuration. If the cell indicates that it no longer supports LP-WUS or UE does not hold a valid LP-WUS configuration, the UE stops using LR for receiving the WI.

The network can configure the UE with thresholds (signal strength/quality of LP-WUS/LP-SS etc) for determining if the UE is out of coverage of LP-WUS and the UE can determine if it is within the coverage area of LP-WUS based on the measurements of received LP-WUS and/or Low Power Synchronization Signal (LP-SS) if available. If the UE is out of LP-WUS coverage area, it relies on the Main Radio (MR) for receiving Paging early indication and/or paging message.

Proposal 1: UE can be configured with threshold conditions for determining if it is in coverage area of LP-WUS

Proposal 2: UE uses LR to perform measurement on LP-WUS and/or LP-SS to determine if UE is in coverage area of LP-WUS

## 2.2. Awareness at network if UE is using LR for receiving Wake up indication:

The information of whether a UE is using LR for monitoring LP-WUS can aid the network in scheduling LP-WUS and PEI/Paging resources. The UEs’ usage of LR for receiving the wake up indication can be based on whether the UE is in the coverage area of LP-WUS or not and UE can move in and out of coverage area of LP-WUS. If the UE were to report every time it starts or stops using the LR, this can lead to high signaling load and also increased power consumption at UE as it needs to transition out of IDLE/INACTIVE state to report the change in usage of LR.

Proposal 3: UE does not report to network if UE starts/stops using LR for monitoring LP-WUS

## 2.3. Action upon receiving wake up indication in LP-WUS

## The UE monitors for wake up indication carried as part of LP-WUS and upon receiving the WI intended for this UE or WI intended for the subgroup the UE belongs to, the LR indicates to the MR to monitor for paging in the next valid Paging occasion (PO).

Proposal 4: UE monitors next valid PO upon reception of LP-WUS with wake up indication for the UE or WI for the subgroup the UE belongs to.

The network can configure a minimum gap between reception of WUS and next valid PO. The network configures the minimum gap based on UE reported ramp up time of Main Radio.

Proposal 5: Network configures a minimum gap between reception of LP-WUS with WI and the next valid PO

Proposal 6: UE reports the gap requirement between WUS reception and associated PO monitoring based on the ramp up time of MR

# 3	Conclusion

In section 2, the following proposals are made,

Proposal 1: UE can be configured with threshold conditions for determining if it is in coverage area of LP-WUS

Proposal 2: UE uses LR to perform measurement on LP-WUS and/or LP-SS to determine if UE is in coverage area of LP-WUS

Proposal 3: UE does not report to network if UE starts/stops using LR for monitoring LP-WUS

Proposal 4: UE monitors next valid PO upon reception of LP-WUS with wake up indication for the UE or WI for the subgroup the UE belongs to.

Proposal 5: Network configures a minimum gap between reception of LP-WUS with WI and the next valid PO

Proposal 6: UE reports the gap requirement between WUS reception and associated PO monitoring based on the ramp up time of MR

# 4	Reference

[1] 3GPP TR 38.869 v0.1.0 Study on low-power wake up signal and receiver for NR