3GPP TSG-RAN WG2 Meeting #123                 	R2-2308833

Toulouse, France, 21th – 25th August, 2023

Agenda Item:	7.8.2

Source: 	vivo

Title:         	Discussion on combination event for NR UAV

Document for: 	Discussion and Decision

- Introduction

In RAN2#122 meeting , RAN2 made the following agreements for measurement reporting enhancement for NR UAV [1]:

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

This contribution further discusses the open issues for combination event for NR UAV.

- Discussion

In RAN2#122 meeting, the below agreement was made:

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

Whether to support other event Ax + event H1/H2

The combination of event A4 and event H1/H2 has been agreed and its motivation is for vertical mobility improvement and can reduce the signaling overhead of measurement report. The similar motivation can also apply to other combination of event Ax and event H1/H2. The network does not need to configure event Ax and event H1/H2 separately. Therefore, we support other event Ax can be combined with event H1/H2.

Proposal 1：Event A3/A5 can be combined with Event H1/H2.

For the combination event, the network can configure a timeToTrigger applying to event Ax and event H1/H2. If the entering condition for this combination event is satisfied, since we have agreed to introduce new event, it is reasonable and straightforward that when both event Ax and event H1/H2 are fulfilled during TimeToTrigger, the UE can trigger the measurement reporting procedure.

Proposal 2: The network can configure a timeToTrigger for combination event. When both event Ax and event H1/H2 are fulfilled during TimeToTrigger, the UE triggers the measurement reporting.

For event A3, A4 and A5, measurement report can be triggered when leaving condition is fulfilled. To combine the leaving conditions, we still consider the following three options:

- Option 1: Either of the event Ax and event H1/H2 satisfies the leaving condition.

- Option 2: Both event Ax and event H1/H2 satisfies the leaving condition.

- Option 3: One of the specific event Ax and event H1/H2 satisfies the leaving condition.

Although option 1 may trigger the measurement reporting frequently, it can give more information for interference management of both horizontal and vertical directions. For option 2 and option 3, it can reduce the measurement report but may impact handover performance.

Proposal 3: The UE can trigger the measurement reporting procedure when either of the event Ax and event H1/H2 satisfies the leaving condition.

The ping-pong issue may exist in both event Ax or event H1/H2. Therefore, to prevent the ping-pong issue, for event Ax or event H1/H2, hysteresis should be considered.

Proposal 4: The network can configure two hysteresis for event Ax and event H1/H2 of combination event.

For event A3, A4 and A5 in LTE, if one cell fulfills the triggering condition, it will be added to the cellsTriggeredList. When the number of cells in cellsTriggeredList is larger than or equal to numberOfTriggeringCells, it will initiate the measurement reporting procedure. The similar procedure can also apply to combination event. For each combination event, the network can configure a corresponding numberOfTriggeringCells.

For combination event, since the event Ax is linked to the corresponding height region of event H1/H2, if the UE flies out of a specific height region of event H1/H2, the cellsTriggeredList should not be updated. In other words, the UE only update cellsTriggeredList based on the event Ax and the UE is in the corresponding height region.

Proposal 5: The network can configure a corresponding numberOfTriggeringCells for combination event.

Proposal 6: For combination event, the UAV UE only update cellsTriggeredList based on the entry/leaving of Ax event when the UAV is in the corresponding height range.

- Conclusion

In this paper, the following proposal are given:

Proposal 1：Event A3, A4 and A5 can be considered to combined with Event H1/H2.

Proposal 2: The entering condition for the combination event can be satisfied when both entering condition of event Ax and event H1/H2 are fulfilled.

Proposal 3: The network can configure a timeToTrigger for combination event. When the entering condition of the combination is satisfied and both event Ax and event H1/H2 are fulfilled during TimeToTrigger, the UE can trigger the measurement reporting procedure.

Proposal 4: The UE can trigger the measurement reporting procedure when either of the event Ax and event H1/H2 satisfies the leaving condition.

Proposal 5: The network can configure two hysteresis for event Ax and event H1/H2 of combination event.

Proposal 6: The network can configure a corresponding numberOfTriggeringCells for combination event.

Proposal 7: For combination event, if the event H1/H2 satisfies the leaving condition, the UE should empty the cellsTriggeredList. if the event Ax satisfies the leaving condition, the UE should remove the concerned cell in the cellsTriggeredList

- References

[1] R2-2306543, Report from Session for on NES, UAV Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning.