# 3GPP TSG RAN2#123  	                                                 R2-2308836

# Toulouse, France, Aug 21st – 25th, 2023

Agenda item:	7.16.2.1

Title:	Further Discussion on Functionality/Model Id based LCM

Source:	ZTE Corporation, Sanechips

Document for:	Discussion and Decision

Introduction

In general aspect of RAN1 discussion, both the functionality based LCM and model ID based LCM have been comprehensively discussed. Some initial agreements in RAN1 are achieved and captured in the current TR draft [1]. However, there is lack of discussion of some LCM functionalities in RAN2 so far as now, such as, functionality/model identification, UE capability. Considering the functionality/model identification is coupling with the UE capability reporting, in this contribution, we try to discuss these two LCM functionalities and share our views on the corresponding specification impact.

Discussion

Model Id/Functionality based LCM

AI based functionality LCM

Regarding the AI/ML functionality based LCM, there are following description in the draft TR:

For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM will be studied. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. Editor’s note: RAN2 should address and study impact on RRC protocol, including UE capability reporting and other related signalling. After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities are studied. Editor’s note: RAN2 to discuss in this section technical inputs related to reporting updates to the applicability of functionalities.

According to above TR, the information can be obtained as observation 1:

According to the description in the TR, the AI/ML functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where the configuration(s) is(are) supported based on conditions indicated by UE capability.

In general, in our understanding of observation 1, ‘condition’ is elementary unit for each AI based functionality, one or more conditions can be associated with an AI based functionality, NW should configure the RRC configuration on the basis of these conditions from which the AI based functionality can work well. For example:

For an AI based functionality of AI based spatial beam prediction, it may include: condition #1: the number of beams in set B; condition #2: the number of beams in set A; Condition #3 is Reference signal of the set A is SSB;… etc; NW configured the RRC configuration of the AI based functionality on the basis of those conditions.

Fig.1: One model of the AI based functionality

For UE sided model and UE part of two sided model regarding the functionality based LCM，RAN2 assumes one AI based functionality is associated with a combination of multiple conditions.

According to above TR, some other information can be obtained as observation 2:

For UE sided model(s) and UE part of two sided model(s) within the AI based functionality , The UE capability reporting is taken as a start point for AI based functionality identification at NW side.

According to observation 2, the UE capability reporting is indeed a natural alternative to report the conditions for NW to configure the RRC configuration of AI based functionality. There are two options for UE to report conditions to NW via UE capability reporting :

-  Option 1: All conditions regarding an AI based feature are reported to NW via UE capability reporting.

-  Option 2: Only condition combination for each AI based function regarding an AI based feature are reported to NW via UE capability reporting

In option 1, all the conditions regrading an AI based feature (e.g. AI based spatial beam management) are reported via UE capability reporting, the NW make a decision to combine them into a set to compose an AI based functionality according to the current scenario, wireless environment, etc, and then configure the corresponding RRC configuration of the AI based functionality to UE. Consider the combination performed by NW is according to the random scenario, UE should support all the possibilities of the combinations , it is suspected whether the current UE can afford such a larger number of AI based functionalities/models .

Fig.2: The illustration of the condition based UE capability reporting (option 1)

In option 2, UE just report the supported AI based functionalities with a number of combinations of conditions as in figure-3. The option 2 is more straightforward since UE directly indicate the NW the supported AI based functionality. The concern of this option is the lack of flexibility. As we all know, the UE capability is static for a long time if it has been requested by gNB and stored in CN, for each time of accessing the NW, gNB firstly retrieve the UE capability from CN rather than UE, in this sense, the update of AI based functionalities may not be renewed timely in the UE capability. As a result, NW may have no idea about the update of AI based functionalities for a long time due to the retrieving of obsolete UE capability from CN.

Fig.3 The illustration of the AI based functionality based UE capability reporting

According to above analysis, it can be seen the option 1 and option 2 goes to two extremely opposite way, the option 1 can achieve the highest flexibility but with a big challenge to the UE device ability. The option 2 can be easily realized in UE but the lack of flexibility and static property of UE capability may cause the update of AI based functionality not being renewed timely.

Only using UE capability reporting to report the conditions for AI based feature may require the high UE ability of storing a large number of AI models for AI based functionalities, and the UE capability reporting may cause the update of AI based functionalities not been renewed timely in the UE capability.

According to observation 3, only UE capability reporting cannot fulfill the requirement of condition reporting for AI based functionality (e.g. 1: update information can be forwarded to NW as soon as possible  2: No need to store a large number of AI models for all the possibilities of combinations of condition), in this sense, it is suggested to introduce an additional procedure/reporting as the second step which is subsequent to UE capability reporting as the first step to inform the current AI based functionality supported by UE to NW.

-  Option 3: Using two-step condition reporting to NW.

In this option, there maybe a lot of realizations, some examples are given out for understanding:

- Example 1: In the first step, UE report a part of conditions regarding an AI based feature to NW by UE capability reporting; In the second step, UE report the rest of conditions regarding the AI based featuer to NW with a real-timer reporting framework.

- Example 2: In the first step, UE report all conditions regarding an AI based feature to NW by UE capability reporting; In the second step, UE report several AI based functionalities in the form of combinations of conditions with a real-time reporting framework.

- Example 3:…

In example 1, NW will configure the RRC configurations of AI based functionalities on the basis conditions reported by UE capability plus the conditions reported by a real-time reporting framework;

In example 2, NW will configure the RRC configuration of AI based functionalities on the basis of conditions combination reported by a real-time reporting framework. Some companies may ask why UE not directly report the combinations of conditions with a real-time reporting framework, that is because UE only need report the condition index instead of the condition contents result due to the conditions contents have been already stored in the CN which can save the signaling overhead of reporting.

We understand, at this stage, it is not easy for RAN2 to determine which example is better since the conditions for each sub-use case is not crystal clear. Above examples are just exhibited for getting the ball rolling. All in all, the sprite of the option 3 is to use two-step UE ‘capability’ reporting to replace the legacy UE capability reporting, that is, the first step, UE need to use UE capability reporting to report information regarding conditions to NW, the second step, UE use a real-time reporting framework (e.g. UAI) to report some other information regarding conditions to NW. Then NW is able to identify the AI based functionality the UE currently support by taking the combined information in step 1 and step 2 into account.

For UE sided model and UE part of two sided model regarding the functionality indentification, RAN2 assume two-step conditions reporting is applied, whereas UE capability reporting is considered as the first step, one real-time report framework (e.g. UAI) is considered as the second step.

Model Id based LCM

Regarding the AI/ML functionality based LCM, there are following description in the draft TR:

For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. After model identification, necessity, mechanisms, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models are studied. Editor’s note: RAN2 to discuss in this section technical inputs related to reporting updates to the applicability of models.

In the first paragraph of above, the yellow highlighted wording describes that, for model identification, one model is associated with a specific condition/configurations associated with UE capability and additional conditions (e.g. site, scenarios, datasets)

Regarding the model identification , RAN1 understands that the model identification is a procedure that NW have a same understanding on a UE sided model by the condition/configuration and additional condition (e.g. site, scenarios, datasets) received from UE side.

However, in the previous discussion in RAN2, the global model Id is introduced, for how to assign a global model Id to a model, the following agreements are achieved:

- Model ID can be used to identify model or models for the following LCM purposes:

  - model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step).

  - (e.g. for so called “model ID based LCM”)

- How to achieve globality of the Model ID is FFS.

  - Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point:

  - Direction1: Pre-defined/hard-coded global unique model ID

  - Direction3: Assigned global unique model ID via specific ID management node.

  - Note: Other global unique model ID definition is not precluded.

  - Model ID structure, if any, is FFS

RAN2 assumes that model Id is a globally unique which is assigned to one AI model by either offline engineering (e.g. pre-defined and/or hardcoded) or specific Id management node.

If we see the observation 4 and observation 5 together, one rational deduction is obtained that the target of the model identification is to assign an identified model with a global model Id, we need to confirm it in RAN2 If this is right understanding.

For UE sided model and UE part of two sided model regarding the model Id based identification, RAN2 assumes the model identification is a procedure that targets to assign a global model Id to the identified Model.

According to the two directions of model Id assignment, it is assumed that the model identification also have two directions, one is offline model identification where one AI model is assigned with a model Id by offline testing between vendors, the other one is online model identification that is a procedure involving a specific Id management node and UE.

For UE sided model and UE part of two sided model regarding the model Id based identification, RAN2 assumes the model identification can be via either offline testing or an online procedure involving UE and a specific Id management.

For UE sided model and UE part of two sided model regarding the model Id based LCM，RAN2 assumes offline model identification have no any RAN2 and RAN1 specification impact.

Regarding the model identification with online procedure, we do not think the UE capability reporting framework can afford such online procedure, the reason is shown as below:

- UE capability reporting is a procedure that is triggered by NW while the model identification procedure is intuitively triggered by UE itself.

- UE capability reporting is a procedure that is terminated at gNB just stored in CN (e.g. transparent to CN) while gNB have no ability to guarantee to allocate a globally unique model Id.

In this sense, we understanding the online identification is a procedure that may involve UE, and CN which must have SA impact. According to the SI, SA does not get involved, so we just make an assumption here for online

For UE sided model and UE part of two sided model regarding the model Id based LCM, RAN2 assume the online model identification is a procedure involving UE and Id management node which may have SA impact, which can be discussed in WI stage. .

Regarding the update reporting as the following EN:

Editor’s note: RAN2 to discuss in this section technical inputs related to reporting updates to the applicability of models.

In RAN2 discussion, we have the following agreements:

- RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified.

One intention of global Id is to identify the historical version of one model, that means, the update of a model will be allocated a new global model Id rather than keeping the original one. With this logical, the update reporting of a model is the model identification procedure mentioned above.

Regarding the update of the UE sided/UE part model regarding the model Id based LCM, RAN 2 assumes, the reporting updates is to preform the model identification procedure to obtain a new global model Id.

Conclusion

In this contribution, we provide our further views on the identified issues for general aspects of common AI/ML framework. We have the following observations and proposals:

Observation 1:	According to the description in the TR, the AI/ML functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where the configuration(s) is(are) supported based on conditions indicated by UE capability.

Observation 2:	For UE sided model(s) and UE part of two sided model(s) within the AI based functionality , The UE capability reporting is taken as a start point for AI based functionality identification at NW side.

Observation 3:	Only using UE capability reporting to report the conditions for AI based feature may require the high UE ability of storing a large number of AI models for AI based functionalities, and the UE capability reporting may cause the update of AI based functionalities not been renewed timely in the UE capability.

Observation 4:	Regarding the model identification , RAN1 understands that the model identification is a procedure that NW have a same understanding on a UE sided model by the condition/configuration and additional condition (e.g. site, scenarios, datasets) received from UE side.

Observation 5:	RAN2 assumes that model Id is a globally unique which is assigned to one AI model by either offline engineering (e.g. pre-defined and/or hardcoded) or specific Id management node.

Proposal 1:	For UE sided model and UE part of two sided model regarding the functionality based LCM，RAN2 assumes one AI based functionality is associated with a combination of multiple conditions.

Proposal 2:	For UE sided model and UE part of two sided model regarding the functionality indentification, RAN2 assume two-step conditions reporting is applied, whereas UE capability reporting is considered as the first step, one real-time report framework (e.g. UAI) is considered as the second step.

Proposal 3:	For UE sided model and UE part of two sided model regarding the model Id based identification, RAN2 assumes the model identification is a procedure that targets to assign a global model Id to the identified Model.

Proposal 4:	For UE sided model and UE part of two sided model regarding the model Id based identification, RAN2 assumes the model identification can be via either offline testing or an online procedure involving UE and a specific Id management.

Proposal 5:	For UE sided model and UE part of two sided model regarding the model Id based LCM，RAN2 assumes offline model identification have no any RAN2 and RAN1 specification impact.

Proposal 6:	For UE sided model and UE part of two sided model regarding the model Id based LCM, RAN2 assume the online model identification is a procedure involving UE and Id management node which may have SA impact, which can be discussed in WI stage. .

Proposal 7:	Regarding the update of the UE sided/UE part model regarding the model Id based LCM, RAN 2 assumes, the reporting updates is to preform the model identification procedure to obtain a new global model Id.

References

[1] RAN2 TP to TR 38.843