# 3GPP TSG RAN2#123  	                                                 R2-2308843

# Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	7.20.2

Title:	Further consideration on the PCell Configured with two TA

Source:	ZTE Corporation, Sanechips

Document for:	Discussion and Decision

Introduction

In the post email discussion [1],the UE initiated CBRA for acquiring TA are discussed, and the most important issue has been identified, that is, how to model PTAG when 2TA is configured, one PTAG or two PTAG. In here, we share our views on the determination.

Discussion

2 PTAG or 1 PTAG?

First of all, assuming 1 PTAG is adapted, that means all UL transmission would be considered as non-sync if just one of TAGs for PCell is expired which may be caused by the link issue from one TRP. In this case, it seems not necessary to interrupt UP date transmission from the other link whose TA is still valid.

Two PTAG-model can save the data interruption time if UE encounter the link issue for one TRP.

Secondly, the current definition in MAC spec would not be challenged if we apply the two PTAG-model as shown in below:

Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

Two PTAG-model does not challenge the legacy definition of the TAG.

In conclusion:

Tow-PTAG-model is adopted for PCell configured with 2 TAG

Conclusion

In this contribution, we provide our further views on the identified issues for general aspects of common AI/ML framework. We have the following observations and proposals:

Observation 1:	Two PTAG-model can save the data interruption time if UE encounter the link issue for one TRP.

Observation 2:	Two PTAG-model does not challenge the legacy definition of the TAG.

Proposal 1:	Tow-PTAG-model is adopted for PCell configured with 2 TAG

References

[1]: Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)