3GPP TSG RAN WG2#123	R2-2308847

Toulouse, France, August 21-25, 2023

Agenda Item:	7.25.1

Source:	CMCC

Title:	Discussion on the reply LS on applicability of SIB19 for NR ATG

Document for:	Discussion, Decision

Introduction

A reply LS [1] on applicability of SIB19 for NR ATG is sent from RAN4 to RAN2, as follows:

RAN4 would like to thank RAN2 for the reply LS on the applicability of the SIB19 for ATG. RAN4 has discussed the question from RAN2 on required ATG ground station reference location accuracy and had the following agreements: - Base station is allowed to not provide the exact base station location. The exact supported accuracy according to RAN4 requirements is still under discussion and it could be sent to RAN2 if it is needed by RAN2. - RAN4 assumes the BS location accuracy does not need to be specified. - UE performs the frequency/time pre-compensation based on the indicated BS location and the actual UE location, where the UE frequency/timing pre-compensation does not consider the error due to the misalignment between the indicated BS location and the actual BS location. In addition, RAN4 would like to respectfully inform RAN2 that RAN4 has agreed to introduce the mechanism of koffset in ATG system as NTN, whether and how to configure koffset is up to network implementation. RAN4 kindly asks RAN2 to take this information into account when considering the applicability of SIB19 for ATG and design corresponding signaling for it. RAN4 kindly asks RAN2 to take the above information into account. 2. Actions: To RAN2 group. ACTION: RAN4 kindly asks RAN2 to take the above information into account.

In this contribution, we would like to provide some considerations on the reply LS.

Discussion

## 2.1 Accuracy of ATG ground station reference location

In this LS, RAN4 takes the RAN2’s concerns on security risk resulted from the exposure of gNB’s accurate location information, and explained that Base station is allowed to not provide the exact base station location. However, RAN4 assumes the BS location accuracy does not need to be specified, since the UE frequency/timing pre-compensation does not consider the error due to the misalignment between the indicated BS location and the actual BS location during UE performs the frequency/time pre-compensation. Hence, there is no impact on RAN2 specification for this point.

Observation 1: RAN4 takes the RAN2’s concerns on security risk resulted from the exposure of gNB’s accurate location information, and explained that Base station is allowed to not provide the exact base station location.

Observation 2: RAN4 assumes the BS location accuracy does not need to be specified, since the UE frequency/timing pre-compensation does not consider the error due to the misalignment between the indicated BS location and the actual BS location during UE performs the frequency/time pre-compensation.

Proposal 1: There is no impact on RAN2 specification resulted from accuracy of ATG ground station reference location.

## 2.2 The required parameters for ATG

Furthermore, RAN4 also mentioned that koffset mechanism is agreed to introduce in ATG system based on RAN4 discussion. And the indicated BS location is also essential to help UE perform the frequency/time pre-compensation. Then considering the exact supported ATG ground station reference location accuracy according to RAN4 requirements is still under discussion, whether the parameter related to the accuracy is involved in the system information could wait for RAN4 discussion progress. In addition, in another LS [2]:

For inter-frequency cell reselection in RRC IDLE and INACTIVE states, RAN4 has agreed to define two sets (set1 and set2) of cell-reselection requirements for ATG UEs (one based on Rel-15 NR reselection requirements and other based on Rel-17 HST requirements). RAN4 has agreed that the network may indicate via system information, whether set1 or set2 requirements apply for a particular cell based on cell configuration.

Therefore, regarding the parameters provided to ATG UE, koffset, a indicated BS location and a cell reselection requirements set indication are needed to be included in the system information.

Proposal 2: It is proposed that koffset, a indicated BS location and a cell reselection requirements set indication are needed to be included in the system information.

## 2.3 Whether the SIB19 could be reused

Considering that SIB19 could help UE obtain the network type(e.g.NTN or TN) information implicitly, if we reuse the SIB19 in ATG system may lead to some confusion and requiring additional discussion on how to address the issue for NTN type acquisition. Furthermore, only just a few parameters have been identified in RAN4 , which is more lean than SIB19, therefore, it is proposed to define a new separate SIBxx, which may not introduce excessive workload and is more distinct.

Proposal 3: It is better to introduce a new separate SIBxx to provide the ATG specific information.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on the reply LS on applicability of SIB19 for NR ATG and have the following observations and proposals:

Observation 1: RAN4 takes the RAN2’s concerns on security risk resulted from the exposure of gNB’s accurate location information, and explained that Base station is allowed to not provide the exact base station location.

Observation 2: RAN4 assumes the BS location accuracy does not need to be specified, since the UE frequency/timing pre-compensation does not consider the error due to the misalignment between the indicated BS location and the actual BS location during UE performs the frequency/time pre-compensation.

Proposal 1: There is no impact on RAN2 specification resulted from accuracy of ATG ground station reference location.

Proposal 2: It is proposed that koffset, a indicated BS location and a cell reselection requirements set indication are needed to be included in the system information.

Proposal 3: It is better to introduce a new separate SIBxx to provide the ATG specific information.

Reference

- R2-2307038_R4-2310058 Reply LS on applicability of SIB19 for NR ATG

- R2-2307040_R4-2310152 LS on signaling for NR ATG