3GPP TSG-RAN WG2 Meeting #123											    R2-2308940

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	Discussion on RACH-less handover

Agenda Item:	7.12.2.1

Document for:	Discussion and Decision

# Introduction

According to the discussion on RACH-less solution in the previous RAN2 meetings, RAN2 made the following agreements:

RAN2#119 meeting agreements [1]: - R2 assumes RACH-less procedure may be considered for on-board RRC_CONNECTED UEs, which are to be handed over together with the mobile IAB-node (would depend also on the assumptions for UL synch).

RAN2#121bis-e meeting agreements [2]: Agreements on RACH-less handover: - RACH-less for mIAB scenario, if agreed in the end, will cover only the case of same-TA. - Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB). - RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration: - Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details. - Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command). RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

RAN2#122 meeting agreements [3]: Agreements on RACH-less handover: - RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured). - For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report). - for the UL grant and HO completion in RACH-less HO: - Both type-1 configured grant and dynamic grant are supported - FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI). - Send LS to RAN3 to check whether there are issues / feasibility concerns. Approved: LS on UE RACH-less handover for mobile IAB to RAN3 in R2-2306817

In this contribution, we provide our understandings on the remaining issues for the mIAB RACH-less solution.

# Discussion

## RACH-less handover in mobile IAB

Since RAN2 agreed to “cover only the case of same-TA” for the RACH-less for mIAB scenario, then one may consider that the UE may be configured with multiple TAGs, and the IAB-donor-CU needs to indicate which TAG’s TA is used for the target cell of the UE. However, the RACH-less for mIAB is “for on-board RRC_CONNECTED UEs”, there seems no clear use case “for on-board RRC_CONNECTED UEs” configured with multiple TAGs. Thus, we consider that no TAG ID needs to be indicated to the UE. The UE can always use the TA of the PTAG

Observation 1: There is no clear use case that the on-board RRC_CONNECTED UE is configured with multiple TAGs.

Unlike LTM which uses the LTM Cell Switch MAC CE to indicate the TA for the target cell, the RACH-less solution for mIAB is not for the UE configured with multiple candidate configurations of target cells (i.e. not for LTM). Thus, we think that the handover command of the RRCReconfiguration message can indicate the UE to reuse the TA of the source PTAG.

Considering the use case of the on-board RRC_CONNECTED UEs with very short transmission range to the vehicle, it is common that the UE would use TA=0 for uplink transmission. Then we think that the RRC HO command should also be able to indicate the TA=0 used for the target cell.

Proposal 1: RRC HO command indicates whether TA=0 or the TA of the source PTAG is reused for the target PTAG.

Regarding the completion confirmation of the RACH-less solution, it seems that the RACH-less for NTN and the RACH-less for LTM would have different solutions. Since the discussion for the LTM is still ongoing for the completion confirmation of the RACH-less solution, we think that the completion confirmation of the RACH-less solution for mIAB can wait a bit to align with the conclusion made in other items.

Observation 2: The completion confirmation signaling of the RACH-less solution for mIAB can wait for the conclusion made in other items, i.e. NTN and LTM.

Regarding the supervision timer for the RACH-less solution, we think that the T304 can be reused, as the starting condition for the supervision timer for the RACH-less solution for mIAB is different from the starting condition for the supervision timer for the RACH-less solution for LTM, which starts at the reception the LTM Cell Switch MAC CE.

Proposal 2: T304 is reused to supervise the RACH-less of mIAB.

Regarding the stop condition of the supervision timer of RACH-less for mIAB, we think that the UE can stop the supervision timer of RACH-less for mIAB, when the UE receives the completion confirmation of the RACH-less solution for mIAB.

Proposal 3: T304 for mIAB RACH-less is stopped when the UE receives the completion confirmation of mIAB RACH-less.

When T304 for the RACH-less of mIAB expires, we think that the legacy UE behaviours (i.e. initiating RRC reestablishment) can be reused.

Observation 3: When T304 for the RACH-less of mIAB expires, the legacy UE behaviour (i.e. initiating RRC reestablishment) is reused.

Regarding how to configured the CG-RACH-less or DG-RACH-less for the UE, we think that the DG-RACH-less can be used as the default solution. This means that when the TA for RACH-less is indicated in the handover command, and when the gNB does not provide the Type-1 CG configuration for the target cell, the UE should monitor the PDCCH to get the DG resource. We think that no matter whether to use CG resource or DG resource for the mIAB RACH-less, the legacy configuration can be reused, and RAN1 specification does not require any enhancement.

Proposal 4: For mIAB RACH-less, if the Type-1 CG configuration for the target cell is not configured, the UE monitors the target cell PDCCH for the UL grant to the target cell.

# Conclusion

According to the above analysis, we have the following observations and proposals.

Observation 1: There is no clear use case that the on-board RRC_CONNECTED UE is configured with multiple TAGs.

Observation 2: The completion confirmation signaling of the RACH-less solution for mIAB can wait for the conclusion made in other items, i.e. NTN and LTM.

Observation 3: When T304 for the RACH-less of mIAB expires, the legacy UE behaviour (i.e. initiating RRC reestablishment) is reused.

Proposal 1: RRC HO command indicates whether TA=0 or the TA of the source PTAG is reused for the target PTAG.

Proposal 2: T304 is reused to supervise the RACH-less of mIAB.

Proposal 3: T304 for mIAB RACH-less is stopped when the UE receives the completion confirmation of mIAB RACH-less.

Proposal 4: For mIAB RACH-less, if the Type-1 CG configuration for the target cell is not configured, the UE monitors the target cell PDCCH for the UL grant to the target cell.

# Reference

[1] RAN2#119 meeting minutes

[2] RAN2#121bis-e meeting minutes

[3] RAN2#122 meeting minutes