3GPP TSG-RAN WG2 Meeting #123	R2-2308982

Toulouse, France, August 21st - 25th, 2023

Agenda:	7.25.4

Source:	Ericsson

Title:	Report of [AT123][102][NTN Self Ev] CP/UP latency assumptions (Ericsson)

Document for:	Discussion and decision

# 1	Introduction

ITU-R has defined requirements for the satellite component of IMT-2020 [1]. The set of requirements and the corresponding evaluation guidelines are based on the terrestrial procedure, to which 3GPP has submitted TR 37.910 [2]. Similar to the terrestrial case, the requirements include a CP and UP latency targets which have been set to 40 ms and 10 ms, respectively [1], to take into account the longer propagation delay inherent to satellite communications.

# 2	Summary of remaining issues (first round)

## 2.1	Propagation delay (RTD)

The most significant difference with respect to the terrestrial evaluation documented in TR 37.910 [2] is the addition of the satellite’s long propagation delay, i.e., round trip delay (RTD). During RAN2#122, the topic was briefly discussed, and the following agreement was reached:

- For RTD we consider the same scenario as considered by RAN1

RAN1 has already made some progress in their parameter and scenario selection for IMT-2020 simulations. During RAN1#112bis-e and RAN1#113, the following relevant agreement were made [3]:

- The evaluation performed by RAN1 will consider at least the following scenario:

  - Transparent payload without ISL

  - S-band (2<GHz)

  - LEO-600

  - Handheld UEs

- For peak spectral efficiency and peak data rate parameters:

  - The parameters are chosen based on “ideal conditions”: 90degree elevation angle, 0dB atmospheric loss, 0dB shadow fading margin, 0dB scintillation loss, 0dB polarization loss, 0dB additional losses.

  - Companies to provide in RAN1#113 realistic parameters, declaring the assumptions and evaluations leading to those parameters.

Even though there seems to be an overall consensus for the baseline scenario, submissions to RAN2#123 reflect a variety of views:

For the feeder and service link delay consider LEO at 600 km with both UE and GW/gNB at nadir.[6]

The propagation delay (RTD) due to service and feeder links is 8 ms for both control and user plane in the LEO-600 scenario (corresponding to 90° elevation angle).[4]

RAN2 agrees to consider RTD of LEO 600km in user plane/control plane latency evaluation.[8]

In LEO-600, with a minimum elevation angle for both feeder link and service link, i.e., 10° for service link and 10° for feeder link. [5]

The propagation delay is also calculated based on LEO satellite orbit altitude of 600km. For evaluation purpose, 79° elevation angle for both gNB and UE are assumed. [11]

Q1. To calculate the round-trip delay, can “ideal conditions” be assumed for the evaluation of UP and CP latency? From RAN2 perspective, “ideal conditions” refer to the case where both UE and gNB are located at the satellite’s nadir, i.e., elevation angles are 90 degrees.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| THALES | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes | According to the explanation in the definition of Q1 on what ‘ideal conditions’ mean. |
| Xiaomi | Yes |  |
| Huawei, HiSilicon | Yes, with comments | It is clear which one is the ideal condition, at the same this ideal condition is really … ”ideal”. We could try to also analyze an “average” condition. If it is not difficult to conclude on this, we can even document our findings. |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Turkcell | Yes |  |

Summary: Consensus to assume both UE and gNB are located at the satellite’s nadir, i.e., elevation angles are 90 degrees. One company commented that findings for alternative scenarios (“average condition”) can be also documented.

RAN2 assumes that both UE and gNB are located at the satellite’s nadir, i.e., elevation angles are 90 degrees, for the calculation of round trip delay (RTD).

Once the baseline scenario is settled, the next issue relates to whether feeder and/or service link propagation delays are used to calculate the overall propagation delay. There are two opposing views on the topic:

Feeder and service links propagation delays are not included in the time budget to satisfy the user plane and control plane latency requirement.[1]

Feeder and service link delays need to be taken into account in the user and control plane latency analysis for NTN.[3]

Q2. Do feeder and service link delays need to be included in the propagation delay computation?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| THALES | See comment | The feeder and service link delays could be included in the propagation delay computation but they are not included in the time budget to satisfy user/control plane latency. Otherwise, in a majority of scenarios different to the ideal one in Q1 (LEO-1200, LEO-600 with 30° elevation for UE and GTW, GEO), we may not satisfy the time budget because of the propagation delay. |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes | Those are significant component of the overall delay, so cannot be skipped. |
| Xioami | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes |  |
| Turkcell | Yes |  |

Summary: All companies agree that feeder and service link delays should be included in the propagation delay computation. Additionally, one company noted that these delays should not be included in the time budget to satisfy the user and control plane latencies. Rapporteur wants to highlight that the Report ITU-R M.2412 provides an example of the elements in the calculation, e.g., include the assumed time-of-flight between the terminal and the base station.

Feeder and service link delays are included in the propagation delay computation (RTD).

## 2.2	Mobility interruption time

According to the definition, mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during mobility transitions. This parameter was already discussed in RAN2#123-bis reaching the following agreement:

- Confirm 0ms mobility interruption time is achieved by NR in beam mobility.

Even though this was already discussed in the previous meeting and there seems to be an overall consensus, there are submissions to RAN2#123 which reflect a variety of views:

The inter cell level mobility interruption time is not evaluated for the IMT-2020 Satellite self-evaluation.[4]

It is sufficient to consider beam-based mobility to achieve 0ms mobility interruption and other scenarios (e.g., service link and feeder link switch) will not be considered for mobility interruption time evaluation.[7]

The interruption time during mobility across cell should also be evaluated. [5]

Q3. Given mobility interruption time refer to the shortest time duration supported by the system, does RAN2 need to consider the inter-cell/inter-satellite scenario?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No |  |
| THALES | No |  |
| Qualcomm | No |  |
| ZTE | No |  |
| Lenovo | No |  |
| Nokia |  | It depends what do we assume under the term ‘mobility’. If inter-cell handover is considered then the agreement taken at RAN2#122 is not valid and the answer here is YES. |
| Xiaomi | No |  |
| Huawei, HiSilicon |  | Similar to Nokia. Given that NTN not only contains the scenario where the cell/satellite never change, the inter-cell mobility should be also evaluated, at least for reference. |
| MediaTek | No |  |
| Ericsson | No |  |
| Turkcell |  | Agree with Nokia |

Summary: The majority of the companies (8/11) agree that the evaluation of mobility interruption shall not consider the inter-cell scenario. An agreement was already made in RAN2#122 for the intra-cell scenario. Two companies argue that the evaluation may also document other scenarios, for reference. Rapporteur wants to highlight again that the definition of mobility interruption in this context refers to “the shortest time duration supported by the system”.

For the mobility interruption evaluation, RAN2 (8/11) assumes that it is sufficient to consider beam-based mobility in NTN. Other scenarios (e.g., service link and feeder link switch) will not be considered for this evaluation.

## 2.3	Satellite processing delay

In addition to longer propagation delays, NTN introduces a satellite payload which is transparent to the UE-BS communication, i.e., it forwards the signal without perform any processing on it. The processing delay related to the satellite corresponds to the routing of the signal from the feeder receiving antenna to the service link transceiver and inversely and its amplification [4]. Again, there are two opposing views:

The satellite on-board processing delay in transparent mode can be considered negligible (i.e. << 1ms).[1]

The value may be non-negligible and RAN1 needs to provide it [2].

Q4. Can the satellite on-board processing delay in transparent architecture be considered negligible?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| THALES | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| Nokia | Not sure | Not sure if RAN2 is the right group to assess what is the satellite onboard processing delay. This is probably something that is outside of 3GPP ecosystem’s scope. |
| Xiaomi | Yes |  |
| Huawei, HiSilicon | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Turkcell | Yes |  |

Summary: The majority of the companies (10/11) agree that the satellite on-board processing in transparent architecture can be considered negligible for the IMT-2020 evaluation. One company thinks that RAN2 is not the right group to assess this value.

From RAN2 perspective, satellite on-board delay can be considered negligible.

## 2.3	Control Plane procedure

As defined in Report ITU-R M.2410, control plane latency refers to the transition time from a most “battery efficient” state (e.g., Idle state) to the start of continuous data transfer (e.g. Active state). In RAN2#122, the following relevant agreements were made:

- Evaluate the control plane latency from RRC_INACTIVE to RRC_CONNECTED.

- Evaluate the control plane latency based on the 2-step RACH.

Figure 1. Illustration of CP signalling during transition from RRC_INACTIVE to RRC_CONNECTED mode.

Q5. Can the following high-level control plane procedure be used for NR NTN?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| THALES | Yes |  |
| Qualcomm | Yes | As defined in Figure 1. |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| Xiaomi | Yes |  |
| Huawei, HiSilicon | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Turkcell | Yes |  |

Summary: Consensus.

RAN2 assumes the CP procedure defined in Figure 1 as the baseline for the CP evaluation.

Q6. Provide company input about the detailed procedure and delay assumptions.

| Step | Description | CP latency for UL data transfer | Company comments |
| --- | --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |  |
| 2.1 | Transmission of MsgA (RACH Preamble + PUSCH) | Ts (the length of 1 slot / non-slot) | Qualcomm : Preamble and PUSCH are not transmitted at the same time. There is at least additional 1 ms gap between preamble and PUSCH. Huawei, HiSilicon: We need to separate the transmission of preamble and PUSCH and also need to consider the interval between preamble and pusch. |
| 2.2 | Propagation delay UE -> BS | RTD/2 | THALES: true if the WA is that the satellite position (regarding to UE and GTW) difference between UE->BS and BS->UE is negligible. |
| 3 | Preamble detection and processing in gNB | 3 ms (Same as for reception of Msg3 in NR TN) | ZTE: shall be MsgA detection and processing, which includes both preamble and PUSCH. |
| 4.1 | Transmission of MsgB (RA response and RRC resume) | Ts (the length of 1 slot / non-slot) | Huawei, HiSilicon:MsgB also can include the RRC resume. |
| 4.2 | Propagation delay BS -> UE | RTD/2 | THALES: Same as 2.2. |
| 5 | UE Processing Delay | 7 ms (Same as for reception of Msg4 in NR TN) | ZTE: There are additional NT,1+NT,2+0.5 ms, which is UE Processing Delay for decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Resume Request) |
| 6 | Transmission of RRC Resume Complete and data | 0 |  |

## 2.4	User Plane procedure

As defined in Report ITU-R M.2410, user plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). In RAN2#122, the following relevant agreement was made:

- For user plane latency evaluation, HARQ disabling should be assumed.

Given HARQ mechanism is disabled, the rapporteur has noticed that some contributions assume the radio interface to be error-free (p=0), while other believe blind retransmissions can be considered without impact in the time budget calculation [5]. In [4], it is proposed to consider a realistic (p=0.1) and its effect on the RLC ARQ mechanism. In this case, the RLC ARQ latency for loss recovery should be included in the user plane latency.

Q7. Which of the following should be included the total user plane latency calculation to account for a realistic error rate (p=0.1) in the radio interface?

a	RLC ARQ.

b	Blind retransmissions.

c	None.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | B |  |
| THALES | A see comment | Either we decide to consider an ideal situation (maximum power, at the nadir, no blockage...) with no losses and then p=0.0 or we consider a more realistic situation with a target loss for BLER of p=0.1 and then, it is reallistic to say that this is RLC ARQ that will handle the retransmission of losses. In that case, the RLC ARQ latency with p=0.1 should be included in the user plane latency. In my understanding, if we choose blind retransmission, we increase a lot the overhead (compared to RLC ARQ with feedback) and thus it reduces the peak data rate. RAN1 should be informed about the retransmission mechanism chosen for the calculation of data rates. |
| Qualcomm | None | Now we think the interpretation of RAN2 agreement of using HARQ disable scenario is that probably it is sufficient to consider p = 0. So lets not consider p = 0.1. |
| ZTE | comments | RAN2 can start with P= 0. Whether to consider P=0.1 may be relevant to other assumptions we made. For example, if both satellite and UE at nadir is assumed for RTD, then the need to have retransmission is less due to LOS. But still p=0.1 is more realistic, and we are open to consider it. |
| Lenovo | comments | Agree with ZTE to start with P=0 and p=0.1 can be further discussed. |
| Nokia | A with comments | For LEO600 we should perhaps consider sending feedback, so option A is preferred, rather than option B. |
| Xiaomi | None |  |
| Huawei, HiSilicon | B with comments | In terms of latency, RLC ARQ is similar to HARQ. Since HARQ is disabled, RLC ARQ shouldn’t be considered either, and blind retransmissions can be used instead. |
| MediaTek | C | Better to start with p = 0. |
| Ericsson | C | As a starting point. |
| Turkcell | B |  |

Summary: The following options have been discussed.

a	RLC ARQ. (2/11)

b	Blind retransmissions. (3/11)

c	None. (6/11)

A slight majority of the companies (6/11) think that RAN2 should start with the lossless scenario (p=0). One company argues that blind retransmissions can be used, RLC ARQ shouldn’t be considered when HARQ is disabled. Another company thinks that blind retransmissions would decrease the achievable data rate and RLC ARQ will handle possible losses in the radio interface.

RAN2 (6/11) assumes a lossless scenario (p=0) for the User plane evaluation. FFS whether losses (p=0.1) scenario is considered later. FFS RLC ARQ or blind retransmissions are used in that case.

### 2.4.1	DL latency

Given the assumption to disable HARQ feedback, the delay due to retransmission does not need to be considered.

Q8. Provide company input about the detailed procedure and delay assumptions.

| ID | Component | Notations | Value | Company comments |
| --- | --- | --- | --- | --- |
| 1 | BS processing delay | tBS,tx The time interval between data arrival and packet generation. | Tproc,2/2, with d2,1 = d2,2 = 0. Tproc,2 is defined in Section 6.4 of TS 38.214. |  |
| 2 | DL frame alignment (transmission alignment) | tFA,DL The time interval between packet generation and the next Tx opportunity. | TFA Length of one slot, since TFA is bounded by the slot duration. |  |
| 3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |  |
| 4 | One-way propagation time BS -> satellite -> UE | tprop | RTD/2 |  |
| 5 | UE processing delay | tUE,rx The time interval between PDSCH reception and decoding of the data. | Tproc,1/2, with d1,1 = 0. Tproc,1 is defined in Section 5.3 of TS 38.214. |  |
|  | Total UP latency for DL | TDL = (tBS,tx + tFA,DL) + (tDL_duration + tprop)+ tUE,rx |  | ZTE: Depends on outcome of P7, for p=0 it is fine. If retransmission with p=0.1 is considered, the table will need to be updated to consider retransmission. Huawei, HiSilicon: If we only chose to analyze the best case, we can consider no retransmission, but if we also analyze a realistic scenario, an “average” scenario, then the total latency need to include the delay of (some) blind retransmissions. |

### 2.4.2	UL latency

Given the agreement to disable HARQ feedback, the rapporteur sees different interpretations in several contributions. Some assumed that this only applies to the DL [10], while others have assumed it also applies to the UL thanks to HARQ mode B [5,7].

Q9. HARQ mode B (no retransmissions) is assumed to calculate the UP uplink latency.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| THALES | Yes |  |
| Qualcomm | Yes |  |
| ZTE |  | For realistic consideration, we can consider both with retransmission (assuming p=0.1 ) and without retransmission. |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| Xiaomi | Yes |  |
| Huawei. HiSilicon |  | See our paper and our comments for the DL (above) and UL (below). RAN2 needs to decide if we only cover the absolute best case or we also include a more realistic case. In case we chose the first, we anyway need to clearly explain the assumptions that we took. |
| MediaTek | Yes |  |
| Ericsson | Yes | Align with majority. |
| Turkcell |  | We need to first clarify assumption |

Summary: The majority of the companies (8/11) agree that HARQ mode B is used to calculate User plane UL latency. Two companies highlight that we should align both User plane cases whether a lossless scenario (p=0) or a more realistic scenario is considered (p=0.1).

RAN2 assumes HARQ mode B to calculate the User plane uplink latency.

Q10. Provide company input about the detailed procedure and delay assumptions.

| ID | Component | Notations | Value | Company input |
| --- | --- | --- | --- | --- |
| 1.1 | UE processing delay | tUE,tx The time interval between data arrival and packet generation. | Tproc,2/2, with d2,1 = d2,2 = 0. Tproc,2 is defined in Section 6.4 of TS 38.214. |  |
| 1.2 | UL frame alignment (transmission alignment) | tFA,UL The time interval between packet generation and the next Tx opportunity. | TFA Length of one slot, since TFA is bounded by the slot duration. |  |
| 1.3 | TTI for UL data packet transmission | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |  |
| 1.4 | One-way propagation time UE -> satellite -> BS | tprop | RTD/2 |  |
| 1.5 | BS processing delay | tBS,rx The time interval between PUSCH reception and decoding of the data. | Tproc,1/2, with d1,1 = 0. Tproc,1 is defined in Section 5.3 of TS 38.214. |  |
|  | UL data transfer | T1 The sum of steps 1.1 to 1.5. | T1 = (tUE,tx + tFA,UL) + (tUL_duration + tprop) + tBS,rx | ZTE: Depends on outcome of P9, For p=0 it is fine. If retransmission with p=0.1 is considered, the table will need to be updated to consider retransmission. Huawei, HiSilicon: If we only chose to analyze the best case, we can consider no retransmission, but if we also analyze a realistic scenario, an “average” scenario, then the total latency need to include the delay of (some) blind retransmissions. |

# 3	Second round

For the IMT-2020 self-evaluation of the satellite component, the baseline scenario to evaluate CP and UL latency requirements is the best-case. This does not preclude the report of other cases. In fact,

- At the moment, RAN2 assumes the best-case scenario even though RAN2 understands that it might not be a common scenario in some cases.

Review of the agreements from first email round.

- RAN2 assumes that both UE and gNB are located at the satellite’s nadir, i.e., elevation angles are 90 degrees, for the calculation of round trip delay (RTD).

- For agreement.

- Feeder and service link delays are included in the propagation delay computation (RTD).

- Thales thinks that for the case under consideration is OK, but for other cases, it might be difficult to comply with the requirements (where propagation time is larger)

- Qualcomm thinks we can consider the scenario of 600 KM altitude as a baseline. Select the best case scenario to comply with the requirements.

- Huawei we need to make an statement that we are considering the best case even though it might not be a realistic use-case.

- For agreement, given the assumptions from Proposal 1.

- For the mobility interruption evaluation, RAN2 assumes that it is sufficient to consider beam-based mobility in NTN. Other scenarios (e.g., service link and feeder link switch) will not be considered for this evaluation.

- Huawei rephrase the proposal and remove the later part so that we do not preclude other scenarios later.

- For agreement rephrased as -> “For the mobility interruption evaluation, RAN2 assumes that for now it is sufficient to consider beam-based mobility in NTN”.

- From RAN2 perspective, satellite on-board delay can be considered negligible.

- For agreement.

- RAN2 assumes the CP procedure defined in Figure 1 as the baseline for the CP evaluation.

- For agreement.

- RAN2 assumes a lossless scenario (p=0) for the User plane evaluation. FFS whether losses (p=0.1) scenario is considered later. FFS RLC ARQ or blind retransmissions are used in that case.

- HW thinks that we have explicitly considered HARQ disabled and wonders why we should consider RLC ARQ. For the evaluation, blind rtx might be sufficient.

- Thales wonder if it is realistic to have a p=0. From the physical layer perspective there may be losses so we need to consider a retransmission mechanism. Should we consider the RLC layer or blind retransmission.

- QC thinks the first FFS is optional and the second FFS can be further discussed.

- For agreement rephrased as- > “RAN2 assumes a lossless scenario (p=0) for the User plane evaluation. For the best-case scenario, RAN2 will not consider retransmissions.”

- RAN2 assumes HARQ mode B to calculate the User plane uplink latency.

- ZTE thinks that if the intention is not to consider retransmissions, there’s no need to agree to this (it is implicit).

- QC also understands that Proposal 7 applies to both UL and DL.

## 3.1	CP latency

| Step | Description | CP latency for UL data transfer | Company comments |
| --- | --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |  |
| 2.1 | Transmission of MsgA |  |  |
| 2.1.1 | Transmission of RACH Preamble | Length of the preamble according to the PRACH format as specified in [6 38.211] |  |
| 2.1.2 | Transmission interval | 1 ms / Ask RAN1 | QC thinks this value is configurable by RRC and we should consider the minimum value for this scenario. HW thinks that we need to check with RAN1 to confirm the assumptions. |
| 2.1.3 | Transmission of PUSCH payload (RRCResumeRequest) | Ts (the length of 1 slot / non-slot) |  |
| 2.2 | Propagation delay UE -> BS | RTD/2 |  |
| 3 | MsgA detection and processing delay in gNB (preamble, L2, RRC) | 3 ms (Same as for reception of Msg3 in NR TN) / Ask RAN1 |  |
| 4 | Transmission of MsgB |  |  |
| 4.1 | Transmission of MsgB (RA response and RRCResume) | Ts (the length of 1 slot / non-slot) |  |
| 4.2 | Propagation delay BS -> UE | RTD/2 |  |
| 5 | Processing delay in UE of RRC Resume including RA response | 7 ms (Same as for reception of Msg4 in NR TN) / additional NT,1+NT,2+0.5 ms / Ask RAN1 |  |
| 6 | Transmission of RRC Resume Complete and data | 0 |  |

Summary:

The companies present in the discussion think the above detailed procedure to be agreeable.

Regarding the values in steps 2.1.2, 4.1 and 5, RAN2 can make an assumption based on the values that can be configured (e.g., RRC) and request (if needed) RAN1 to confirm those values.

## 3.2	UP latency DL

Not treated during the F2F offline discussion.

| ID | Component | Notations | Value | Company comments |
| --- | --- | --- | --- | --- |
| 1.1 | BS processing delay | tBS,tx The time interval between data arrival and packet generation. | Tproc,2/2, with d2,1 = d2,2 = 0. Tproc,2 is defined in Section 6.4 of TS 38.214. |  |
| 1.2 | DL frame alignment (transmission alignment) | tFA,DL The time interval between packet generation and the next Tx opportunity. | TFA Length of one slot, since TFA is bounded by the slot duration. |  |
| 1.3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |  |
| 1.4 | One-way propagation time BS -> satellite -> UE | tprop | RTD/2 |  |
| 1.5 | UE processing delay | tUE,rx The time interval between PDSCH reception and decoding of the data. | Tproc,1/2, with d1,1 = 0. Tproc,1 is defined in Section 5.3 of TS 38.214. |  |

## 3.3	UP latency UL

Not treated during the F2F offline discussion.

| ID | Component | Notations | Value | Company input |
| --- | --- | --- | --- | --- |
| 1.1 | UE processing delay | tUE,tx The time interval between data arrival and packet generation. | Tproc,2/2, with d2,1 = d2,2 = 0. Tproc,2 is defined in Section 6.4 of TS 38.214. |  |
| 1.2 | UL frame alignment (transmission alignment) | tFA,UL The time interval between packet generation and the next Tx opportunity. | TFA Length of one slot, since TFA is bounded by the slot duration. |  |
| 1.3 | TTI for UL data packet transmission | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |  |
| 1.4 | One-way propagation time UE -> satellite -> BS | tprop | RTD/2 |  |
| 1.5 | BS processing delay | tBS,rx The time interval between PUSCH reception and decoding of the data. | Tproc,1/2, with d1,1 = 0. Tproc,1 is defined in Section 5.3 of TS 38.214. |  |

## 3.4	Further assumptions

For the previous delay estimations, [5] proposes to take a few other assumptions into consideration.

- NR FDD

- Only NTN bands are considered (n255, n256).

- UE capabilities 1 & 2

- Resource type mapping A &B

- SCS 15 kHz. FFS whether to report results for other SCS (e.g., 30).

- RAN2 assumes the following for the evaluation of CP and UP latency:

- NR FDD

- Only NTN bands are considered (n255, n256).

- UE capabilities 1 & 2

- Resource type mapping A &B

- SCS 15 kHz for the baseline scenario. FFS other supported scenarios (e.g., 30).

# 4	Conclusions & proposals

List of proposals for agreement:

- At the moment, RAN2 assumes the best-case scenario even though RAN2 understands that it might not be a common scenario in some cases.

- RAN2 assumes that both UE and gNB are located at the satellite’s nadir, i.e., elevation angles are 90 degrees, for the calculation of round trip delay (RTD).

- Given the assumptions of Proposal 1, feeder and service link delays are included in the propagation delay computation (RTD).

- For the mobility interruption evaluation, RAN2 assumes that for now it is sufficient to consider beam-based mobility in NTN.

- From RAN2 perspective, satellite on-board delay can be considered negligible.

- RAN2 assumes the CP procedure defined in Figure 1 as the baseline for the CP evaluation.

- RAN2 assumes a lossless scenario (p=0) for the User plane evaluation. For the best-case scenario, RAN2 will not consider retransmissions.

- RAN2 assumes the following for the evaluation of CP and UP latency:

- NR FDD

- Only NTN bands are considered (n255, n256).

- UE capabilities 1 & 2

- Resource type mapping A &B

- SCS 15 kHz for the baseline scenario. FFS other supported scenarios (e.g., 30 kHz).

# 5	References

R1-2304123, Feature lead summary #4 on evaluation methodology for IMT-2020 Satellite, RAN1.

ITU-R M.2514, "Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020".

R1-2306083,

R2-2307322, Discussion on IMT-2020 Satellite self-evaluation for Latency and Mobility, THALES, RAN2#123,  Toulouse, France, August 2023

R2-2307496, Self-Evaluation for NR NTN, Huawei, HiSilicon, RAN2#123,  Toulouse, France, August 2023

R2-2307586, On CP and UP Latency for IMT-2020 NTN Self Evaluation, Nokia, Nokia Shanghai Bell, RAN2#123,  Toulouse, France, August 2023

R2-2307624, RAN2 aspects on evaluation methodology for IMT-2020 Satellite, Qualcomm Incorporated, RAN2#123,  Toulouse, France, August 2023

R2-2308508, Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology, ZTE Corporation, Sanechips, RAN2#123,  Toulouse, France, August 2023

R2-2308903, Satellite IMT-2020 self-evaluation: CP latency, Ericsson, RAN2#123,  Toulouse, France, August 2023

R2-2308905, Satellite IMT-2020 self-evaluation: UP latency, Ericsson, RAN2#123,  Toulouse, France, August 2023

R1-2307551, Discussion on self-evaluation methodology for IMT-2020 satellite radio interface, OPPO, RAN1#114, Toulouse, France.