3GPP TSG-RAN WG2 Meeting #123 meeting	                      R2-2309002

Toulouse, France, 20th – 25th August, 2023

Agenda item:		7.5.4.2

Source:		Lenovo

Title:		Summary of [AT123][201][XR] XR MAC CEs

Document for:		Discussion and Decision

# Introduction

This document is providing the summary of the F2F offline discussion [AT123][201][XR] XR MAC CEs.

# Discussion

Agreements from XR session on Monday:

Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG. When UE triggers reporting delay information for a LCG, UE also reports the buffer status associated with the remaining time. RAN2 aims to define a single MAC CE for the DSR reporting (including the buffer status). FFS if this extends BSR MAC CE or is a new MAC CE.

Based on above agreements made in the XR session on Monday, the following topics were on the agenda of the offline discussion:

Question 1 (MAC CE format for DSR info)

- Option 1: introduce a new separate MAC CE for reporting DSR (remaining delay + BS)

  - Define a new (DSR) MAC CE separately from the BSR MAC CE, e.g. UE reports BSR MAC CE containing BSR only for BSR trigger, and reports new DSR MAC CE (remaining delay + associated BS) for DSR trigger

- Option 2: extend a BSR MAC CE to carry also DSR (remaining delay + BS) info

Question 2 (trigger conditions for DSR)

- DSR info reporting is triggered based on NW configured threshold, e.g. if remaining delay is below threshold reporting is triggered?

  - Further details: (single) threshold per LCG or per LCH

Question 3 (number of DSR info(s) per LCG)

- Option 1: Single (lowest remaining delay/most urgent) DSR info is reported per LCG

- Option 2: multiple DSR (remaining delay + BS) info is reported per LCG

  - Details FFS (how many DSR info, which are reported)

Question 4 (Remaining delay info format)

- Option 1: Remaining delay is explicitly signalled (in ms)

  - Intra-UE prioritization needs to be addressed?

- Option 2: exact remaining delay is not signalled but implicitly indicated by reporting event itself (remaining delay below threshold)

Summary of the offline discussion

During the offline discussion, which was held on Tuesday in the morning coffee break mainly the first and second question were discussed, whereas question 3 was only briefly touched upon and question 4 was not discussed.

On question 1 after clarifying the differences between option 1 and option 2, a majority of companies expressed a preference for Option 1, i.e. define a new separate MAC CE for DSR reporting. The main arguments in favour of Option 1 – even though both options would work - were mainly that separating DSR reporting from BSR reporting seems like a cleaner and simple solution as DSR reporting and BSR reporting have for example different triggers. DSR and BSR reporting can be considered as separate scheduling information and hence should not be coupled. It would be also easier from Network configuration perspective to decouple them, e.g. NW may not always configure DSR reporting. Furthermore, from specification perspective it would be beneficial to separate those two reporting procedures and define separate MAC CEs even though it might lead to slightly more overhead.

It was suggested to agree on option 1 as a working assumption.

On question 2 there was a majority to introduce a triggering event for DSR reporting based on a network configured threshold, e.g. if remaining delay is below a threshold UE reports DSR information. The threshold should be configured per LCG to keep it simple. The option of configuring multiple thresholds for a LCG was not discussed in detail and hence should be kept FFS.

Given the discussion, it is proposed that RAN2 agrees on introducing threshold based DSR reporting, where a threshold is configured per LCG. FFS whether multiple thresholds can be configured for a LCG.

# Conclusion

In this contribution the summary of the F2F offline discussion [AT123][201][XR] XR MAC CEs is provided. It is proposed that RAN2 discusses the following in the CB session on Thursday:

Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS.

Proposal: Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS.

# References