3GPP TSG-RAN WG2 Meeting #123	R2-2309121

# Toulouse, France, 21st – 25th August 2023

Agenda Item:	5.2

Source:	Ericsson

Title:	Summary of [501][V2X/SL] SL 38.300 corrections (Ericsson)

Document for:	Discussion, Decision

# Introduction

[AT123][501][V2X/SL] SL 38.300 corrections (Ericsson)

Scope: Discuss and conclude R2-2308431/R2-2308432.

Intended outcome: Discussion summary in R2-2309121 and 38.300 CR in R2-2309122/R2-2309123 (if needed)

Deadline: 8/24 08:30 (CET) (Email approval)

The discussion includes two phases.

– Phase I is to collect companies views on the changes. The suggested deadline for companies' feedback: Wednesday, 2023-08-23 1100 CET.

– Phase II is to summarize the companies’ views and provide a summary report. If there is majority view on changes, also provide 38.300 CRs. The deadline is Wednesday, 2023-08-23 1600 CET.

# Summary

## Contact information

| Company | Name | Email Address |
| --- | --- | --- |
| Ericsson | Min Wang | Min.w.wang@ericsson.com |
| OPPO | Qianxi Lu |  |
| InterDigital | Martino Freda | martino.freda@interdigital.com |
| Apple | Zhibin Wu | Zhibin_wu@apple.com |
| Huawei, HiSilicon | Tao Cai | tao.cai@huawei.com |
| Xiaomi | Xing Yang | Yangxing1@xiaomi.com |
| vivo | Jing LIANG | liangjing@vivo.com |
| Qualcomm | Qing Li | qinli@qti.qualcomm.com |
| Samsung | Hyunjeong Kang | hyunjeong.kang@samsung.com |
| Lenovo | Jing HAN | Hanjing8@lenovo.com |

## 1st changes

### 1st change

Add clarification that measurements for sidelink are only CBR in clause 7.1

The first change is shown in the below

## 7.1	Services and Functions

The main services and functions of the RRC sublayer over the Uu interface include:

-	Broadcast of System Information related to AS and NAS;

-	Paging initiated by 5GC or NG-RAN;

-	Establishment, maintenance and release of an RRC connection between the UE and NG-RAN including:

-	Addition, modification and release of carrier aggregation;

-	Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR.

-	Security functions including key management;

-	Establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs);

-	Mobility functions including:

-	Handover and context transfer;

-	UE cell selection and reselection and control of cell selection and reselection;

-	Inter-RAT mobility.

-	QoS management functions;

-	UE measurement reporting and control of the reporting;

-	Detection of and recovery from radio link failure;

-	NAS message transfer to/from NAS from/to UE.

The sidelink specific services and functions of the RRC sublayer over the Uu interface include:

-	Configuration of sidelink resource allocation via system information or dedicated signalling;

-	Reporting of UE sidelink information;

-	Measurement configuration and reporting of CBR related to sidelink;

-	Reporting of UE assistance information for SL traffic pattern(s).

Q1: Do companies agree with 1st change as shown in the above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| OPPO |  | Yet there is another SL RSRP measurement that can be configured by Uu-RRC, not sure if it should be covered as well. If so, maybe no harm to leave it as it is as a generalized wording. |
| InterDigital | No | Agree with OPPO that the existing text already covered the more general case of SL measurements (including CBR) |
| Apple | No | The change seems wrong. |
| Huawei | No | Agree with OPPO on SL RSRP measurement |
| Xiaomi | No | General description is more forward compatible. |
| vivo | No | Agree with companies that the current text is fine in a more general way. |
| Qualcomm | No w. comment | Current text doesn’t break any feature. |
| ZTE | No |  |
| Samsung | No | As OPPO pointed out, the current text covers SL-RSRP measurement configuration for unicast connection as well as CBR. |
| Lenovo | No | General description is enough. |
| CATT | No |  |

Rapporteur summary

Out of 12 companies, 1 company provided the answer “Yes”, 11 companies provided the answer “No”. given there is majority view to not support this change.

rapporteur would like to make the below proposal.

1st change in R2-2308431/R2-2308432 is not pursued.

### 2nd change

Clarified that SL RRC decides measurements for sidelink but also the RLC mode. in clause 16.9.2.6

The change is shown in the below

#### 16.9.2.6	RRC

The RRC sublayer provides the following services and functions over the PC5 interface:

-	Transfer of a PC5-RRC message between peer UEs;

-	Maintenance and release of a PC5-RRC connection between two UEs;

-	Detection of sidelink radio link failure for a PC5-RRC connection.

-	Measurement configuration and reporting related to sidelink;

-	RLC mode to be used between two UEs;

A PC5-RRC connection is a logical connection between two UEs for a pair of Source and Destination Layer-2 IDs which is considered to be established after a corresponding PC5 unicast link is established as specified in TS 23.287 [40]. There is one-to-one correspondence between the PC5-RRC connection and the PC5 unicast link. A UE may have multiple PC5-RRC connections with one or more UEs for different pairs of Source and Destination Layer-2 IDs.

Separate PC5-RRC procedures and messages are used for a UE to transfer UE capability and sidelink configuration to the peer UE, as specified in TS 38.331 [3]. Both peer UEs can exchange their own UE capability and sidelink configuration using separate bi-directional procedures in both sidelink directions.

If it is not interested in sidelink transmission, if sidelink RLF on the PC5-RRC connection is declared, or if the Layer-2 link release procedure is completed as specified in TS 23.287 [40], UE releases the PC5-RRC connection.

Q2: Do companies agree with 2nd change as shown in the above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| OPPO | No | RLC mode to be used between two UEs;” is not needed, since it is just part of RRC configured parameter |
| InterDigital | No | Agree with OPPO. |
| Apple |  | The “RLC mode” addition seems too detail for Stage- 2 spec |
| Huawei, HiSilicon | Comments | Yes first link, No to second line on RLC mode |
| vivo | No | For the RLC mode issue, agree with companies that thi is no needed. For the measurement part, we are also wondering if this can already be covered by ‘Transfer of a PC5-RRC message between peer UEs’. |
| Qualcomm | No | Don’t see the need for stage 2 |
| ZTE | No |  |
| Samsung | No for 2nd change | PC5 RRC is used to configure SLRB configuration parameters including RLC mode. |
| Lenovo | No | Agree with OPPO, at least RLC part is not needed. No strong view on measurement part |
| CATT | No |  |

Rapporteur summary

Out of 12 companies, 1 company provided the answer “Yes”, 11 companies provided the answer “No” to the second line in the 2nd change, while companies are ok (no strong view) to the first line in the second change.

Rapporteur would like to make the below proposal.

1st line in the 2nd change in R2-2308431/R2-2308432 is agreed and 2nd line in the 2nd change in R2-2308431/R2-2308432 is not pursued.

### 3rd change

In clause 16.9.3.1, fix identation on the last bulled of the section

The change is shown in the below

#### 16.9.3.1	General

For NR sidelink communication, the UE can operate in two modes as specified in 5.7.2 for resource allocation in sidelink:

-	Scheduled resource allocation, characterized by:

-	The UE needs to be RRC_CONNECTED in order to transmit data;

-	NG-RAN schedules transmission resources.

-	UE autonomous resource selection, characterized by:

-	The UE can transmit data when inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when outside NG-RAN coverage;

-	The UE autonomously selects transmission resources from resource pool(s).

For NR sidelink communication, the UE performs sidelink transmissions only on a single carrier.

Q2: Do companies agree with 3rd change as shown in the above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| InterDigital |  | Fine to make the change if we agree on a CR. |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes | Can piggyback on other agreed CRs |
| ZTE | Yes |  |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| CATT | Yes |  |

Rapporteur summary

Out of 12 companies, all companies agree with the change.

Rapporteur would like to make the below proposal.

rapporteur would like to suggest further discussions for the issue.

3rd change in R2-2308431/R2-2308432 is agreed.

### 4th change

In clause 16.9.4.2, Clarified that UE may also request SIB on-demand while in RRC_CONNECTED

The change is shown in the below

#### 16.9.4.2	Control of connected Ues

The UE in RRC_CONNECTED performs NR sidelink communication and/or V2X sidelink communication, as configured by the upper layers. The UE sends Sidelink UE Information to NG-RAN in order to request or release sidelink resources and report QoS information for each destination.

NG-RAN provides RRCReconfiguration message to the UE in order to provide the UE with dedicated sidelink configuration. The RRCReconfiguration message may include SL DRB configuration(s) for NR sidelink communication and/or V2X sidelink communication as well as mode 1 resource configuration and/or mode 2 resource configuration. If UE has received SL DRB configuration via system information, UE should continue using the configuration to perform sidelink data transmissions and receptions until a new configuration is received via the RRCReconfiguration message.

If the NG-RAN does not provide provide a configuration for NR sidelink communication and/or V2X sidelink communication via the RRCReconfiguration message, the UE may request SIB12 (for resource pool configuration and SL DRB configuration for NR sidelink communication) and SIB13, SIB14 (for resource pool configuration for V2X sidelink communication) on-demand while in RRC_CONNECTED according to TS 38.331 [12].

NG-RAN may also configure measurement and reporting of CBR for NR sidelink communication and V2X sidelink communication, and reporting of location information for V2X sidelink communication to the UE via RRCReconfiguration message.

During handover, the UE performs sidelink transmission and reception based on configuration of the exceptional transmission resource pool or configured sidelink grant Type 1 (for NR sidelink communication only) and reception resource pool of the target cell as provided in the handover command.

Q2: Do companies agree with 4th change as shown in the above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| OPPO |  | If the NG-RAN does not provide provide a configuration for NR sidelink communication and/or V2X sidelink communication via the RRCReconfiguration message, the UE may request SIB12 (for resource pool configuration and SL DRB configuration for NR sidelink communication) and SIB13, SIB14 (for resource pool configuration for V2X sidelink communication) on-demand while in RRC_CONNECTED according to TS 38.331 [12]. It seems to say for RRC_CONNECTED UE, it can rely on SIB for pool and SLRB configuration, rather than dedicated RRC, which is not aligned with our understanding, i.e., network would always use dedicated RRC for the control of RRC_CONNECTED. |
| InterDigital | No | Same understanding as OPPO. Similar to LTE, the UE in RRC_CONNECTED relies on dedicated pool configuration only. |
| Apple | No | Same view as OPPO. |
| Huawei | No | This would be strange behavior, UE can ask for dedicated configuration directly. |
| Xiaomi | comments | In RRC, it doesn’t prevent the UE in RRC_CONNECTED to use configuration provided by SIB12. According to the following spec, if RRCReconfiguration doesn’t provide the configuration on used frequency, UE in RRC_CONNECTED is allowed to apply the configuration provided in SIB12. The else part doesn’t exclude UE in RRC_CONNECTED. RAN2 shall clarify whether UE in RRC_CONNECTED can use configuration from SIB12. ### 5.8.8 Sidelink communication transmission A UE capable of NR sidelink communication that is configured by upper layers to transmit NR sidelink communication and has related data to be transmitted shall: - if the conditions for NR sidelink communication operation as defined in 5.8.2 are met: 2> if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12: 3> if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message: …. 3> else: 4> if the cell chosen for NR sidelink communication transmission provides SIB12: |
| vivo | Comments | It is not clear these stage-2 modifications refers to which TS 38.331 part. It may be explained by the proposed company. In our understanding the UE may not request these SIBs unless e.g. the UE has not stored a valid version of the SIB. |
| Qualcomm | No | Don’t see the need. |
| ZTE | No with comments | Share same view with xiaomi, at least in current specification, UE can use configuration from SIB in case no dedicated configuration is obtained. But if dedicated configuration is received, UE should follow it, instead of SIB12. In RRC_CONNECTED, the UE applies the NR sidelink communications parameters provided in RRCReconfiguration (if any). In RRC_IDLE or RRC_INACTIVE, the UE applies the NR sidelink communications parameters provided in system information (if any). For other cases, UEs apply the NR sidelink communications parameters provided in SidelinkPreconfigNR (if any). When UE performs state transition between above three cases, the UE applies the NR sidelink communications parameters provided in the new state, after acquisition of the new configurations. Before acquisition of the new configurations, UE continues applying the NR sidelink communications parameters provided in the old state.. For this change, on-demand SIB request is a general Uu interface procedure, we do not need to duplicate this for sidelink again. |
| Samsung | No | For V2X sidelink communication, SL DRB configuration is up to UE implementation, so the change “and/or V2X sidelink communication” is incorrect. “The RRCReconfiguration message may include SL DRB configuration(s) for NR sidelink communication and/or V2X sidelink communication as well as mode 1 resource configuration and/or mode 2 resource configuration.” RRC_CONNECTED UE does not have to rely on SIB for TX resource so the following change is not needed. “If the NG-RAN does not provide provide a configuration for NR sidelink communication and/or V2X sidelink communication via the RRCReconfiguration message, the UE may request SIB12 (for resource pool configuration and SL DRB configuration for NR sidelink communication) and SIB13, SIB14 (for resource pool configuration for V2X sidelink communication) on-demand while in RRC_CONNECTED according to TS 38.331 [12].” |
| Lenovo | No | Agree with Xiaomi and ZTE. |
| CATT | No | The CR doesn’t convince us. |

Rapporteur summary

Out of 12 companies, 1 company agrees with the change, 11 companies don’t with the change.

Rapporteur would like to make the below proposal.

4th change in R2-2308431/R2-2308432 is not pursued.

### 5th change

In clause 16.9.4.3, clarified that UE may also request SIB on-demand while in RRC_IDLE and RRC_INACTIVE.

The change is shown in the below

#### 16.9.4.3	Control of idle/inactive Ues

The UE in RRC_IDLE or RRC_INACTIVE performs NR sidelink communication and/or V2X sidelink communication, as configured by the upper layers. NG-RAN may provide common sidelink configuration to the UE in RRC_IDLE or RRC_INACTIVE via system information for NR sidelink communication and/or V2X sidelink communication. UE receives resource pool configuration and SL DRB configuration via SIB12 for NR sidelink communication as specified in TS 38.331 [12], and/or resource pool configuration via SIB13 and SIB14 for V2X sidelink communication as specified in TS 38.331 [12].

The UE may request SIB12 (for resource pool configuration and SL DRB configuration for NR sidelink communication) and SIB13, SIB14 (for resource pool configuration for V2X sidelink communication) on-demand while in RRC_IDLE or RRC_INACTIVE, according to TS 38.331 [12].

When the UE performs cell reselection, the UE interested in V2X service(s) considers at least whether NR sidelink communication and/or V2X sidelink communication are supported by the cell. The UE may consider the following carrier frequency as the highest priority frequency, except for the carrier only providing the anchor carrier:

-	the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration, if configured to perform both NR sidelink communication and V2X sidelink communication;

-	the frequency providing NR sidelink communication configuration, if configured to perform only NR sidelink communication.

-	the frequency providing V2X sidelink communication configuration, if configured to perform only V2X sidelink communication.

Q2: Do companies agree with 5th change as shown in the above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Apple | No | Seems redundant as the current text already say “UE receives resource pool configuration and SL DRB configuration via SIB12 for NR sidelink communication as specified in TS 38.331 [12], and/or resource pool configuration via SIB13 and SIB14 for V2X sidelink communication as specified in TS 38.331 [12].” |
| Huawei | Comments | With the first paragraph “ NG-RAN may provide…..” and “ UE receives ‘, also SIB 12/13/14 would be on-demand SI, is it really critical to have the added paragraph? We are fine to add if majority agrees to add. |
| Xiaomi | No | Similar view as apple and HW |
| vivo |  | Similar comment as in above question. |
| Qualcomm | No | Realy don’t see the needs for stage 2. |
| ZTE | No | For this change, on-demand SIB request is a general Uu interface procedure, we do not need to duplicate this for sidelink again. |
| Samsung |  | We wonder whether the change is really essential. |
| Lenovo | No | We do not see it is essential to add this part |
| CATT | No |  |

Rapporteur summary

Out of 10 companies, 1 company agrees with the change, 9 companies don’t with the change.

Rapporteur would like to make the below proposal.

5th change in R2-2308431/R2-2308432 is not pursued.

# Conclusion

We have the following proposal:

Proposal 1	1st change in R2-2308431/R2-2308432 is not pursued.

Proposal 2	1st line in the 2nd change in R2-2308431/R2-2308432 is agreed and 2nd line in the 2nd change in R2-2308431/R2-2308432 is not pursued.

Proposal 3	3rd change in R2-2308431/R2-2308432 is agreed.

Proposal 4	4th change in R2-2308431/R2-2308432 is not pursued.

Proposal 5	5th change in R2-2308431/R2-2308432 is not pursued.

# Reference

[1] R2-2308431	Miscellaneous stage 2 corrections for sidelink	Ericsson	CR	Rel-16	38.300	16.13.0	0700	-	F	5G_V2X_NRSL-Core

[2] R2-2308432	Miscellaneous stage 2 corrections for sidelink	Ericsson	CR	Rel-17	38.300	17.5.0	0701	-	A	5G_V2X_NRSL-Core