3GPP TSG-RAN WG2 Meeting #123		 R2-2309199

Toulouse, France: 21st August – 25th August 2023	upate of R2-2308596

Source:	Qualcomm Incorporated

Title:	Discussion on AI/ML Architecture General

Agenda Item:	7.16.2.1 Architecture General

Document for:	Discussion and Decision

# Introduction & Background

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface has been approved [1], with the main goal of exploring the benefits of augmenting the air interface with features enabling improved support of AI/ML-based algorithms for enhanced performance and/or reduced complexity/overhead.

Through studying a few carefully selected use cases, the goal is to identify a common AI/ML framework, including functional requirements of AI/ML architecture, which could be used in subsequent projects. The study should also identify areas where AI/ML could improve the performance of air-interface functions.

The study will serve to identify what is required for an adequate AI/ML model characterization and description establishing pertinent notation for discussions and subsequent evaluations. Various levels of collaboration between the gNB and UE are identified and considered. Specification impact will be assessed to improve the overall understanding of what would be required to enable AI/ML techniques for the air interface.

The SI consists of studying individual use cases as well as deriving a general framework for AI/ML. Below we summarize the goal of the study as shown in [1,2] relevant to the general framework:

AI/ML model, terminology, and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting separate or joint ML operations.

- Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures, and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

The SI further defines responsibility for different WGs for accessing potential specification impacts [1,2], whereas the RAN2 study access protocols aspects of the potential specification impacts, as mentioned below:

- […]

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input

    - Collaboration level-specific specification impact per use case

In this contribution, we discuss the following,

- Indication of applicable condition (e.g., AI/ML algorithms dependency on locality, gNB configuration, availability of models or underlying models (for functionality-based LCM), and others

# Protocols design aspects (Architecture General)

In the contribution paper, we discuss the following,

- Functionality-based LCM and applicable conditions indication,

- Model ID-based LCM and applicable conditions indication

## Functionality-based LCM and applicable condition indication

In the RAN1#112bis-emeeting [3], for the functionality-based LCM and applicable condition indication, RAN1 made the following agreement,

---------------------

Agreement

- Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities.

---------------------

Based on the above RAN1 agreement, we believe that for determining applicable functionality, there are two open issues for functionality

- Open issue 1: Indication of requested additional conditions to determine applicable functionalities [among configured/identified functionalities], and

- Open issue 2: Procedures for indicating applicable functionality

Observation 1: Based on RAN1#112bis-emeeting, two open issues can be considered for determining applicable functionality

- Open issue 1: Indication of requested additional conditions to determine applicable functionalities [among configured/identified functionalities], i.e., the network (e.g., gNB) maybe need to signal what additional conditions are required from/by the UE for determining applicable functionality, and

- Open issue 2: Procedures for indicating applicable functionality / additional conditions, i.e., requirement and different options for signaling applicable functionality / additional conditions for determining applicable functionality.

    - Indication of requested applicable conditions

In RAN1#112#112bis-emeeting, RAN1 made the following agreement

-------------------

Agreement

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

  - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

  - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

    - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

    - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

    - FFS: Other aspects that may constitute Functionality

  - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

- For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

  - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

  - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

  - FFS: relationship between functionality-based LCM and model-ID-based LCM

- Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

-------------------

- According to the agreements above, the additional condition examples can be scenarios, sites, and datasets.

Observation 2: The determination of applicable functionality can be performed at the network / UE based on additional conditions.

- For determining the applicable condition at the network, the additional condition should be provided to the network. The UE can report the additional conditions based on the indication of additional conditions request from the network.

- For determining the applicable condition at the UE, the network can indicate the relevant additional condition, in the form of assistance information,  that should be taken into account to determine applicable functionalities.

Proposal 1: Depending on the use case, the network may indicate assistance information (e.g., scenarios, site, datasets) to help the UE determine the applicable functionalities.

    - Procedure for applicable functionality / additional condition indication

Based on the RAN1#112bis-emeeting agreements, the following options are possible for applicable condition indications

- Alt-1: Applicable functionalities / additional conditions are indicated after the configuration of functionalities at the UE. Applicable functionalities are used for determining which functionality to activate among configured functionalities, for a given use case.

- Alt-2: Applicable functionalities / additional conditions are indicated before the configuration of functionalities at the UE. Applicable functionalities are used for determining the functionality configuration.

Observation 3: The following options are possible for applicable functionalities / additional condition indications

- Alt-1: Applicable functionalities / additional conditions are indicated after the configuration of functionalities at the UE. Applicable functionalities are used for determining which functionality to activate among configured functionalities, for a given use case.

- Alt-2: Applicable functionalities / additional conditions are indicated before the configuration of functionalities at the UE. Applicable functionalities are used for determining the functionality configuration.

- Note that currently, the UE does not accept a configuration that it cannot comply with immediately. While in Alt-2, the complete information is provided to the network for appropriate configuration. It may also require frequent functionality configuration based on changing applicable functionalities information.

Observation 4: According to the current UE behavior, UE does not accept a configuration that it cannot comply with immediately after applying.

Proposal 2: As a baseline, the UE only accepts a functionality configuration that it can comply with immediately. It is left for stage 3 discussion whether the UE will accept a configuration that it cannot comply with immediately.

      - Procedures for indicating applicable information to determine functionality configuration

As discussed above, currently UE does not accept a configuration that it cannot comply with immediately. Therefore, the applicable condition should be provided to the network to assist with functionality configuration. While connected to the same cell, the UE may update its applicable functionalities information. For updating the applicable functionalities information, the UE can use existing “UE Assistance Information”.

Proposal 3: Applicable functionality information can be update can be initiated by the UE. The specific messages carrying this information is left for stage 3.

## Model ID-based LCM and applicable condition indication

    - Meta info and applicable conditions

The meta information is by definition side-information about the model that will not be part of the standard and not transmitted over the air.

Anything transmitted over the air will not be called meta-information, but becomes part of the “Conditions” or “Additional conditions” defining the feature “applicability”.

As such, we propose to agree that:

Proposal 4: Meta-information is not be specified.

While the contents is not standardized, we may discuss what kind of information may be carried in the meta information. We understand that information may be required for model identification, for model inference (i.e., input/output of the model), and for model management and control purposes, such as

- Model identification, such as an ID, and

- Model inference information, such as model input/output, etc.

- Model management and control purpose, such as applicable conditions, etc.

- Additional information as needed by the implementation.

    - Use of meta info for model management and control procedures

Note that meta info is not UE-specific information rather it is model-specific information. The initial model identification procedure will not be in the scope of RAN2.The network should map the provided meta information with the assigned ID for subsequent operations. The meta information is stored in the network for control and management purposes.

Observation 5: Meta info is not UE-specific information rather it is model-specific information.

Proposal 5: The model’s meta information is stored at the network (e.g., NG-RAN) for control and management purposes, in the subsequent operations.

Proposal 6: The model ID allocation or assignment procedure is outside the scope of RAN2.

# Conclusions

Based on the following observations:

Observation 1: Based on RAN1#112bis-emeeting, two open issues can be considered for determining applicable functionality

- Open issue 1: Indication of requested additional conditions to determine applicable functionalities [among configured/identified functionalities], i.e., the network (e.g., gNB) maybe need to signal what additional conditions are required from/by the UE for determining applicable functionality., and

- Open issue 2: Procedures for indicating applicable functionality / additional conditions, i.e., requirement and different options for signaling applicable functionality / additional conditions for determining applicable functionality.

Observation 2: The determination of applicable functionality can be performed at the network / UE based on additional conditions.

- For determining the applicable condition at the network, the additional condition should be provided to the network. UE can report the additional conditions based on the indication of additional conditions request from the network.

- For determing the applicable condition at the UE, the network can indicate the relevant additional condition, in the form of assistance information, that should be taken into the account to determine applicable functionalities.

Observation 3: The following options are possible for applicable functionalities / additional condition indications

- Option 2Alt-1: Applicable functionalities / additional conditions are indicated after the configuration of functionalities at the UE. Applicable functionalities are used for determining which functionality to activate among configured functionalities, for a given use case.

- Alt-2: Applicable functionalities / additional conditions are indicated before the configuration of functionalities at the UE. Applicable functionalities are used for determining the functionality configuration.

Observation 4: According to the current UE behavior, UE does not accept a configuration that it cannot comply with immediately after applying.

Observation 5: Meta info is not UE-specific information rather it is model-specific information.

We propose that:

Proposal 1: Depending on the use case, the network may indicate assistance information (e.g., scenarios, site, datasets) to help the UE determine the applicable functionalities.

Proposal 2: As a baseline, the UE only accepts a functionality configuration that it can comply with immediately. It is left for stage 3 discussion whether the UE will accept a configuration that it cannot comply with immediately.

Proposal 3: Applicable functionality information can be update can be initiated by the UE. The specific messages carrying this information is left for stage 3.

Proposal 4: Meta-information is not be specified.

Proposal 5: The model’s meta information is stored at the network (e.g., NG-RAN) for control and management purposes, in the subsequent operations.

Proposal 6: The initial model ID allocation or assignment procedure is outside the scope of RAN2.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

- RAN1#109-emeeting report

- RAN1#110 meeting report

- RAN1#110bis-meeting report

- R2-2211101, “Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online”, RAN2#119bis-e, Oct 2022.

- Final_Minutes_report_RAN1#111_v100, “Final Report of 3GPP TSG RAN WG1 #111 v1.0.0  (Toulouse, France, 14th – 18th November 2022)”, Nov 2022.

- R2-2300002, “Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France”, RAN2#120, Nov 2022.

- Draft_Minutes_report_RAN1#112_v040, “Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0  (Athens, Greece, 27th February – 3rd March 2023)” Mar 2023.

- Draft_R2_121_meeting_report_v1, “Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece”, Ran2#121, Feb 2023.

- Draft_Minutes_report_RAN1#112b-e_v010, “Draft Report of 3GPP TSG RAN WG1 #112bis-e v0.1.0,” April 2023.

- Draft_R2_121bis_meeting_report_v2, “Draft Report of 3GPP TSG RAN WG2 meeting #121bis-e Online,” April 2023.