3GPP TSG RAN WG2#123	R2-2309256

Toulouse, France, 21st - 25th August 2023

3GPP TSG RAN WG1 #114		R1-2308477

Toulouse, France, August 21-25, 2023

Title:	LS on Details of power headroom information for assumed PUSCH

Response to:

Release:	Rel-18

Work Item:	NR_cov_enh2

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact person:	Paul Marinier

paul.marinier@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 sent LS to RAN2 on power headroom information for assumed PUSCH [R1-2308376, RAN1#114]. RAN1 would like to inform RAN2 that RAN1 has made agreement on details of power headroom information for assumed PUSCH:

Agreement For reporting of power headroom information for assumed PUSCH using target waveform different from waveform of actual PUSCH, support the following: - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH contains: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - If UE reports power headroom information for assumed PUSCH in a PUSCH transmission, legacy PHR is also reported in the same PUSCH transmission. - No consensus in RAN1 if the following applies or not: if UE reports legacy PHR in a PUSCH transmission, power headroom information for assumed PUSCH is also reported. - Note: RAN endorsed the following at RAN#100: “RAN2 will not work on PHR triggering procedure for dynamic waveform switching in Rel-18 UL Coverage enh WI” [RP-231498]. Send LS to RAN2 to inform above agreement.

# 2	Actions

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to make necessary specification changes to support reporting of power headroom information for assumed PUSCH, considering the above information.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #114-bis	9th Oct – 13th Oct 2023		Xiamen, CN

TSG RAN WG1 Meeting #115		13th Nov – 17th Nov 2023	Chicago, US