3GPP TSG-WG RAN2 Meeting #123	R2-2309156

Toulouse, France, August 21 – 25, 2023

Title:	Reply LS on carrier mapping for unicast SL CA

Response to:	Reply LS on carrier mapping for unicast SL CA (R2-2307060/ S2-2307794)

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	RAN2

To:	SA2

Cc:

Contact person:

Name:	Giwon Park

E-mail Address:	giwon.park@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 would like to thank SA2 regarding the response LS on carrier mapping for unicast SL CA. RAN2 discussed the answers provided by SA2 during RAN2#123 and would like to provide the following RAN2 agreements on SA2’s question and carrier mapping for unicast SL CA.

Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how can UE's AS layer obtain the mapping between L2 ID and frequencies?

Agreement on SA2 question:

1: 	On Question 1 of S2-2307794, reply SA2 "RAN2's question 1 is intended to discuss after PC5 link establishment. And RAN2 assume that the AS layer may maintain a mapping between old L2 ID (before PC5 link establishment) and new L2 ID (after PC5 link establishment) by its implementation".

Agreement on SL CA before UC link is established:

1: 	SL CA is not applied before UC link is established.

2. Actions:

To SA2: RAN2 respectfully asks SA2 to take the above information into account in related work and whether any issue on the RAN2 assumption is observed by SA2.

3. Dates of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting#123-bis	October 09 – 13, 2023	Xiamen, China

TSG RAN WG2 Meeting#124	November 13 – 17, 2023	Chicago, USA