3GPP TSG RAN WG2#123	R2-2307005

Toulouse, France, 21st - 25th August 2023

3GPP TSG RAN WG1 #113	R1-2306182

Incheon, Korea, May 22nd – 26th, 2023

# Title:	Reply LS on HARQ Enhancements

# Response to:	R1-2304324 (R2-2304274)

# Release:	Release 18

Work Item:	IoT_NTN_enh-Core

Source:	RAN1

To:	RAN2

Cc:	-

Contact Person:

Name:	Zuomin Wu

E-mail Address:	wuzuomin@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2’s LS on HARQ enhancements. Regarding RAN2 raised questions, RAN1 has discussed the corresponding responses in the RAN1#113 meeting. The following RAN1 agreements are achieved.

Agreement Adopt the response below to RAN2’s question 3: - Understanding 2 is the correct understanding. Agreement For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms. - Note: this implies a RAN1 specification change in Rel-18 Agreement For the response to RAN2’s question 1a: provide the agreement above. Agreement Adopt the response below to RAN2’s question 1b: - For an UL HARQ process with HARQ mode B for eMTC UEs, the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process is 1 ms for Type B half-duplex FDD operation, and there is no minimum time restriction between the end of PUSCH transmission and the start of MPDCCH monitoring for full-duplex FDD operation. Agreement Adopt the response below to RAN2’s question 2: - Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision. Agreement For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms. - Note: this implies a RAN1 specification change in Rel-18 Agreement - For the response to RAN2’s question 1a: provide the agreement above.

Please find RAN1 responses below:

RAN2’s question 1a: For an UL HARQ process with HARQ mode B for NB-IoT UEs, what is the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process?

RAN1 response:

For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms.

- Note: this implies a RAN1 specification change in Rel-18.

For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms.

- Note: this implies a RAN1 specification change in Rel-18.

RAN2’s question 1b: For an UL HARQ process with HARQ mode B for eMTC UEs, what is the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process?

RAN1 response:

For an UL HARQ process with HARQ mode B for eMTC UEs, the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process is 1 ms for Type B half-duplex FDD operation, and there is no minimum time restriction between the end of PUSCH transmission and the start of MPDCCH monitoring for full-duplex FDD operation.

RAN2’s question 2: For UL multiple TB scheduling, which of the following HARQ mode combinations does RAN1 intend to support for eMTC and NB-IoT?

- Case 1: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A

- Case 2: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode B

- Case 3: some HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A and the others are configured with HARQ mode B

RAN1 response:

Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision.

RAN2’s question 3: For the above RAN1 agreement, which is the correct understanding?

- Understanding 1: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for the same HARQ process in a period of Y=12(ms) from the end of reception of the NPDSCH.

- Understanding 2: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for all the HARQ processes in a period of Y=12(ms) from the end of reception of the NPDSCH.

RAN1 response:

Understanding 2 is the correct understanding.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above responses into account in the future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#114	21 - 25 August, 2023	Toulouse, France

RAN1#114-bis	9 - 13 October, 2023	Xiamen, China